IEEE P802.11
Wireless LANs

Proposed Reply Comments of IEEE 802 on FCC Docket 99-231 on Modifying the Rules for Spread Spectrum Devices

Date:	November 10, 1999

Author:	IEEE 802.11 Regulatory Ad-Hoc Group

Summary

Attached is a proposal for reply comments to the FCC NPRM in OET Docket 99-231. The document is a proposed reply referencing comments received by the FCC in the first phase of the proceding.

The text was generated by the 802.11 regulatory ad-hoc group, based on submissions and discussions held at the November 1999 802 meeting.  It was reviewed and approved by the 802.11 Working Group Plenary session by a vote of 25 Approve, 0 Dis-approve, 1 Abstain. The Local and Metropolitan Area Networks Standards Committee of the Institute of Electrical and Electronic Engineers (IEEE-LMSC) authorized the study and accepted the finding of the Regulatory Study Group tasked with investigating the reply comments.

November 11, 1999

Magalie R. Salas, Esquire

Secretary

Federal Communications Commission

445 12th St. SW

Washington DC 20554

Re:	 Amendment of Part 15 of the Commission's Rules for Spread Spectrum Devices, ET Docket No. 99-231

Dear Ms. Salas:

The Regulatory Ad-Hoc Group (RAHG) of the IEEE 802.11 Working Group submitted comments opposing the proposed Part 15 rule changes to increase the maximum bandwidth allowed for frequency hopping devices in response to the Commission's Notice of Proposed Rulemaking regarding unlicensed spread spectrum devices. The RAHG agreed that the existing rules for direct sequence systems are adequate, with the additional requirement as proposed by the Commission that a processing gain calculation be included for systems which have fewer than 10 chips per symbol. The RAHG also advised the Commission of our concerns regarding the alternative Gaussian noise test as proposed.

The RAHG provided extensive analysis showing that the proposed rules change permitting wide bandwidth frequency hopping systems would result in increased interference to systems complying with the current rules even with the lowered power level restraints proposed. A number of commenters asserted that there would be no increase in interference while a number agreed with RAHG that there would be an increase in interference. Intersil and Nokia supplied analysis in addition to that of the RAHG showing increased interference. There was no analysis presented supporting the claim that the proposal would not increase interference.

Most commenters agreed that the CW jammer test requirement was sufficient to qualify direct sequence systems. However, some commenters proposed that only a Gaussian noise qualification test is sufficient for direct sequence systems with fewer than 10 chips per symbol. The commenters in favor of such a test did not address the complexities that RAHG described. The RAHG continues to assert that the CW jammer test provides sufficient assurance that a direct sequence system meets the spreading rules indicated by the calculation and declaration. The RAHG believes that the proposed alternative Gaussian noise jamming test should be excluded, until a detailed test procedure specifically designed for evaluating processing gain is developed.  Inclusion of this test even as an option without an accompanying test procedure invites inaccurate and widely variable test results.

In summary the RAHG found no comments which effectively disputed its claim of increased interference if wideband frequency hopping is permitted, nor any compelling evidence that the CW jammer test in conjunction with a mathametical declaration was insufficient for demonstrating direct sequence processing gain. The RAHG thus urges the Commission to reject the proposed increase in frequency hopping bandwidth and not to impose the Gaussian noise test requirement on direct sequence systems.

Respectfully,

James T. Carlo (jcarlo@ti.com)			Vic Hayes (vichayes@lucent.com)

Chair, IEEE 802 LAN/MAN Standards			Chair, IEEE 802.11, Wireless LANs

Texas Instruments			Lucent Technology

9208 Heatherdale Drive					Zadelstede 1-10

Dallas TX 75234			3431 JZ Nieuwegein, the Netherlands

cc:

Chairman William E. Kennard

Commissioner Susan Ness

Commissioner Harold Furchtgott-Roth

Commissioner Michael K. Powell

Commissioner Gloria Tristani

Dale Hatfield

Julius P. Knapp

Neal L. McNeil

Karen Rackley

John A. Reed

Anthony Serafin

See for example, the comments of Proxim at C, HomeRF at 3 and Breezecom at 5.

See for example, the comments of Nokia at II, Intersil comments of September 3, 1999 and Aironet at 3.

See the comments of Aironet at 5 and Proxim at 6.