P1541/D2

1999-12-24

# Draft
Standard Prefixes for Binary Multiples

Sponsored by the

Standards Coordinating Committee on Quantities, Units, and Letter Symbols (SCC14)

of the

IEEE Standards Association

Copyright © 1999 by the Institute of Electrical and Electronics Engineers, Inc.

345 East 47th Street

New York, NY 10017, USA

All rights reserved.

This is an unapproved draft of a proposed IEEE Standard, subject to change. Permission is hereby granted for IEEE Standards Committee participants to reproduce this document for purposes of IEEE standardization activities. If this document is to be submitted to ISO or IEC, notification shall be given to the IEEE Copyright Administrator. Permission is also granted for member bodies and technical committees of ISO and IEC to reproduce this document for purposes of developing a national position. Other entities seeking permission to reproduce this document for standardization or other activities, or to reproduce portions of this document for these or other uses, must contact the IEEE Standards Department for the appropriate license. Use of information contained in this unapproved draft is at your own risk.

IEEE Standards Department

Copyright and Permissions

445 Hoes Lane, P.O. Box 1331

Piscataway, NJ 08855-1331, USA

# Introduction

[To be prepared]

(This introduction is not part of IEEE P1541, Standard Prefixes for Binary Multiples.)

This standard was prepared by a subcommittee of IEEE SCC14 that comprised the following members:

James R. Frysinger, Chair

Gordon Aubrecht

Bruce B. Barrow

Stanley Ehrlich

David Goldman

Stan Jakuba

Joseph Langenstein

John Scott

Ralph Showers

Barry N. Taylor

Lorelle Young

Alan Zuckerwar

The following persons were on the balloting committee: (To be provided by IEEE editor at time of publication.)

Contents

Introduction	ii

1. Overview	5

1.1 Scope	5

1.2 Purpose	5

2. Definitions	5

3. Prefixes for binary multiples	5

4. Principles of usage	6

4.1 The SI prefixes	6

4.2 Application of the prefixes for binary multiples	6

5. Bibliography	6

# Draft Standard Prefixes for Binary Multiples

# 1. Overview

This standard is divided into three clauses. Clause 1 states the scope and purpose of the standard. Clause 2 defines the prefixes for binary multiples. Clause 3 gives guidance on the application of these prefixes.

## 1.1 Scope

This standard defines names and letter symbols for prefixes that denote multiplication of a unit by the binary multiplier 210 and powers thereof.  The prefixes are for use with all units, in all fields, where multiplication by a binary multiplier is found to be appropriate.

## 1.2 Purpose

In recent years confusion has resulted from the fact that the SI prefixes—kilo, mega, giga, etc. —have been used sometimes with their correct meaning as decimal multipliers, and sometimes with a special meaning as binary multipliers. The purpose of this standard is to establish prefixes for binary multiples in order to make precise and unambiguous communication possible, especially within the fields of data processing and information systems. This standard is fully consistent with an equivalent standard that has been adopted by the International Electrotechnical Commission [B1].

# 2. Definitions

2.1 bit: binary digit

2.2 byte: a group, usually eight, of adjacent bits operated upon as a unit

2.3 octet: a group of eight adjacent bits operated upon as a unit

# 3. Prefixes for binary multiples

The prefixes given in Table 1 shall be used to indicate multiplication by powers of 210.

PLACE TABLE 1 HERE

Table 1—Prefixes

| Factor | Name | Symbol | Origin | Related SI Prefixes |
| --- | --- | --- | --- | --- |
| 210 | kibi | Ki | kilobinary: (210) | kilo: (103) k |
| 220 | mebi | Mi | megabinary: (210)2 | mega: (103)2 M |
| 230 | gibi | Gi | gigabinary: (210)3 | giga: (103)3 G |
| 240 | tebi | Ti | terabinary: (210)4 | tera: (103)4 T |
| 250 | pebi | Pi | petabinary: (210)5 | peta: (103)5 P |
| 260 | exbi | Ei | exabinary: (210)6 | exa: (103)6 E |

Example:

—	one kibibit	1 Kib = 210 b = 1024 b

—	one megabyte	1 MB = 106 B = 1,000,000 B

—	one mebibyte	1 MiB = 220 B = 1,048,576 B

—	one megaoctet	1 Mo = 106 o = 1,000,000 o

—	one mebioctet	1 Mio = 220 o = 1,048,576 o

Note: In English the first syllable in the binary prefix name should be pronounced in the same way as in the first syllable of the corresponding SI prefix. The second syllable should be pronounced “bee”.

# 4. Principles of usage

## 4.1 The SI prefixes

The SI prefixes shall not be used to denote multiplication by powers of two.

## 4.2 Application of the prefixes for binary multiples

The general public is becoming better and better acquainted with the International System of Units (SI), the modern metric system, and will normally interpret kilo, mega, etc., in their proper decimal sense. The prefixes for binary multiples make it possible for specialists in the information sciences to communicate with precision. The use of these prefixes should be restricted to audiences that are familiar with mathematical notations such as 210 and (210)n. They should not ordinarily appear in the descriptive literature, such as instruction manuals and labels, that accompanies products sold to the general public.

Example 1: The common 90-mm diskette, as formatted for PC use, is usually advertised as containing 1.44 megabytes. It is actually formatted for 1440 kibibytes, and the “mega” in this application is neither a proper decimal prefix nor a binary prefix.  The resulting hybrid “megabyte” is equal to 1000 kibibytes. In communication with the general public the capacity of the diskette should be listed as 1.47 MB.

Example 2:  The following “Frequently Asked Question” appears in an instruction manual widely distributed in 1999:

Q:  The formatted capacity of my hard drive seems smaller than what was ordered.  Why?

A:  Your operating system assumes that 1 MB equals 1,048,576 bytes.  Drive manufacturers consider 1 MB as equal to 1,000,000 bytes.  Thus if the drive is advertised as 6.4 gigabytes (6,400,000,000 bytes) the operating system sees it as approximately 6.1 GB (6,400,000,000/1,048,576,000 = 6.1035…).

Note that in this example the “gigabyte” is equal to 1000 mebibytes.

# 5. Bibliography

[B1]	International Standard IEC 60027-2, Letter symbols to be used in electrical technology, Part 2: Telecommunications and electronics, Amendment 2, 1999-01.