IEEE P802.18
Radio Regulatory - TAG

| [Comments on the Petition for Rulemaking – Amendment of Part 15 Rules for License-Exempt 57-64 GHz Band] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-06-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| James Gilb | SiBeam | San Diego, CA | 858-484-4339 | Last name at ieee dot org |
| Gary Baldwin | SiBeam | Sunnyvale, CA | 510-745-7334 | First initial, Last name at sibeam dot com |

Abstract

Re: [IEEE P802.15-04/0250r5 (PAR), IEEE P802.15-05/0054r0, etc.]

Abstract: [FCC OET RM-11104 opposing comments.]

Purpose: [On 30Sep04 the Wireless Communications Association (WCA) filed a petition for rulemaking with the FCC RM-11104.  These are opposing comments for your consideration and request that the IEEE review and approve this as our IEEE802 filing to the FCC as well.  This is in support of Project 802.15.3c PAR http://standards.ieee.org/board/nes/projects/802-15-3c.pdf.]

Notice: This document has been prepared to assist IEEE 802.18. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.18.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Michael Lynch> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.18 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

June 9, 2005

Mr. Bruce Franca

Acting Chief

Office of Engineering and Technology

Federal Communications Commission

445 Twelfth Street, S.W.

Washington, D.C. 20054 USA

| Subject: | Comments on the Petition for Rulemaking – Amendment of Part 15 Rules for License-Exempt 57-64 GHz Band |
| --- | --- |
| Re: | RM-11104 |

Dear Mr. Franca:

The Institute of Electrical and Electronics Engineers, Inc. (“IEEE”) is a non-profit organization with over 365,000 members in over 150 countries. One of its activities is to develop consensus based standards for a wide range of technologies and applications. The 802.15.3c (“the Standard”) is a project that was formed in March 2005 and has the charter of developing a 60 GHz Wireless Personal Area Network (“PAN”) alternate Physical Layer standard based on the IEEE Std 802.15.3-2003.

The past decade has seen considerable success in increasing the bandwidth from the core of a network to the home and the enterprise. However, the bandwidth in the home and the enterprise is inadequate or non-existent to support the new generation of applications such as high definition television (“HDTV”) connectivity, video gaming and file transfer. These applications will require data rate from 500 Mbps to over 2 Gbps. The Standard, which will be in full compliance with the Part 15.255 rules, will foster the development of semiconductor devices, software and equipment to fill this need.

The market for the Standard based product will be vast. Consider the market for HDTV, which is a good indicator of the potential market for the Standard based product. According to the market research firm InStat, about 10 million households currently have HDTV sets. This number is expected to soar to 52 million by 2009. The market driver for the Standard is to provide an inter-operable, scalable, low cost and high data rate mobile wireless solution.

The existing rules under Part 15.255 are designed to accommodate both indoor and outdoor applications.

The petition RM-11104 filed by the WCA effectively creates two rules, one for the outdoor and the other for the indoor. They are incompatible and unfair to the indoor product. The petition, if approved, will disrupt the performance of the WPAN. Therefore, the IEEE strongly opposes the Petition for Rulemaking filed by the Wireless Communications Association International (“WCA”) on the following grounds:

- The market for the Standard based product will be considerably larger than that for the outdoor point-to-point equipment. The effort by the IEEE and the industry to develop a standard is a testament to the market potential. The ratification of the standard will drive the development of new technologies at 60 GHz and at higher millimeter wave frequencies. WCA’s petition will hinder the development of the P803.15.3c consensus based standard.

- The WCA states that “the main limiter to the widespread usage of this valuable spectrum is the limited link distances that can be achieved under the current operating rules.” This problem has been addressed with the FCC opening the 70 GHz and 80 GHz bands. Therefore, the WCA should consider moving to 70 GHz and 80 GHz, which provides 10 GHz of aggregated spectrum and longer link distance than at 60 GHz. The non-exclusive licensing approach for the 70 GHz and 80 GHz bands make them more attractive to the service providers than the unlicensed 60 GHz band. The Commission adopted Part 15.255 rules in August 1998 for indoor and outdoor wireless applications. Admittedly, it has taken some time, but the IEEE and the industry have responded to the Commission’s action by taking the first step – create a standard based on the rules under Part 15.255.

- The WCA also states that the petition will not cause significant “potential for interference between links.”Error: Reference source not found On the contrary, the petition will cause significant interference as the window links will reflect energy and interfere with the Standard based product. Commercial windows have high power reflectivity at millimeter wave frequencies as they are designed for energy efficiency. The reflected power from a window link will disrupt a network based on Part 15.255 compliant product.

- The WCA’s petition requests that “should the Commission determine that enforcement of section 15.255(i) continues to be in the public interest, it should treat window links as it would any outdoor link under the rule, and clarify that the transmitter ID requirement does not apply to point-to-point transmissions that are directed through a window.” This request is problematic because, as stated in the previous paragraph, a significant portion of the transmitter power of a window link will reflect and cause interference with Part 15.255 compliant product. This is less likely to occur with an outdoor link. The transmitter ID is a powerful tool in identifying the interferer and expeditiously resolving the problem. As a WPAN network is closely identified with a user, the ease with which the user can resolve the interference defines the user’s positive experience with the technology.

- The WCA’s request to increase the power level is very disturbing. As stated earlier, the reflected power from a window link will affect the performance of the window link. With the increased EIRP, the reflected power will increase considerably and could potentially damage the Part 15.255 compliant product.

- The Commission must note that if the WCA’s petition is approved, there is no safe guard to ensure that an unlicensed point-to-point transmitter with an EIRP of 77.3 dBm (54 kW) is pointing through a window. This raises the possibility of exposing the general population to RF radiation.

In conclusion, the Commission must reject the WCA’s petition for rulemaking. When the P802.15.3c standard is ratified, currently slated for 2H 2007, it will foster competition, create new technologies and develop new applications for the first mass market for the millimeter wave technology. This will occur only if the rules under Part 15.255 are protected.

I would be glad to provide the Commission with more information regarding this matter.

Sincerely yours,

March 28, 2005, press release titled High Definition TV Service Now in 10 Million Homes.

See Reply Comments of the Wireless Communications Association Inc., RM-11105 (filed November 29, 2004).

See Allocations and Service Rules for the 71–76 GHz, 81–86 GHz, and 92–95 GHz Bands.

According to the petition, “users of high gain, point-to-point antennas the 57-64 GHz band will be deemed in compliance with the rule if they transmit an average EIRP of no more than 82 dBm, with a reduction of 2 dB for every dB that the transmitting antenna gain is below 51 dBi.”

As per WCA’s petition, an antenna with an aperture diameter of 24 inches (61 centimeters) can have a maximum EIRP of 77.3 dBm.

IEEE PAR 802.15.3c: http://standards.ieee.org/board/nes/projects/802-15-3c.pdf