Working Document towards a Draft Revision of :

RECOMMENDATION  ITU-R  M.1450-2*,**

Characteristics of broadband radio local area networks

(Questions ITU-R 212/8 and ITU-R 142/9)

(2000-2002-2003)

The ITU Radiocommunication Assembly,

considering

a)	that broadband radio local area networks (RLANs) are widely used for fixed, semi-fixed (transportable) and nomadic computer equipment for a variety of broadband applications;

b)	that broadband RLAN standards currently being developed are compatible with current wired LAN standards;

c)	that it is desirable to establish guidelines for broadband RLANs in various frequency bands;

d)	that broadband RLANs should be implemented with careful consideration to compatibility with other radio applications;

e)	that the above guidelines should not limit the effectiveness of broadband RLANs but be used to enhance their application,

recommends

1	that for guidance on existing broadband RLAN standards,  Table 2 can be referred to; (NOTE 2)

2	that for details on methods of multiple access and modulation techniques for broadband RLANs in mobile applications, Table 3 can be referred to;

3

4

5

6	that for guidance on fixed applications of RLANs refer to Recommendation ITU-R F.1244.
[Editorial note: the reference to Recommendation ITU-R F.1244 should ultimately be replaced with reference to the draft new Report [BWA.REQ], which has been developed by WP 9B and is intended to incorporate the relavent information from Rec. F. 1244, after it is approved by Study Group 9.

NOTE 1 – Acronyms and terminology used in this Recommendation are given in Table 1.

NOTE 2 – The Annex provides detailed information on how to obtain complete standards described in Table 3.

TABLE  1

Acronyms and terms used in this Recommendation

| AP | Access point |
| --- | --- |
| Access Method | Scheme used to provide multiple access to a channel |
| Bit Rate | The rate of transfer of a bit of information from one network device to another |
| BPSK | Binary phase shift keying |
| BRAN | Broadband Radio Access Networks |
| Channel-ization | Bandwidth of each channel and number of channels that can be contained in the RF bandwidth allocation |
| CSMA/CA | Carrier sensing multiple access with collision avoidance |
| DFS | Dynamic frequency selection |
| EIRP | Effective Isotropic Radiated Power |
| ETSI | European Telecommunications Standards Institute |
| Frequency Band | Nominal operating spectrum of operation |
| HIPERLAN2 | High Performance Radio LAN 2Institute of Electrical and Electronics Engineers |
| HiSWAN | Hi Speed Wireless Access Networks |
| HSWA | High Speed Wireless Access |
| IEEE | Institute of Electrical and Electronics Engineers |
| LAN | Local Area Network |
| Modulation | The method used to put information onto an RF carrier |
| MMAC | Multimedia Mobile Access Communication |
| OFDM | Orthagonal Frequency Division Multiplexing |
| PSD | Power Spectral Density |
| RF | Radio Frequency |
| RLAN | Radio Local Area Network |
| TPC | Transmit Power Control |
| Tx power | (Transmitter Power) – RF power in Watts produced by the transmitter |

TABLE  1 (end)

TABLE  1 (end)

TABLE  2

Methods of multiple access and modulation techniques

| Frequency band | Multiple access | Modulation technique |
| --- | --- | --- |
| UHF | CSMA/CA | CCK |
|  | FDMA | PBCC |
|  | TDMA |  |
|  | SSMA-DS |  |
|  | SSMA-FH |  |
| SHF | CSMA/CA | GMSK/FSK |
|  | FDMA | BPSK-OFDM |
|  | TDMA-FDD | QPSK-OFDM |
|  | TDMA-TDD | BPSK 8-PSK-OFDM 16-QAM-OFDM |
|  | TDMA/EY-NPMA | 64-QAM-OFDM |

TABLE  3(#)

RLAN characteristics for frequency sharing *

| Characteristics |  | 802.11- 1999 (R2003) (IEEE 802.11b) |  | 802.11- 1999 (R2003) (IEEE 802.11a(1),j) |  | 802.11- 1999 (R2003) (IEEE 802.11g (1)) |  | ETSI BRAN HIPERLAN 2 (1), (2) |  | MMAC HSWA HiSWAN a(1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| General |  |  |  |  |  |  |  |  |  |  |
| Frequency band |  | 2 400-2 483.5 MHz |  | 4 900-5 000 MHz(Note 3) 5 150-5 250 MHz 5 250-5 350 MHz(3) 5 470-5 725 MHz 5 725-5 825 MHz [5 725-5 850 MHZ (NOTE 4) |  | 2 400- 2 483.5 MHz |  | 5 150-5 350 and 5 470-5 725 MHz(3) |  | 5 150 to 5 250 MHz(3), (6) |
| Channelization |  | 5 MHz |  | 5 MHz |  | 5 MHz |  | 20 MHz |  | 20 MHz |
| Antenna |  | Various |  | Various |  | Various |  |  |  |  |
| Gain approximate |  | 0-6 dBi (Omni) |  | 0-6 dBi (Omni) |  | 0-6 dBi (Omni) |  |  |  |  |
| Radiation Pattern |  | Omni Directional |  | Omni Directional |  | Omni Directional |  |  |  |  |
| Occupied Bandwidth |  | 802.11b mask (Figure 2) |  | OFDM mask (Figure 1) |  | OFDM mask (Figure 1) |  | OFDM mask (Figure 1) |  | OFDM mask (Figure 1) |
| Transmitter |  |  |  |  |  |  |  |  |  |  |
| Tx Power |  | 1 000mW. 8 dBm/3KHz |  | 4 900-5 000 MHz (see ‘j’ revision) 5 150-5 250 MHz 50 mW 5 250-5 350 MHz 5 470-5 575 MHz 200 mW 11 dBm/MHz (8) 5 725-5 825 MHz 1000 mW 17 dBm/MHz 5 725-5 850 MHz 1 000 mW 8 dBm/3KHz |  | 1 000 mW 8 dBm/3KHz |  | 5 150-5 350 MHz 200 mW maximum mean e.i.r.p and use of transmitter power control 5 470-5 725 MHz 1 W maximum mean e.i.r.p. and use of transmitter power control |  | 5 150-5 250 MHz 10 mW/MHz e.i.r.p(6) |

| Transmitter |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Interference Mitigation | CSMA/CA | CSMA/CA | CSMA/CA | DFS/TPC |  |
| Receiver |  |  |  |  |  |
| Sensitivity | Listed in Standard | Listed in Standard | Listed in Standard |  |  |

TABLE  3 (end)

|  |  | - | - | - | - |
| --- | --- | --- | --- | --- | --- |
| (1) Parameters for the physical layer are common between IEEE 802.11a and ETSI BRAN HIPERLAN 2 and HiSWANa. (2) WATM (Wireless ATM) and advanced IP with QoS are intended for use over ETSI BRAN HIPERLAN 2 physical transport. (3) For the band 5 150 to 5 250 MHz, No. 5.447 of the Radio Regulations (RR) applies. (4) This requirement refers to FCC 15.247 in the United States of America. (5) This requirement refers to EUROPE ETS 300-328. (6) This requirement refers to JAPAN MPHPT ordinance for Regulating Radio Equipment, Articles 49-20 and 49-21. (7) All values from FCC amendment of the Commission’s Rules to Docket No. 96-102 provide for operation of unlicensed NII (RM-8648) devices in the 5 GHz frequency range (RM-865). Also reflected in Canadian Radio Standard Specification RSS-210. (8) Some administrations have higher power . (#) Note that some administrations regulate transmitter power vs. EIRP. Further it should be recognized that power or EIRP limits may vary by region or by administration. NOTE 3. 4 900-5 000 MHz Power levels and channels specified in 802.11- 1999 (R2003)revj are used by some administrations Band : 4 900 - 5 091 MHz - Fixed wireless access-, licensed- <250mW EIRP and <50 mW/MHz EIRP for licensed access Band : 4 900 – 5.091 MHz - Nomadic access - unlicensed -10 mW/MHz EIRP Band 5150 – 5 250 MHz – Unlicensed - 10mW/MHz EIRP NOTE 4. 5 470 – 5 725 MHz have DFS rules in regions and administrations which must be consulted. 5 725-5 825 MHz in the USA come under the UNII FCC Part 15 rules. 5 825-5 850 come under the FCC Part 18 rules which expressly forbid telecommunications in this 25 MHz. |  |  |  |  |  |

Figure 1 Spectrum mask is for systems 802.11a, 11g, HIPERLAN2 and HiSWAN

Figure 2

802.11b Transmit spectrum mask

Figure 3

Spectrum masks for IEEE 802.11 a, b, g

Annex 1
Additional information on RLANS

The HYPERLAN2 standards are TS 101 475 for the physical layer & TS 101 761 for the DLC Layer, and these can be downloaded from the ETSI Publications Download Area at: http//:etsi.org/pda/queryform.asp

The IEEE 802.11 standards can be downloaded from http://standards.ieee.org/get802/

IEEE 802.11 has developed a set of standards for RLANs, 802.11- 1999 (R2003), which have been harmonized with IEC/ISO. The medium access control (MAC) and physical characteristics for wireless local area networks (LANs) are specified in ISO/IEC 8802-11:2005, which is part of a series of standards for local and metropolitan area networks. The medium access control unit in ISO/IEC 8802-11:2005 is designed to support physical layer units as they may be adopted dependent on the availability of spectrum. ISO/IEC 8802-11:2005 contains five physical layer units:  four radio units, operating in the 2 400-2 500 MHz band and in the bands comprising 5.15-5.25 GHz, 5.25-5.35 GHz, 5. 47-5 725 GHz,and 5.725-5.825 GHz, and one baseband infrared (IR) unit. One radio unit employs the frequency-hopping spread spectrum (FHSS) technique, two employ the direct sequence spread spectrum (DSSS) technique, and another employs the orthogonal frequency division multiplexing (OFDM) technique.

1.	ISO/IEC 8802-11:2005, Information technology – Telecommunications and information exchange between systems – Local and metropolitan area networks – Specific requirements – Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications

--

-

-

--

is about 20 dB under the frequency selective fading channel with 300 ns delay spread. The proposed physical layer approach allows us to use this high bit rate RLAN system not only in indoor areas but also outdoor areas forming parts of locations such as universities, factories, and shopping malls, etc.

TABLE  4

Major simulation parameters

| Raw data rate | 26.6 Mbit/s |
| --- | --- |
| Modulation/detection | DQPSK/differential detection |
| FFT size | 64 samples |
| Number of subcarriers |  |

-

*	This Recommendation was jointly developed by Radiocommunication Study Groups 8 and 9, and future revisions should be undertaken jointly.

*	*	This Recommendation should be brought to the attention of Telecommunication Standardization Study Group 17, and Radiocommunication Study Groups 3 and 4.

*

ISO/IEC 8802-11:2005, Information technology – Telecommunications and information exchange between systems – Local and metropolitan area networks – Specific requirements – Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications.