| Received: 10 January 2007 TECHNOLOGY | INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- | --- |
|  | RADIOCOMMUNICATION STUDY GROUPS | Revision 1 to Document 8F/1079-E |
|  |  | 10 January 2007 |
|  |  | English only |

Subject:		Question ITU-R 229-1/8

WiMAX Forum

ADDITIONAL TECHNICAL DETAILS SUPPORTING IP-OFDMA 
AS AN IMT-2000 TERRESTRIAL RADIO INTERFACE

In Document 8F/1079, WiMAX Forum submitted detailed technical material in support of inclusion of IP-OFDMA as an IMT-2000 terrestrial radio interface. Some material, namely tables of Generic Requirements and Objectives as per Recommendation ITU-R M.1225, were inadvertently left out during editorial handling of the document before submission to WP 8F. The attachment to this document presents a revision to 8F/1079 and includes those tables. Moreover, WiMAX Forum has taken this opportunity to fix some editorial errors in 8F/1079 as well as addition of more detailed information in line with the methodology outlined in Recommendation ITU-R M.1225. Below is a summary of changes to 8F/1079.

- Additional information on support of multiple antenna technologies in Section 1.3.5.

- A new Section 2.2 containing tables of Generic Requirements and Objectives.

- Link budgets in Section 2.3.4 based on assumptions of Recommendation ITU-R M.1225.

- Additional information in technology description template contained in Section 2.

- Additional information in Section 3, self-evaluation.

- Minor editorial corrections and clarifying text throughout the document.

It should be noted that, once all changes are accepted, the attachment to this document could replace 8F/1079.

Attachment

WiMAX Forum

ADDITIONAL TECHNICAL DETAILS SUPPORTING IP-OFDMA 
AS AN IMT-2000 TERRESTRIAL RADIO INTERFACE

# Introduction

The Institute of Electrical and Electronics Engineers, Inc. (IEEE) has submitted, in Document 8F/1065, a proposal to add the terrestrial air interface IP-OFDMA into Recommendation ITU-R M.1457, “Detailed specifications of the radio interfaces of International Mobile Telecommunications-2000 (IMT-2000)”, in accordance with the ITU-R process for the addition of new radio interface technologies. Document 8F/1065, however, states:

“It should be noted that Section 3 does not contain all the information required, since it is expected that other organizations will provide the complementary material, including the evaluation.”

# Proposal

The WiMAX Forum® hereby respectfully submits supporting material to complement the IEEE’s submission of IP-OFDMA Radio Transmissions Technology (RTT). The supporting material is divided into three sections.

Section 1 contains additional information on IP-OFDMA technology and the standard it is based upon. It also includes a description of the WiMAX Network Reference Model developed by the WiMAX Forum and used here as a framework for evaluating the IP-OFDMA radio interface.

Section 2 contains additional technical material to complement the technology description template in Document 8F/1065, as required by Recommendation ITU-R M.1225 and the update process of Recommendation ITU-R M.1457 as described in Circular Letter 8/LCCE/95. In order to facilitate the process, this section also includes technical material on system capacity and coverage of IP-OFDMA.

Section 3contains a self-evaluation of the proposed IP-OFDMA RTT, as required by the update process of Recommendation ITU-R M.1457 described in Circular Letter 8/LCCE/95.

The WiMAX Forum is looking forward to a continued cooperation with ITU-R Working Party 8F on this and other matters of mutual interest. If further information is required, we will provide it for the May 2007 meeting of Working Party 8F or earlier. The WiMAX Forum requests expeditious inclusion of the proposed IP-OFDMA RTT in the draft revision to Recommendation ITU-R M.1457-6 in time for its planned approval at the next Study Group 8 meeting in June 2007.

# Abbreviations

Table 1

Abbreviations

| Abbreviation | Description |
| --- | --- |
| 3GPP | 3G Partnership Project |
| 3GPP2 | 3G Partnership Project 2 |
| AAS | Adaptive Antenna System also Advanced Antenna System |
| ACK | Acknowledge |
| AES | Advanced Encryption Standard |
| AES-CCM | AES Counter mode with CBC-MAC |
| AG | Absolute Grant |
| AMC | Adaptive Modulation and Coding |
| A-MIMO | Adaptive Multiple Input Multiple Output (Antenna) |
| ASM | Adaptive MIMO Switching |
| ARQ | Automatic Repeat reQuest |
| ASN | Access Service Network |
| ASP | Application Service Provider |
| BE | Best Effort |
| CC | Chase Combining (also Convolutional Code) |
| CCM | Counter with Cipher-block chaining Message authentication code |
| CINR | Carrier to Interference + Noise Ratio |
| CMAC | block Cipher-based Message Authentication Code |
| CP | Cyclic Prefix |
| CQI | Channel Quality Indicator |
| CQICH | Channel Quality Indicator CHannel |
| CSN | Connectivity Service Network |
| CSTD | Cyclic Shift Transmit Diversity |
| CTC | Convolutional Turbo Code |
| DL | Downlink |
| EAP | Extensible Authentication Protocol |
| EAP-AKA | EAP-Authentication and Key Agreement |
| EAP-TLS | EAP-Translation Layer Security |
| MSCHAPv2 | Microsoft Challenge-Handshake Authentication Protocol v2 |
| EESM | Exponential Effective SIR Mapping |
| EIRP | Effective Isotropic Radiated Power |
| ErtVR | Extended Real-Time Variable Rate |
| FBSS | Fast Base Station Switch |
| FCH | Frame Control Header |
| FDD | Frequency Division Duplex |
| FFT | Fast Fourier Transform |
| FTP | File Transfer Protocol |
| FUSC | Fully Used Subchannels |
| HARQ | Hybrid Automatic Repeat reQuest |
| HHO | Hard Hand-Off |
| HMAC | keyed Hash Message Authentication Code |
| HO | Hand-Off |
| HTTP | Hyper Text Transfer Protocol |
| IE | Information Element |
| IEEE | The Institute of Electrical and Electronics Engineers, Inc. |
| IEFT | Internet Engineering Task Force |
| IFFT | Inverse Fast Fourier Transform |
| IP | Internet Protocol |
| IR | Incremental Redundancy |
| ISI | Inter-Symbol Interference |
| LDPC | Low-Density-Parity-Check |
| LOS | Line of Sight |
| MAC | Media Access Control |
| MAI | Multiple Access Interference |
| MAN | Metropolitan Area Network |
| MAP | Media Access Protocol |
| MBS | Multicast and Broadcast Service |
| MIMO | Multiple Input Multiple Output (Antenna) |
| MMS | Multimedia Message Service |
| MPLS | Multi-Protocol Label Switching |
| MS | Mobile Station |
| MSO | Multi-Services Operator |
| NACK | Not Acknowledge |
| NAP | Network Access Provider |
| NLOS | Non Line-of-Sight |
| NRM | Network Reference Model |
| nrtPS | Non-Real-Time Packet Service |
| NSP | Network Service Provider |
| OFDM | Orthogonal Frequency Division Multiplex |
| OFDMA | Orthogonal Frequency Division Multiple Access |
| PER | Packet Error Rate |
| PF | Proportional Fair (Scheduler) |
| PKM | Public Key Management |
| PKM-REQ/RSP | PKM Request/Response |
| PUSC | Partially Used Subchannels |
| QAM | Quadrature Amplitude Modulation |
| QPSK | Quadrature Phase Shift Keying |
| RAN | Radio Access Network |
| RG | Relative Grant |
| RR | Round Robin (Scheduler) |
| RRI | Reverse Rate Indicator |
| RTG | Receive/transmit Transition Gap |
| RTT | Radio Transmissions Technology |
| rtPS | Real-Time Packet Service |
| SDMA | Space (or Spatial) Division (or Diversity) Multiple Access |
| SF | Spreading Factor |
| SFN | Single Frequency Network |
| SGSN | Serving GPRS Support Node |
| SHO | Soft Hand-Off |
| SIM | Subscriber Identify Module |
| SINR | Signal to Interference + Noise Ratio |
| SIMO | Single Input Multiple Output (Antenna) |
| SISO | Single Input Single Output (Antenna) |
| SLA | Service Level Agreement |
| SM | Spatial Multiplexing |
| SMS | Short Message Service |
| SNR | Signal to Noise Ratio |
| S-OFDMA | Scalable Orthogonal Frequency Division Multiple Access |
| SS | Subscriber Station |
| STC | Space Time Coding |
| TDD | Time Division Duplex |
| TEK | Traffic Encryption Key |
| TTG | Transmit/receive Transition Gap |
| TTI | Transmission Time Interval |
| TU | Typical Urban (as in channel model) |
| UE | User Equipment |
| UGS | Unsolicited Grant Service |
| UL | Uplink |
| UMTS | Universal Mobile Telephone System |
| USIM | Universal Subscriber Identify Module |
| VoIP | Voice over Internet Protocol |
| VPN | Virtual Private Network |
| VSF | Variable Spreading Factor |
| WAP | Wireless Application Protocol |
| WiMAX | Worldwide Interoperability for Microwave Access |

# 1	IP-OFDMA Detailed Technology Description

The IEEE 802.16 Working Group develops and supports the IEEE 802.16 air interface standard for Broadband Wireless Access systems. The amendment IEEE Std 802.16e-2005  along with the base IEEE Std 802.16-2004  provides the basis for the IP-OFDMA air interface for combined fixed and mobile broadband wireless access.

IEEE Std 802.16 offers a flexible set of parameters and features to meet a range of global requirements. Due to this flexibility, interoperability with respect to the required features needs to be to ensured. Interoperability testing is a key function of the WiMAX Forum. Therefore, the WiMAX Forum has developed profiles specifying particular features and parameter sets from IEEE 802.16 sufficient to ensure interoperability.

The IP-OFDMA RTT is consistent with the WiMAX Forum Mobile System Profile being commercialized by members of WiMAX Forum under the name “Mobile WiMAX TM”. The WiMAX Forum Mobile System Profile  as illustrated in Figure 1, is derived from the mandatory and optional feature sets described in IEEE Std 802.16. This profile is used for air interface certification to foster global interoperability. WiMAX Forum Mobile profiles include recommended 5 and 10 MHz bandwidth, aligned with IP-OFDMA proposal, for global deployment.

Figure 1

WiMAX Forum Mobile System Profile

IEEE® 802.16e Mobile Broadband Wireless Amendment

IEEE® 802.16-2004 Fixed Broadband Wireless Standard

WiMAX Forum Mobile System Profile

Mandatory

and Optional Features

The WiMAX Mobile System Profile supports the deployment of fully interoperable systems compatible with IP-OFDMA. The profile includes optional Base Station features providing flexibility for various deployment scenarios and regional requirements to enable optimization for capacity, coverage, etc.

## 1.1	Mobile WiMAX Network Architecture

The IP-OFDMA radio interface is suitable for use in an all-IP architecture, with support for IP-based packet services.  This allows for scalability and rapid deployment since the networking functionality is primarily based on software services.

In order to deploy successful and operational commercial systems, there is need for support beyond the IEEE 802.16 air interface specifications, which only address layers 1 and 2 (PHY and MAC). The WiMAX Forum specifies the Mobile WiMAX Network Architecture  describing the upper layer of the Radio Access Network and Core Network.  Furthermore, the systems can also operate with core network of other IMT-2000 systems.

### 1.1.1	Architecture Principles

The following basic tenets have guided the Mobile WiMAX Network Architecture development.

- The architecture is based on a packet-switched framework, including native procedures based on IEEE Std 802.16, appropriate IETF RFCs and Ethernet standards.

- The architecture permits decoupling of access architecture (and supported topologies) from connectivity IP service.  Network elements of the connectivity system are independent of the IEEE 802.16 radio specifics.

- The architecture allows modularity and flexibility to accommodate a broad range of deployment options such as:

- Small-scale to large-scale (sparse to dense radio coverage and capacity) networks

- Urban, suburban, and rural radio propagation environments

- Licensed and/or licensed-exempt frequency bands

- Hierarchical, flat, or mesh topologies, and their variants

- Co-existence of fixed, nomadic, portable and mobile usage models

Support for Services and Applications: The end-to-end Mobile WiMAX Network Architecture includes a) Support of voice, multimedia services and other mandated regulatory services such as emergency services and lawful interception, b) Access to a variety of independent Application Service Provider (ASP) networks in an neutral manner, c) Mobile telephony communications using VoIP, d) Support interfacing with various interworking and media gateways permitting delivery of incumbent/legacy services translated over IP (for example, SMS over IP, MMS, WAP) to WiMAX access networks and e) Support delivery of IP Broadcast and Multicast services over WiMAX access networks.

Interworking and Roaming is another key strength of the end-to-end Mobile WiMAX Network Architecture with support for a number of deployment scenarios. In particular, there will be support of a)  Loosely-coupled interworking with existing wireless networks such as those specified in 3GPP and 3GPP2 or existing wireline networks such as DSL and MSO, with the interworking interface(s) based on a standard IETF suite of protocols, b) Global roaming across WiMAX operator networks, including support for credential reuse, consistent use of AAA for accounting and billing, and consolidated/common billing and settlement, c) A variety of user authentication credential formats such as subscriber identify modules (SIM/USIM, R-UIM), username/password, digital certificates.

## 1.2	WiMAX Network Reference Model

IEEE Std 802.16 specifies a radio interface but not the network in which it is to be used, instead leaving an open interface to higher network layers. The WiMAX Forum specifies the Network Reference Model (NRM) to describe a practical and functional network making use of the IP-OFDMA air interface. This NRM is described here because it serves as a framework for evaluating the performance of the IP-OFDMA radio interface.

The NRM is a logical representation of the network architecture. The NRM identifies functional entities and reference points over which interoperability is achieved between functional entities.  The architecture has been developed with the objective of providing unified support of functionality needed in a range of network deployment models and usage scenarios (ranging from nomadicity to full mobility).

Figure 2 illustrates the NRM, consisting of the logical entities MS, ASN, and CSN, as well as clearly identified reference points for interconnection of the logical entities. The figure depicts the key normative reference points R1-R5. Each of the entities, MS, ASN and CSN, represents a grouping of functional entities. Each of these functional entities may be realized in a single physical device or may be distributed over multiple physical devices according to allocation defined by ASN profiles.

The intent of the NRM is to allow multiple implementation options for a given functional entity, and yet achieve interoperability among different realizations of functional entities.  Interoperability is based on the definition of communication protocols and data plane treatment between functional entities to achieve an overall end-to-end function, for example, security or mobility management. Thus, the functional entities on either side of a reference point represent a collection of control and bearer plane end-points.

Figure 2

WiMAX Network Reference Model

ASN

CSN

CSN

SS/MS

NAP

Home NSP

Visited NSP

R2

R2

R4

R1

R3

R5

Another ASN

ASP Network or Internet

ASP Network or Internet

The ASN defines a logical boundary and represents a convenient way to describe aggregation of functional entities and corresponding message flows associated with the access services. The ASN represents a boundary for functional interoperability with WiMAX clients, connectivity service functions, and aggregation of functions embodied by different vendors. Mapping of functional entities to logical entities within ASNs as depicted in the NRM may be performed in different ways. The Connectivity Service Network (CSN) is defined as a set of network functions that provide IP connectivity services to the subscriber stations. A CSN may comprise network elements such as routers, AAA proxy/servers, user databases and Interworking gateway devices. Figure 3 provides a more basic view of the many entities within the functional groupings of ASN and CSN.

Figure 3

ASN and CSN Entities

Some general tenets have guided the development of the Network Architecture and include the following: a) Logical separation of IP addressing, routing and connectivity management procedures and protocols, to enable use of the access architecture primitives in standalone and inter-working deployment scenarios, b) Support for sharing of ASN(s) of a NAP among multiple NSPs, c) Support of a single NSP providing service over multiple ASN(s) – managed by one or more NAPs, d) Support for the discovery and selection of accessible NSPs by an MS, e) Support of NAPs that employ one or more ASN topologies, f) Support of access to incumbent operator services through internetworking functions as needed, g) Specification of open and well-defined reference points between various groups of network functional entities (within an ASN, between ASNs, between an ASN and a CSN, and between CSNs), and in particular between an MS, ASN and CSN to enable multi-vendor interoperability, h) Support for evolution paths between the various usage models subject to reasonable technical assumptions and constraints, i) Enabling different vendor implementations based on different combinations of functional entities on physical network entities, as long as these implementations comply with the normative protocols and procedures across applicable reference points, as defined in the network specifications and j) Support for the most basic scenario of a single operator deploying an ASN together with a limited set of CSN functions, so that the operator can offer basic Internet access service without consideration for roaming or interworking.

The WIMAX architecture also supports IP services, in a standard mobile IP compliant network. The flexibility and interoperability supported by this network architecture provides operators with the opportunity for a multi-vendor implementation of a network even with a mixed deployment of distributed and centralized ASN’s in the network. The WiMAX network architecture has the following major features:

Security

The end-to-end Network Architecture is based upon a security framework that is independent of the ASN topology and applies consistently across both new and internetworking deployment models and various usage scenarios. In particular, it supports: a) Strong mutual device authentication between an MS and the network, based on the IEEE 802.16 security framework, b) All commonly deployed authentication mechanisms and authentication in home and visited operator network scenarios based on a consistent and extensible authentication framework, c) Data integrity, replay protection, confidentiality and non-repudiation using applicable key lengths, d) Use of MS initiated/terminated security mechanisms such as Virtual Private Networks (VPNs), and e) Standard secure IP address management mechanisms between the MS and its home or visited NSP.

Mobility and Handovers

The end-to-end Network Architecture has extensive capabilities to support mobility and handovers. It a) supports IPv4 or IPv6 based mobility management. Within this framework, and as applicable, the architecture accommodates MS equipment with multiple IP addresses and simultaneous IPv4 and IPv6 connections, b) supports roaming between NSPs, c) utilizes mechanisms to support seamless handovers at up to vehicular speeds— satisfying well-defined bounds of service disruption. Some of the additional capabilities for mobility include the support of: i) dynamic and static home address configurations, ii) dynamic assignment of the Home Agent in the service provider network as a form of route optimization, as well as in the home IP network as a form of load balancing and iii) dynamic assignment of the Home Agent based on policies.

Scalability, Extensibility, Coverage and Operator Selection

The end-to-end Network Architecture has extensive support for scalable, extensible operation and flexibility in operator selection.  In particular, it  a) enables a user to manually or automatically select from available NAPs and NSPs,  b) enables ASN and CSN system designs that easily scale upward and downward – in terms of coverage, range or capacity, c) accommodates a variety of ASN topologies - including hub-and-spoke, hierarchical, and/or multi-hop interconnects, d) accommodates a variety of backhaul links, both wireline and wireless with different latency and throughput characteristics, e) supports incremental infrastructure deployment, f) supports phased introduction of IP services that in turn scale with increasing number of active users and concurrent IP services per user, g) supports the integration of base stations of varying coverage and capacity - for example, pico, micro, and macro base stations and  e) supports flexible decomposition and integration of ASN functions in ASN deployments in order to enable use of load balancing schemes for efficient use of radio spectrum and network resources.

Additional features pertaining to manageability and performance of the Network Architecture include: a) Support for a variety of online and offline client provisioning, enrollment, and management schemes based on open, broadly deployable, IP-based, industry standards, b) Accommodation of Over-The-Air (OTA) services for MS terminal provisioning and software upgrades, and c) Accommodation of the use of header compression/suppression and/or payload compression for efficient use of the radio resources.

Multi-Vendor Interoperability

Another key aspect of the Network Architecture is the support of interoperability between equipment from different manufacturers within an ASN and across ASNs. This includes interoperability between: a) BS and backhaul equipment within an ASN, and b) Various ASN elements (possibly from different vendors) and CSN, with minimal or no degradation in functionality or capability of the ASN.

Quality of Service

The Network Architecture has provisions for support of the QoS mechanisms defined in IEEE Std 802.16. In particular, it enables flexible support of simultaneous use of a diverse set of IP services. The architecture supports: a) differentiated levels of QoS, coarse-grained (per user/terminal) and/or fine-grained (per service flow), b) admission control, c) bandwidth management and d) implementation of policies as defined by various operators for QoS based on their SLAs (including policy enforcement per user and user group as well as factors such as location, time of day, etc.). Extensive use is made of standard IETF mechanisms for managing policy definition and policy enforcement between operators.

Interworking with Other Networks

The Network Architecture supports loosely coupled interworking with existing wireless or wireline core networks such as GSM/GPRS, UMTS, HSDPA, CDMA2000, RLAN, DSL, and cable modem operator networks on the basis of the IP/IETF suite of protocols.

## 1.3	Physical Layer Description

### 1.3.1	OFDMA Basics

OFDM is a multiplexing technique that subdivides the bandwidth into multiple frequency sub-carriers as shown in Figure 4. In an OFDM system, the input data stream is divided into several parallel sub-streams of reduced data rate (thus increased symbol duration) and each sub-stream is modulated and transmitted on a separate orthogonal sub-carrier. The increased symbol duration improves the robustness of OFDM to delay spread. Furthermore, the introduction of the cyclic prefix (CP) can completely eliminate Inter-Symbol Interference (ISI) as long as the CP duration is longer than the channel delay spread. The CP is typically a repetition of the last samples of data portion of the block that is appended to the beginning of the data payload as shown in Figure 5. The CP prevents inter-block interference and makes the channel appear circular and permits low-complexity frequency domain equalization.  A perceived drawback of CP is that it introduces overhead, which effectively reduces bandwidth efficiency. While the CP does reduce bandwidth efficiency somewhat, the impact of the CP is similar to the “roll-off factor” in raised-cosine filtered single-carrier systems. Since OFDM signal power spectrum has a very sharp fall of at the edge of channel, a larger fraction of the allocated channel bandwidth can be utilized for data transmission, which helps to moderate the loss in efficiency due to the cyclic prefix.

Figure 4

Basic Architecture of an OFDM System

OFDM exploits the frequency diversity of the multipath channel by coding and interleaving the information across the sub-carriers prior to transmissions. OFDM modulation can be realized with efficient Inverse Fast Fourier Transform (IFFT), which enables a large number of sub-carriers with low complexity. In an OFDM system, resources are available in the time domain by means of OFDM symbols and in the frequency domain by means of sub-carriers. The time and frequency resources can be organized into subchannels for allocation to individual users. Orthogonal Frequency Division Multiple Access (OFDMA) is a multiple-access/multiplexing scheme that provides multiplexing operation of data streams corresponding to multiple users onto the downlink subchannels. It also supports multiple access of various users by means of uplink subchannels.

Figure 5

Insertion of Cyclic Prefix (CP)

### 1.3.2	OFDMA Symbol Structure and Subchannelization

The OFDMA symbol structure consists of three types of sub-carriers as shown in .Figure 6

OFDMA Sub-Carrier Structure

- Data sub-carriers for data transmission.

- Pilot sub-carriers for estimation and synchronization purposes.

- Null sub-carriers for no transmission; used for guard band and zero Hertz sub-carriers.

Active (data and pilot) sub-carriers are grouped into subsets of sub-carriers called subchannels. The IP-OFDMA PHY [1] supports subchannelization in both DL and UL. The minimum frequency-time resource unit of subchannelization is one slot, which is equal to 48 data tones (sub-carriers).

There are two types of sub-carrier permutations for subchannelization; diversity and contiguous. The diversity permutation draws sub-carriers pseudo-randomly to form a subchannel. It provides frequency diversity and inter-cell interference averaging.  The diversity permutations include DL FUSC, DL PUSC and UL PUSC and additional optional permutations. With DL PUSC, for each pair of OFDM symbols, the available or usable sub-carriers are grouped into clusters containing 14 contiguous sub-carriers per symbol, with pilot and data allocations in each cluster in the even and odd symbols as shown in .

Figure 7

DL Frequency Diverse Subchannel

A re-arranging scheme is used to form groups of clusters such that each group is made up of clusters that are distributed throughout the sub-carrier space. A subchannel in a group contains two (2) clusters and is comprised of 48 data sub-carriers and eight (8) pilot sub-carriers. The data subcarriers in each group are further permuted to generate subchannels in the group. Therefore, only the pilot positions in the cluster as shown in . The data subcarriers in the cluster are distributed to multiple subchannels.

Analogous to the cluster structure for DL, a tile structure is defined for the UL PUSC whose format is shown in Figure 8.

Figure 8

Tile Structure for UL PUSC

The available sub-carrier space is split into tiles and six (6) tiles, chosen from across the entire spectrum by means of a re-arranging/permutation scheme, are grouped together to form a slot. The slot is comprised of 48 data sub-carriers and 24 pilot sub-carriers in 3 OFDM symbols.

The contiguous permutation groups a block of contiguous sub-carriers to form a subchannel. The contiguous permutations include DL AMC (Adaptive Modulation and Coding) and UL AMC, and have the same structure. A bin consists of 9 contiguous sub-carriers in a symbol, with 8 assigned for data and one assigned for a pilot.  A slot in AMC is defined as a collection of bins of the type (N x M = 6), where N is the number of contiguous bins and M is the number of contiguous symbols. Thus the allowed combinations are (6 bins, 1 symbol), (3 bins, 2 symbols), (2 bins, 3 symbols) or (1 bin, 6 symbols). AMC permutation enables multi-user diversity by choosing the subchannel with the best frequency response.

In general, diversity sub-carrier permutations perform well in mobile applications while contiguous sub-carrier permutations are well suited for fixed, nomadic, or low mobility environments. These options enable the system designer to trade-off mobility for throughput.

Following figure demonstrates the physical and Logical subchannel allocation in a OFDMA frame.

Figure 9

Physical and Logical Subchannel allocation

### 1.3.3	Scalable OFDMA

IP-OFDMA mode is based upon the concept of Scalable OFDMA. The scalability is supported by adjusting the FFT size while fixing the sub-carrier frequency spacing at 10.94 kHz. Since the resource unit sub-carrier bandwidth and symbol duration is fixed, the impact to higher layers is minimal when scaling the bandwidth. The IP-OFDMA parameters are listed in Table 2.

Table 2

OFDMA Scalability Parameters

| Parameters | Values |  |
| --- | --- | --- |
| System Channel Bandwidth (MHz) | 5 5.6 512 8 | 10 11.2 1024 16 |
| Sampling Frequency (Fp in MHz) |  |  |
| FFT Size (NFFT) |  |  |
| Number of Subchannels |  |  |
| Sub-Carrier Frequency Spacing | 10.94 kHz |  |
| Useful Symbol Time (Tb = 1/f) | 91.4 µs |  |
| Guard Time (Tg =Tb/8) | 11.4 µs |  |
| OFDMA Symbol Duration (Ts = Tb + Tg) | 102.9 µs |  |
| Number of OFDMA Symbols (5 ms Frame) | 48 (including ~1.6 symbols for TTG/RTG) |  |

### 1.3.4	TDD Frame Structure

The IP-OFDMA PHY makes use of Time Division Duplexing. To counter interference issues, TDD does require system-wide synchronization; nevertheless, TDD has numerous advantages:

- TDD enables adjustment of the downlink/uplink ratio to efficiently support asymmetric downlink/uplink traffic, while with FDD, downlink and uplink always have fixed and, generally, equal DL and UL bandwidths. As shown in Table 3, recommended number of UL/DL OFDM symbols can flexibly realize a range of asymmetric downlink/uplink traffic ratio.

Table 3

Number of OFDM Symbols in DL and UL

| Description | Base Station Values |
| --- | --- |
| Number of OFDM Symbols in DL and UL for 5 and 10 MHz BW | (35: 12), (34: 13), (33: 14), (32: 15), (31: 16), (30: 17), (29: 18), (28: 19), (27: 20), (26: 21) |

- TDD assures channel reciprocity for better support of link adaptation, MIMO and other closed loop advanced antenna technologies.  Also, TDD is the preferred mode of operation with respect to the beamforming systems using phased array antennas.

- Unlike FDD, which requires a pair of channels, TDD only requires a single channel for both downlink and uplink, providing greater flexibility for adaptation to varied global spectrum allocations.

- Transceiver designs for TDD implementations are less complex.

Figure 10 illustrates the OFDM frame structure for a TDD implementation. Each frame is divided into DL and UL sub-frames, separated by Transmit/Receive and Receive/Transmit Transition Gaps (TTG and RTG, respectively) to prevent DL and UL transmission collisions. In a frame, the following control information is used:

- Preamble: The preamble, used for synchronization, is the first OFDM symbol of the frame.

- Frame Control Header (FCH): The FCH follows the preamble. It provides the frame configuration information, such as MAP message length, coding scheme, and usable subchannels.

- DL-MAP and UL-MAP: The DL-MAP and UL-MAP provide subchannel allocation and other control information for the DL and UL sub-frames respectively.

- UL Ranging: The UL ranging subchannel is allocated for MSs to perform closed-loop time, frequency, and power adjustment as well as bandwidth requests. Four types of ranging are defined. The different types of ranging are identified by a code and a 2D region in the UL subframe.

  - Initial Ranging- when MS enters (or re-enters) the network,

  - Periodic Ranging once the connection is set up between the MS and the BS,

  - Hand Over Ranging (in case of Hard HO in drop situations) and

  - Bandwidth Request.

- UL CQICH: The UL CQICH channel is allocated for the MS to feedback channel-state information.

- UL ACK: The UL ACK is allocated for the MS to feedback DL HARQ acknowledgement.

Figure 10

IP-OFDMA Frame Structure

### 1.3.5	Other Advanced PHY Layer Features

Adaptive modulation and coding, HARQ, CQICH, and multiple antenna technologies provide enhanced coverage and capacity in mobile applications.

Support for QPSK, 16QAM and 64QAM are mandatory in the DL. In the UL, 64QAM is optional. Both Convolutional Code (CC) and Convolutional Turbo Code (CTC), with variable code rate and repetition coding, are supported. Table 4 summarizes the coding and modulation schemes supported in IP-OFDMA.

Table 4

Supported Coding and Modulation Schemes

|  |  | DL | UL |
| --- | --- | --- | --- |
| Modulation |  | QPSK, 16QAM, 64QAM | QPSK,16QAM, (64QAM optional) |
| Rate | CC | 1/2, 2/3, 3/4 | 1/2, 2/3, 3/4 |
|  | CTC | 1/2, 2/3, 3/4, 5/6 | 1/2, 2/3, 5/6 |
|  | Repetition | x2, x4, x6 | x2, x4, x6 |

The combinations of various modulations and code rates provide a fine resolution of data rates, as shown in Table 4 Table 6 assumes PUSC subchannels with frame duration of 5 milliseconds. Each frame has 48 OFDM symbols, with 44 OFDM symbols available for data transmission. The highlighted values indicate data rates for optional 64QAM in the UL.

Table 5

IP-OFDMA PHY Numerology

| Parameter | Downlink | Uplink | Downlink | Uplink |
| --- | --- | --- | --- | --- |
| System Bandwidth | 5 MHz |  | 10 MHz |  |
| FFT Size | 512 |  | 1024 |  |
| Null Sub-Carriers | 92 | 104 | 184 | 184 |
| Pilot Sub-Carriers | 60 | 136 | 120 | 280 |
| Data Sub-Carriers | 360 | 272 | 720 | 560 |
| Subchannels | 15 | 17 | 30 | 35 |
| Symbol Period, TS | 102.9 µs |  |  |  |
| Frame Duration | 5 ms |  |  |  |
| OFDM Symbols/Frame | 48 (including ~1.6 symbols for TTG/RTG) |  |  |  |
| Data OFDM Symbols | 44 |  |  |  |

Table 6

IP-OFDMA PHY Data Rates with PUSC Subchannel

| Modulation | Code Rate | 5 MHz Channel |  |  | 10 MHz Channel |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Downlink Rate, Mbit/s | Uplink Rate, Mbit/s | Downlink Rate, Mbit/s |  | Uplink Rate, Mbit/s |
| QPSK | 1/2 CTC, 6x | 0.53 | 0.38 | 1.06 |  | 0.78 |
|  | 1/2 CTC, 4x | 0.79 | 0.57 | 1.58 |  | 1.18 |
|  | 1/2 CTC, 2x | 1.58 | 1.14 | 3.17 |  | 2.35 |
|  | 1/2 CTC, 1x | 3.17 | 2.28 | 6.34 |  | 4.70 |
|  | 3/4 CTC | 4.75 | 3.43 | 9.50 |  | 7.06 |
| 16QAM | 1/2 CTC | 6.34 | 4.57 | 12.07 |  | 9.41 |
|  | 3/4 CTC | 9.50 | 6.85 | 19.01 |  | 14.11 |
| 64QAM | 1/2 CTC | 9.50 | 6.85 | 19.01 |  | 14.11 |
|  | 2/3 CTC | 12.67 | 9.14 | 26.34 |  | 18.82 |
|  | 3/4 CTC | 14.26 | 10.28 | 28.51 |  | 21.17 |
|  | 5/6 CTC | 15.84 | 11.42 | 31.68 |  | 23.52 |

The base station scheduler determines the appropriate data rate (or burst profile) for each burst allocation based on the buffer size, channel propagation conditions at the receiver, etc. A Channel Quality Indicator (CQI) channel is utilized to provide channel-state information from the user terminals to the base station scheduler. Relevant channel-state information can be fed back by the CQICH including: Physical CINR, Effective CINR, MIMO mode selection and frequency selective subchannel selection. Because the implementation is TDD, link adaptation can also take advantage of channel reciprocity to provide a more accurate measure of the channel condition (such as sounding).

HARQ is enabled using N channel “Stop and Wait” protocol, which provides fast response to packet errors and improves cell edge coverage. Chase Combining and, optionally, Incremental Redundancy are supported to further improve the reliability of the retransmission. A dedicated ACK channel is provided in the uplink for HARQ ACK/NACK signaling. Multi-channel HARQ operation is supported. Multi-channel stop-and-wait ARQ with a small number of channels is an efficient, simple protocol that minimizes the memory required for HARQ and stalling. IP-OFDMA provides signaling to allow fully asynchronous operation. The asynchronous operation allows variable delay between retransmissions, which gives more flexibility to the scheduler at the cost of additional overhead for each retransmission allocation. HARQ combined together with CQICH and adaptive modulation and coding provides robust link adaptation in mobile environments at vehicular speeds in excess of 120 km/hr.

Multiple antenna technologies typically involve complex vector or matrix operations on signals due to the presence of multiple antenna links between the transmitter and receiver. OFDMA allows multiple antenna operations to be performed on a per-subcarrier basis, where the vector-channels are flat fading. This fact makes the multiple antenna signal processing manageable at both transmitter and receiver side since complex transmitter architectures and receiver equalizers are not required to compensate for frequency selective fading. Thus, OFDMA is very well-suited to support multiple antenna technologies. IP-OFDMA supports a full range of multiple antenna technologies to enhance system performance. The supported multiple antenna technologies include:

- Beamforming (BF) for both the uplink and the downlink: With BF, the system uses multiple-antennas to both receive and transmit signals to improve the coverage and capacity of the system and reduce the outage probability. The BS is usually equipped with two or more antennas, with a typical number being four antennas, and determines so-called antenna weights for both uplink reception and downlink transmission, while the MS is usually equipped with one or two antennas for downlink reception and one antenna for uplink transmission. Note that different BF techniques can be applied in IP-OFDMA since there is no limitation imposed either to the distance among the antenna elements of the BS or the algorithm employed at the BS transceiver; the possibility of beamforming the pilot subcarriers during downlink transmission (feature of dedicated pilots in the mobile WiMAX system profiles) makes the application of specific BF algorithms transparent to the MS receiver.

- Space-Time Coding (STC) for the downlink: Two-antenna transmit diversity is enabled in IP-OFDMA through the use of a space-time block coding code widely known as the Alamouti code. STC is a powerful technique for implementing open-loop transmit diversity, while its performance is further increased in IP-OFDMA since a second antenna is mandated to be present at the MS receiver. Further, STC offers favorable performance in all propagation environments, i.e., it is not constrained by the MIMO channel quality usually represented by the spread of the MIMO channel eigenvalues. As in the BF case where one spatial stream is transmitted over one OFDMA symbol per subcarrier, STC cannot lead to link throughput increase because it transmits two spatial streams over two OFDMA symbols per subcarrier.

- Spatial Multiplexing (SM) for the downlink: Spatial multiplexing is supported to apply higher peak rates and increased throughput whenever this is possible. With spatial multiplexing, two data streams are transmitted over one OFDMA symbol per subcarrier. Since the MS receiver is also equipped with two receive antennas, it can separate the two data streams to achieve higher throughput compared to single antenna, BF, and STC systems. In IP-OFDMA, with 2x2 MIMO SM increases the peak data rate two-fold by transmitting two data streams.

- Collaborative Spatial Multiplexing (CSM), also referred to as virtual spatial multiplexing, for the uplink: In the uplink, each MS is equipped with a single transmit antenna. To increase the uplink performance, two users can transmit collaboratively in the same frequency and time allocation as if two streams were spatially multiplexed from two antennas of the same user. The advantage of the uplink CSM compared to the downlink SM is related to the fact that the transmitted spatial streams are uncorrelated since they originate from spatially displaced MS’s. By additionally considering that the channel correlation factor at the BS can be kept at lower values than that at the MS receiver (space limitations at the MS usually apply leading to smaller inter-antenna distances and, thus, higher correlation values, especially if cross-polarized antennas are not employed), an improved performance of the spatial stream demultiplexing is expected in the uplink compared to the downlink.

Regarding the MIMO operation in the downlink (use of the STC and SM modes), IP-OFDMA supports adaptive switching between STC and SM to maximize the benefit of MIMO depending on the channel conditions. For instance, SM improves peak throughput. However, when channel conditions are poor, e.g., when the signal-to-interference ratio is low or the channel correlation factor is relatively high, the packet error rate (PER) can be high and thus the coverage area where the target PER is met may be limited. STC on the other hand provides large coverage regardless of the channel condition but does not improve the peak data rate. IP-OFDMA supports adaptive switching between multiple MIMO modes to maximize spectral efficiency without compromising on the coverage area.

## 1.4	MAC Layer Description

IP-OFDMA supports the delivery of broadband services, including voice, data, and video. The MAC layer can support bursty data traffic with high peak rate demand while simultaneously supporting streaming video and latency-sensitive voice traffic over the same channel. The resource allocated to one terminal by the MAC scheduler can vary from a single time slot to the entire frame, thus providing a very large dynamic range of throughput to a specific user terminal at any given time. Furthermore, since the resource allocation information is conveyed in the MAP messages at the beginning of each frame, the scheduler can effectively change the resource allocation on a frame-by-frame basis to adapt to the bursty nature of the traffic.

Figure 11

IP-OFDMA QoS Support

### 1.4.1	Quality of Service (QoS) Support

With fast air link, symmetric downlink/uplink capacity, fine resource granularity and a flexible resource allocation mechanism, IP-OFDMA can meet QoS requirements for a wide range of data services and applications.

In the IP-OFDMA MAC layer, QoS is provided via service flows as illustrated in Figure 11. A service flow is a unidirectional flow of packets provided with a particular set of QoS parameters. Before providing a certain type of data service, the Base Station and Mobile Station first establish a unidirectional logical link between the peer MACs, called a connection. The outbound MAC then associates packets traversing the MAC interface into a service flow to be delivered over the connection. The QoS parameters associated with the service flow define the transmission ordering and scheduling on the air interface. The connection-oriented MAC can therefore provide accurate QoS control over the air interface. Since the air interface is usually the bottleneck, the connection-oriented MAC can effectively enable end-to-end QoS control. The service flow parameters can be dynamically managed through MAC messages to accommodate the dynamic service demand. The service flow based QoS mechanism applies to both DL and UL to provide improved QoS in both directions. IP-OFDMA supports a wide range of data services and applications with varied QoS requirements. These are summarized in Table 7.

Table 7

IP-OFDMA Applications and Quality of Service

| QoS Category | Applications | QoS Specifications |
| --- | --- | --- |
| UGS: Unsolicited Grant Service | VoIP | Maximum Sustained Rate Maximum Latency Tolerance Jitter Tolerance |
| rtPS: Real-Time Packet Service | Streaming Audio or Video | Minimum Reserved Rate Maximum Sustained Rate Maximum Latency Tolerance Traffic Priority |
| ErtPS: Extended Real-Time Packet Service | Voice with Activity Detection (VoIP) | Minimum Reserved Rate Maximum Sustained Rate Maximum Latency Tolerance Jitter Tolerance Traffic Priority |
| nrtPS: Non-Real-Time Packet Service | File Transfer Protocol (FTP) | Minimum Reserved Rate Maximum Sustained Rate Traffic Priority |
| BE: Best-Effort Service | Data Transfer, Web Browsing, etc. | Maximum Sustained Rate Traffic Priority |

### 1.4.2	MAC Scheduling Service

The IP-OFDMA MAC scheduling service is designed to efficiently deliver time-sensitive broadband data services including voice, data, and video over time-varying broadband wireless channel. The MAC scheduling service has the following properties that enable this real-time broadband data service:

- Fast Data Scheduler: The MAC scheduler must efficiently allocate available resources in response to bursty data traffic and time-varying channel conditions. The scheduler is located at each base station to enable rapid response to traffic requirements and channel conditions. The data packets are associated to service flows with well defined QoS parameters in the MAC layer so that the scheduler can correctly determine the packet transmission ordering over the air interface. The CQICH channel provides fast channel information feedback to enable the scheduler to choose the appropriate coding and modulation for each allocation. The adaptive modulation/coding combined with HARQ provide robust transmission over the time-varying channel.

- Scheduling for both DL and UL: The scheduling service is provided for both DL and UL traffic. In order for the MAC scheduler to make an efficient resource allocation and provide the desired QoS in the UL, the UL must feed back accurate and timely information as to the traffic conditions and QoS requirements. Multiple uplink bandwidth request mechanisms (such as bandwidth request through ranging channel, piggyback request, and polling) are specified. The UL service flow defines the feedback mechanism for each uplink connection to ensure predictable UL scheduler behavior. Furthermore, with orthogonal UL subchannels, there is no intra-cell interference. UL scheduling can allocate resource more efficiently and better enforce QoS.

- Dynamic Resource Allocation: The MAC supports frequency-time resource allocation in both DL and UL on a per-frame basis. The resource allocation is delivered in MAP messages at the beginning of each frame. Therefore, the resource allocation can be changed on frame-by-frame in response to traffic and channel conditions. Additionally, the amount of resource in each allocation can range from one slot to the entire frame. The fast and fine granular resource allocation allows superior QoS for data traffic.

- UL and DL QoS: The MAC scheduler handles data transport on a connection-by-connection basis. Each connection is associated with a single data service with a set of QoS parameters that quantify the aspects of its behavior. With the ability to dynamically allocate resources in both DL and UL, the scheduler can provide QoS for both DL and UL traffic.

- Frequency Selective Scheduling: The scheduler can operate on different types of subchannels. For frequency-diverse subchannels such as PUSC permutation, where sub-carriers in the subchannels are pseudo-randomly distributed across the bandwidth, subchannels are of similar quality. Frequency-diversity scheduling can support a QoS with fine granularity and flexible time-frequency resource scheduling. With contiguous permutation such as AMC permutation, the subchannels may experience different attenuation. The frequency-selective scheduling can allocate mobile users to their corresponding strongest subchannel. The frequency-selective scheduling can enhance system capacity with a moderate increase in CQI overhead in the UL.

- Admission Control: Admission Control admits service flows based on resource availability. That is, a service flow is either admitted or rejected during service flow creation transaction. Admission Control is implemented on the various network elements: Server, BS and MS.

- Policing: A service flow is prohibited from injecting data traffic that exceeds its Maximum Sustained Traffic Rate. Policing enforces this restriction.

### 1.4.3	Power control and boosting

IP-OFDMA defines two modes of power control.

- Closed Loop Power Control, in which the Base Stations regularly adjusts the transmission level of each terminals based on the measurements done on received data from this terminal.

- Open Loop Power Control, in which the terminal adjusts its transmission level based on the signal strength measured on the received preamble from the serving Base Station. The serving Base Station is furthermore allowed to correct this transmission level, based on received signal strength. This correction is normally performed at very low frequency rate, enough to meet the requirement of the base station.

Furthermore, power boosting on data is a mechanism that can be used by the Base Station in order to extend its coverage. It is particularly convenient in an OFDMA scheme, where some subchannels can be boosted and some others attenuated, on the same OFDM symbol(s). The base station is hence able to use such boosting for further increasing the granularity of its link adaptation and the network load balancing.

### 1.4.4	Mobility Management

Battery life and handover are two critical issues for mobile applications.  IP-OFDMA supports Sleep Mode and Idle Mode to enable power-efficient MS operation. IP-OFDMA also supports seamless handover to enable the MS to switch from one base station to another at vehicular speeds without interrupting the connection.

### 1.4.5	Power Management

IP-OFDMA supports two modes for power efficient operation – Sleep Mode and Idle Mode. Sleep Mode is a state in which the MS conducts pre-negotiated periods of absence from the Serving Base Station air interface. These periods are characterized by the unavailability of the MS, as observed from the Serving Base Station, to DL or UL traffic. Sleep Mode is intended to minimize MS power usage and minimize the usage of the Serving Base Station air interface resources. The Sleep Mode also provides flexibility for the MS to scan other base stations to collect information to assist handover during the Sleep Mode.

Idle Mode provides a mechanism for the MS to become periodically available for DL broadcast traffic messaging without registration at a specific base station as the MS traverses an air link environment populated by multiple base stations. Idle Mode benefits the MS by removing the requirement for handover and other normal operations and benefits the network and base station by eliminating air interface and network handover traffic from essentially inactive MSs while still providing a simple and timely method (paging) for alerting the MS about pending DL traffic.

### 1.4.6	Handover

There are three handover methods supported within the IEEE 802.16 standard – Hard Handover, Fast Base Station Switching, and Macro Diversity Handover. Of these, the HHO is mandatory.

WiMAX Forum Mobile System Profile specifies a set of techniques for optimizing handover within the framework of the IEEE 802.16 standard. These improvements have been developed with the goal of keeping Layer 2 handover delays to less than 50 milliseconds.

When FBSS is supported, the MS and BS maintain a list of BSs that are involved in FBSS with the MS. This set is called an Active Set. In FBSS, the MS continuously monitors the base stations in the Active Set. Among the BSs in the Active Set, an Anchor BS is defined. When operating in FBSS, the MS communicates only with the Anchor BS for uplink and downlink messages, including management and traffic connections. Transition from one Anchor BS to another (i.e. BS switching) is performed without invocation of explicit HO signaling messages. Anchor update procedures are enabled by communicating signal strength of the serving BS via the CQICH. A FBSS handover begins with a decision by an MS to receive or transmit data from the Anchor BS that may change within the active set. The MS scans the neighbor BSs and selects those that are suitable to be included in the active set. The MS reports the selected BSs and the active set update procedure is performed by the BS and MS. The MS continuously monitors the signal strength of the BSs that are in the active set and selects one BS from the set to be the Anchor BS. The MS reports the selected Anchor BS on CQICH or MS initiated HO request message.  An important requirement of FBSS is that the data is simultaneously transmitted to all members of an active set of BSs that are able to serve the MS.

### 1.4.7	Security

IP-OFDMA supports mutual device/user authentication, flexible key management protocol, strong traffic encryption, control and management plane message protection, and security protocol optimizations for fast handovers.

The usage aspects of the security features are:

- Key Management Protocol: Privacy and Key Management Protocol Version 2 is the basis of IP-OFDMA security as defined in the IEEE 802.16 standard. This protocol manages the MAC security using PKM-REQ/RSP messages. PKM EAP authentication, Traffic Encryption Control, Handover Key Exchange, and Multicast/Broadcast security messages all are based on this protocol.

- Device/User Authentication:  IP-OFDMA supports Device and User Authentication using the IETF EAP protocol, providing support for credentials that are based on a SIM, USIM, Digital Certificate, or UserName/Password. Corresponding EAP-SIM, EAP-AKA, EAP-TLS, or EAP-MSCHAPv2 authentication methods are supported through the EAP protocol. Key deriving methods are the only EAP methods supported.

- Traffic Encryption: AES-CCM is the cipher used for protecting all the user data over the IP-OFDMA MAC interface. The keys used for driving the cipher are generated from the EAP authentication. A Traffic Encryption state machine with a periodic key refresh mechanism enables sustained transition of keys to further improve protection.

- Control Message Protection: Control data is protected using AES based CMAC or MD5-based HMAC schemes.

- Fast Handover Support: A 3-way handshake scheme is supported by IP-OFDMA to optimize the re-authentication mechanisms for supporting fast handovers. This mechanism is also useful to prevent man-in-the-middle-attacks.

# 2	Radio Transmission Technology (RTT) Description Template and Capacity and Coverage Analysis

## 2.1	Radio Transmission Technology (RTT) Description Template

With regards to the following tables, document 8F/1065 provided material for Sections A1.1 and A1.2 of the technology description template of M.1225. Sections A1.1 and A1.2 contained here are mostly a repetition of A1.1 and A1.2 in 8F/1065, except for a few additions that are highlighted using the following track change terminology.

Original text, deleted text  new text

The information provided in Sections A1.3, A1.4 and A1.5 is new material to complement the technology description template included in Document 8F/1065.

| A1.1 | Test environment support |  |
| --- | --- | --- |
| A1.1.1 | In what test environments will the RTT operate? | - indoor - outdoor to indoor and pedestrian, - vehicular - mixed |
| A1.1.2 | If the RTT supports more than one test environment, what test environment does this technology description template address? | One template for all |
| A1.1.3 | Does the RTT include any features in support of FWA application? Provide detail about the impact of those features on the technical parameters provided in this template, stating whether the technical parameters provided apply for mobile as well as for FWA applications. | Yes (cf. Rec. ITU-R F.1763). Flexible mixed fixed and mobile design. - QoS - Dynamic bandwidth allocation - Continuous and variable bit rate support - Support of nomadic operation - Support of fixed wireless voice, image, video and data services. |
| A1.2 | Technical parameters NOTE 1 – Parameters for both forward link and reverse link should be described separately, if necessary. |  |
| A1.2.1 | What is the minimum frequency band required to deploy the system (MHz)? | 5 MHz or 10 MHz (10 MHz provides better performance). |
| A1.2.2 | What is the duplex method: TDD or FDD? | TDD |
| A1.2.2.1 | What is the minimum up/down frequency separation for FDD? | N/A |
| A1.2.2.2 | What is requirement of transmit/receive isolation? Does the proposal require a duplexer in either the mobile station (MS) or BS? | Does not require a duplexer. |
| A1.2.3 | Does the RTT allow asymmetric transmission to use the available spectrum? Characterize. | Yes. The ratio of uplink to downlink transmission can be reconfigured on a system-wide basis. |
| A1.2.4 | What is the RF channel spacing (kHz)? In addition, does the RTT use an interleaved frequency plan? NOTE 1 – The use of the second adjacent channel instead of the adjacent channel at a neighbouring cluster cell is called “interleaved frequency planning”. If a proponent is going to employ an interleaved frequency plan, the proponent should state so in § A1.2.4 and complete § A1.2.15 with the protection ratio for both the adjacent and second adjacent channel. | 5 000 kHz or 10 000 kHz The RTT does not use an interleaved frequency plan |
| A1.2.5 | What is the bandwidth per duplex RF channel (MHz) measured at the 3 dB down points? It is given by (bandwidth per RF channel)  (1 for TDD and 2 for FDD). Provide detail. | For 5 MHz (TDD): about 4.7 MHz, depending on the permutation used. For 10 MHz (TDD): about 9.4 MHz, depending on the permutation used. |
| A1.2.5.1 | Does the proposal offer multiple or variable RF channel bandwidth capability? If so, are multiple bandwidths or variable bandwidths provided for the purposes of compensating the transmission medium for impairments but intended to be feature transparent to the end user? | The RTT offers variable RF channel bandwidth capability through the use of OFDMA sub-channelization. |
| A1.2.6 | What is the RF channel bit rate (kbit/s)? NOTE 1 – The maximum modulation rate of RF (after channel encoding, adding of in-band control signalling and any overhead signalling) possible to transmit carrier over an RF channel, i.e. independent of access technology and of modulation schemes. | DOWNLINK For the 10 MHz case this is the calculation: Distributed permutation of sub-carriers Assumptions: 10 MHz channel bandwidth, 32 data symbols per frame (35 symbols in sub-frame, 1 symbol for preamble, 2 symbols for control information), 5 ms frame duration, 64 QAM 5/6 code rate, 30 slots for 2 symbols, 48 data tones per slot. Maximum data rate: 23 040 kbit/s Note 1: The above numbers are calculated based on the maximum DL/UL ratio supported by IP-OFDMA. Note 2: The equivalent maximum data rate number for 5 MHz channel Bandwidth is 11520 kbit/s UPLINK Distributed permutation of sub-carriers Assumptions: 10 MHz channel bandwidth, 18 data symbols per frame (21 symbols in UL sub-frame, 3 symbols for control channels), 5 ms frame duration, 16 QAM 3/4 code rate, 35 slots for 3 symbols, 48 data tones per slot. Maximum data rate: 6 048 kbit/s Note 1: The above numbers are calculated based on the maximum UL/DL ratio supported by IP-OFDMA. Note 2: The equivalent maximum data rate number for 5 MHz channel Bandwidth is 3024 kbit/s. |
| A1.2.7 | Frame structure: describe the frame structure to give sufficient information such as: – frame length, – the number of time slots per frame, – guard time or the number of guard bits, – user information bit rate for each time slot, – channel bit rate (after channel coding), – channel symbol rate (after modulation), – associated control channel (ACCH) bit rate, – power control bit rate. NOTE 1 – Channel coding may include forward error correction (FEC), cyclic redundancy checking (CRC), ACCH, power control bits and guard bits. Provide detail. NOTE 2 – Describe the frame structure for forward link and reverse link, respectively. NOTE 3 – Describe the frame structure for each user information rate. | Frame length: 5 ms The number of time slots per frame: N/A The number of time symbols per frame: 48 symbols (including TTG and RTG gaps) The number of sub-carriers per each symbol: 512 and 1024 FFT for 5 and 10 MHz respectively. Resource allocation: 2 dimensional structure for frequency and time (see section 2.4 of the RTT System Description for more details) Sub-channel structure: see Section 2.2 of the RTT System Description for details Ratio of DL and UL sub-frame: Ranging from 35 symbols: 12 symbols to 26 symbols: 21 symbols (DL:UL) (35: 12), (34: 13), (33: 14), (32:15), (31: 16), (30: 17), (29: 18), (28: 19), (27: 20), (26: 21) TTG / RTG : 105.7 μs / 60 μs Common control overhead : 1 symbol per frame for preamble (see section 2.4 of the RTT System Description for more details) DOWNLINK (See A1.2.5.1) Distributed permutation of sub-carriers The number of sub-carriers per slot : 48 (data) + 8 (pilots) Guard sub-carrier: 184 (including DC sub-carrier) The channel bit or symbol rate is variable, depending on the number of allocated slots, and the modulation and coding rate. Power control rate: no power control Adjacent permutation of sub-carriers The number of sub-carriers per slot : 48 (data) + 6 (pilots) Guard sub-carrier : 160 (including DC sub-carrier) UPLINK Distributed permutation of sub-carriers |
|  |  | The number of subcarriers per slot : 48 (data) + 24 (pilots) Guard subcarrier : 184 (including DC subcarrier) The channel bit or symbol rate is variable, depending on the number of allocated slots, and the modulation and coding rate. Power control rate : 200 Hz Adjacent permutation of subcarriers The number of subcarriers per slot : 48 (data) + 6 (pilots) Guard subcarrier : 160 (including DC subcarrier) |
| A1.2.8 | Does the RTT use frequency hopping? If so, characterize and explain particularly the impact (e.g. improvements) on system performance. | No |
| A1.2.8.1 | What is the hopping rate? | N/A |
| A1.2.8.2 | What is the number of the hopping frequency sets? | N/A |
| A1.2.8.3 | Are BSs synchronized or non-synchronized? | Synchronized in frequency and in time for TDD operation, even though frequency hopping is not used. |
| A1.2.9 | Does the RTT use a spreading scheme? | No |
| A1.2.9.1 | What is the chip rate (Mchip/s)? Rate at input to modulator. | N/A |
| A1.2.9.2 | What is the processing gain? 10 log (chip rate/information rate). | N/A |
| A1.2.9.3 | Explain the uplink and downlink code structures and provide the details about the types (e.g. personal numbering (PN) code, Walsh code) and purposes (e.g. spreading, identification, etc.) of the codes. | N/A |
| A1.2.10 | Which access technology does the proposal use: TDMA, FDMA, CDMA, hybrid, or a new technology? In the case of CDMA, which type of CDMA is used: frequency hopping (FH) or direct sequence (DS) or hybrid? Characterize. | OFDMA |
| A1.2.11 | What is the baseband modulation technique? If both the data modulation and spreading modulation are required, describe in detail. What is the peak to average power ratio after baseband filtering (dB)? | DOWNLINK QPSK, 16 QAM, 64 QAM for data modulation. Spreading modulation does not apply. UPLINK QPSK, 16 QAM for data modulation. Spreading modulation does not apply. PAPR is about 12 dB without any PAPR reduction scheme. |
| A1.2.12 | What are the channel coding (error handling) rate and form for both the forward and reverse links? E.g., does the RTT adopt: – FEC or other schemes? – Unequal error protection? Provide details. – Soft decision decoding or hard decision decoding? Provide details. – Iterative decoding (e.g. turbo codes)? Provide details. – Other schemes? | Convolutional Coding and Convolutional Turbo Coding are supported Modulation schemes: QPSK, 16 QAM and 64 QAM for downlink, QPSK and 16 QAM for uplink. Coding rates: QPSK 1/2, QPSK 3/4, 16 QAM 1/2, 16 QAM 3/4, 64 QAM 1/2, 64 QAM 2/3, 64 QAM 3/4, 64 QAM 5/6. Coding repetition rates: 1x, 2x, 4x and 6x. Unequal error protection: None Soft decision decoding and iterative decoding: It is an implementation issue not covered by the description specification. |
| A1.2.13 | What is the bit interleaving scheme? Provide detailed description for both uplink and downlink. | The bit interleaving scheme is the same for both uplink and downlink. All encoded data bits shall be interleaved by a block interleaver with a block size corresponding to the number of coded bits per the encoded block size. |
| A1.2.14 | Describe the approach taken for the receives (MS and BS) to cope with multipath propagation effects (e.g. via equalizer, Rake receiver, etc.). | To cope with the multipath propagation effect, the cyclic prefix and 1-tap equalizer are employed. The length of cyclic prefix is 1/8 of symbol duration thus 11.4 μs. |
| A1.2.14.1 | Describe the robustness to intersymbol interference and the specific delay spread profiles that are best or worst for the proposal. | The intersymbol interference can be removed by the use of sufficiently longer cyclic prefix than delay spread. |
| A1.2.14.2 | Can rapidly changing delay spread profile be accommodated? Describe. | Yes, delay spread variation within the length of cyclic prefix does not cause the intersymbol interference. |
| A1.2.15 | What is the adjacent channel protection ratio? NOTE 1 – In order to maintain robustness to adjacent channel interference, the RTT should have some receiver characteristics that can withstand higher power adjacent channel interference. Specify the maximum allowed relative level of adjacent RF channel power (dBc). Provide detail how this figure is assumed. | Min adjacent channel rejection at BER=10-6 for 3 dB degradation C/I 11 dB – 16 QAM, 3/4 coding rate 4 dB – 64 QAM, 2/3 coding rate Min non-adjacent channel rejection at BER=10-6 for 3 dB degradation C/I 30 dB – 16 QAM, 3/4 coding rate 23 dB - 64 QAM, 2/3 coding rate |
| A1.2.16 | Power classes | Mobile Station Peak Transmit power (dBm) for 16QAM 1. 18 <= Ptx,max < 21 2. 21 <= Ptx,max < 25 3. 25 <= Ptx,max < 30 4. 30 <= Ptx,max Peak Transmit power (dBm) for QPSK 1. 20 <= Ptx,max < 23 2. 23 <= Ptx,max < 27 3. 27 <= Ptx,max < 30 4. 30 <= Ptx,max |
| A1.2.16.1 | Mobile terminal emitted power : what is the radiated antenna power measured at the antenna? For terrestrial component, give (dBm). For satellite component, the mobile terminal emitted power should be given in e.i.r.p. (effective isotropic radiated power) (dBm). | See A.1.2.16 |
| A1.2.16.1.1 | What is the maximum peak power transmitted while in active or busy state? | See A.1.2.16 |
| A1.2.16.1.2 | What is the time average power transmitted while in active or busy state? Provide detailed explanation used to calculate this time average power. | See A.1.2.16 |
| A1.2.16.2 | Base station transmit power per RF carrier for terrestrial component | See A.1.2.16 |
| A1.2.16.2.1 | What is the maximum peak transmitted power per RF carrier radiated from antenna? | Not limited by RTT |
| A1.2.16.2.2 | What is the average transmitted power per RF carrier radiated from antenna? | Not limited by RTT |
| A1.2.17 | What is the maximum number of voice channels available per RF channel that can be supported at one BS with 1 RF channel (TDD systems) or 1 duplex RF channel pair (FDD systems), while still meeting ITU-T Recommendation G.726 performance requirements? | The maximum number of voice channels per 1 RF channel depends on the bit rate and sampling rate supported by the codecs defined in the G.726. For instance, in case of the bit rate of 16 kbit/s with 20 msec sampling rate, up to 256 users can be supported simultaneously by a 10 MHz RF channel, while meeting the delay requirements of VoIP. In the case of a 5 MHz channel up to 120 users can be supported. The capacity calculated assumes a blocking-limited scenario with Voice Activity Factor = 1, DL 64 QAM 5/6, and UL 16QAM 3/4. |
| A1.2.18 | Variable bit rate capabilities: describe the ways the proposal is able to handle variable baseband transmission rates. For example, does the RTT use: – Adaptive source and channel coding as a function of RF signal quality? – Variable data rate as a function of user application? – Variable voice/data channel utilization as a function of traffic mix requirements? Characterize how the bit rate modification is performed. In addition, what are the advantages of your system proposal associated with variable bit rate capabilities? | Variable bit rate is supported by the flexible resource allocation. By assigning the variable number of sub-channels and using various modulations and coding rates frame by frame, the bit rate for each user can be variable frame by frame. Modulation and coding rate is usually defined by user's RF signal quality (CQI). For higher data rates, the bit rate information is provided to the receiver via scheduling mechanisms and associated control signaling every frame. |
| A1.2.18.1 | What are the user information bit rates in each variable bit rate mode? | The user information bit rates are variable according to the number of sub-channels assigned and modulation and coding rate used. For 10 MHz: DOWNLINK BW: 10 MHz Modulation : QPSK, 16 QAM, 64 QAM Coding rate : 1/2, 2/3, 3/4, 5/6 3312 kbit/s (1/2, QPSK, (DL:UL)=(26:21) symbols) ~ 23040 kbit/s (5/6, 64 QAM, (DL:UL)=(35:12) symbols). See equation below. Note 1: The above numbers are calculated based on the maximum DL/UL ratio supported by IP-OFDMA. Note 2: The equivalent maximum data rate number for 5 MHz channel Bandwidth is 11520 kbit/s UPLINK BW: 10 MHz Modulation : QPSK, 16 QAM Coding rate : 1/2, 3/4 1008 kbit/s (1/2, QPSK, (DL:UL)=(35:12) symbols) ~ 6048 kbit/s (3/4, 16 QAM, (DL:UL)=(26:21) symbols). See equation below. Note 1: The above numbers are calculated based on the maximum UL/DL ratio supported by IP-OFDMA. Note 2: The equivalent maximum data rate number for 5 MHz channel Bandwidth is 3024 kbit/s. Equation used: PHY Data Rate=(Data sub-carriers/Symbol period) × (information bits per symbol) |
| A1.2.19 | What kind of voice coding scheme or codec is assumed to be used in proposed RTT? If the existing specific voice coding scheme or codec is to be used, give the name of it. If a special voice coding scheme or codec (e.g. those not standardized in standardization bodies such as ITU) is indispensable for the proposed RTT, provide detail, e.g. scheme, algorithm, coding rates, coding delays and the number of stochastic code books. | Due to the IP-based characteristics of the radio interface it can utilize any speech codec. |
| A1.2.19.1 | Does the proposal offer multiple voice coding rate capability? Provide detail. | Yes. The RTT supports flexible data rate for each user and also provide variety scheduling services. A constant bit rate is provided by UGS service, while a variable bit rate is provided by ErtPS service. See A.1.2.18, A1.2.20.1 and A1.2.20.2 |
| A1.2.20 | Data services: are there particular aspects of the proposed technologies which are applicable for the provision of circuit-switched, packet-switched or other data services like asymmetric data services? For each service class (A, B, C and D) a description of RTT services should be provided, at least in terms of bit rate, delay and BER/frame error rate (FER). | Yes, a wide range of data services and applications with varied QoS requirements are supported. These are summarized in Table 7 of Section in this submission. |
|  | NOTE 1 – See Recommendation ITU-R M.1224 for the definition of: – “circuit transfer mode”, – “packet transfer mode”, – “connectionless service”, and for the aid of understanding “circuit switched” and “packet switched” data services. |  |
|  | NOTE 2 – See ITU-T Recommendation I.362 for details about the service classes A, B, C and D. |  |
| A1.2.20.1 | For delay constrained, connection oriented (Class A). | The RTT provides UGS (unsolicited grant service), corresponding to the Class A. UGS is characterized as constant and low data rates and low delay data service. |
| A1.2.20.2 | For delay constrained, connection oriented, variable bit rate (Class B). | The RTT provides rtPS (real-time polling service), corresponding to the Class B. rtPS is utilized for low to high data rate services. The RTT provides ErtPS (extended real-time polling service) as well. ErtPS is utilized for low data rate and low delay data services. |
| A1.2.20.3 | For delay unconstrained, connection oriented (Class C). | The RTT provides nrtPS (non-real-time polling service), corresponding to the Class C. nrtPS is utilized for high data rate services. |
| A1.2.20.4 | For delay unconstrained, connectionless (Class D). | The RTT provides BE (best effort service) corresponding to the Class D. BE is utilized for moderate data rate services. |
| A1.2.21 | Simultaneous voice/data services: is the proposal capable of providing multiple user services simultaneously with appropriate channel capacity assignment? | Yes, multiple parallel services are supported with different QoS requirements. Each service is associated with a set of QoS parameters that quantify aspects of its behavior. These parameters are managed using the dynamic service provisions, represented by the DSA and DSC message dialog. |
|  | NOTE 1 – The following describes the different techniques that are inherent or improve to a great extent the technology described above to be presented. Description for both BS and MS are required in attributes from § A1.2.22 through § A1.2.23.2. |  |
| A1.2.22 | Power control characteristics : is a power control scheme included in the proposal? Characterize the impact (e.g. improvements) of supported power control schemes on system performance. | Yes. A closed loop power control scheme and an open loop power control scheme are included. By means of these power control schemes, the interference level is reduced and the uplink system level throughput is increased. |
| A1.2.22.1 | What is the power control step size (dB)? | Power control step size is variable ranging from 0.25 dB to 32 dB. An 8-bit signed integer in power control information element indicates the power control step size in 0.25 dB units. Normally implemented in 1 dB increments. |
| A1.2.22.2 | What are the number of power control cycles per second? | The power control cycle of closed-loop power control is dependent on the rate of power control information element transmission, but less than 200 Hz. Due to TDD nature, the open loop power control cycle is inherently identical to the number of frames per seconds, thus 200 Hz. |
| A1.2.22.3 | What is the power control dynamic range (dB)? | The minimum power control dynamic range is 45 dB. |
| A1.2.22.4 | What is the minimum transmit power level with power control? | The RTT supports 45 dB under the full power assumption |
| A1.2.22.5 | What is the residual power variation after power control when RTT is operating? Provide details about the circumstances (e.g. in terms of system characteristics, environment, deployment, MS-speed, etc.) under which this residual power variation appears and which impact it has on the system performance. | The accuracy for power level control can vary from  0.5 dB to  2 dB depending on the power control step size. Single step size m ¦ Required relative accuracy ¦m¦ = 1dB ¦ +/- 0.5 dB ¦m¦ = 2dB ¦ +/- 1 dB ¦m¦ = 3dB ¦ +/- 1.5 dB Otherwise 4dB< ¦m¦< = 10dB ¦ +/- 2 dB Two exception points of at least 10 dB apart are allowed over the 45 dB range, where in these two points an accuracy of up to +/- 2 dB is allowed for any size step. |
| A1.2.23 | Diversity combining in MS and BS : are diversity combining schemes incorporated in the design of the RTT? | Yes. |
| A1.2.23.1 | Describe the diversity techniques applied in the MS and at the BS, including micro diversity and macro diversity, characterizing the type of diversity used, for example: – time diversity: repetition, Rake-receiver, etc., – space diversity: multiple sectors, multiple satellite, etc., – frequency diversity: FH, wideband transmission, etc., – code diversity: multiple PN codes, multiple FH code, etc., – other scheme. Characterize the diversity combining algorithm, for example, switch diversity, maximal ratio combining, equal gain combining. Additionally, provide supporting values for the number of receivers (or demodulators) per cell per mobile user. State the dB of performance improvement introduced by the use of diversity. For the MS: what is the minimum number of RF receivers (or demodulators) per mobile unit and what is the minimum number of antennas per mobile unit required for the purpose of diversity reception? These numbers should be consistent to that assumed in the link budget template of Annex 2 and that assumed in the calculation of the “capacity” defined at § A1.3.1.5. | The standard supports beamforming, transmit/receive diversity and MIMO. The receiver also supports maximal ratio combining. There is no need for a Rake receiver because it is an OFDM system. |
| A1.2.23.2 | What is the degree of improvement expected (dB)? Also indicate the assumed conditions such as BER and FER. | Please refer to Section . |
| A1.2.24 | Handover/automatic radio link transfer (ALT) : do the radio transmission technologies support handover? Characterize the type of handover strategy (or strategies) which may be supported, e.g. MS assisted handover. Give explanations on potential advantages, e.g. possible choice of handover algorithms. Provide evidence whenever possible. | Yes. The RTT supports handover and also provides means for expediting handover. Each base station broadcasts the information on the list of neighboring base stations and their channel information such as the operating center frequency, preamble index and synchronization periodically. The channel information in this broadcasting is used for a mobile station to synchronize with the neighboring base station. After a mobile station monitors the signal strength of a neighboring base station and seeks suitable base station(s) for handover, the mobile station or its serving base station can initiate handover by handover request message. But only the mobile station can transmit handover indication message to the its serving base station. After transmitting handover indication message, the mobile station stops monitoring the downlink frame of its serving base station and performs network re-entry to target base station. To reduce the handover latency further, the serving base station provides the target base station with network entry information on a mobile station to be handed over the target base station. Further information is available in the IEEE 802.16 standard; Section 6.3.22 MAC layer handover procedures. |
| A1.2.24.1 | What is the break duration (s) when a handover is executed? In this evaluation, a detailed description of the impact of the handover on the service performance should also be given. Explain how the estimate was derived. |  |
| A1.2.24.2 | For the proposed RTT, can handover cope with rapid decrease in signal strength (e.g. street corner effect)? Give a detailed description of: – the way the handover detected, initiated and executed, – how long each of this action lasts (minimum/maximum time (ms)), – the time-out periods for these actions. | Yes. A base station broadcasts the criterion which is being used for mobile station to request handover. The mobile station issues handover request message whenever the criterion is met. The handover criterion depends on the implementation but usually the received signal strength by a mobile station is used. Further information is available in the IEEE 802.16 standard; Section 11.1.7 MOB-NBR-ADV message encodings. |
| A1.2.25 | Characterize how the proposed RTT reacts to the system deployment (e.g. necessity to add new cells and/or new carriers) particularly in terms of frequency planning. | All base stations can use the same frequency or different frequency depending on the frequency reuse deployment scenario. OFDMA sub-channelization allows various permutations of sub-carriers. A distributed permutation of sub-carriers, e.g., PUSC (partial usage of sub-carrier) in this RTT, minimizes interferences from neighboring cells and/or sectors in case of the frequency reuse of 1. Different operators usually use different frequencies. |
| A1.2.26 | Sharing frequency band capabilities : to what degree is the proposal able to deal with spectrum sharing among IMT-2000 systems as well as with all other systems: – spectrum sharing between operators, – spectrum sharing between terrestrial and satellite IMT-2000 systems, – spectrum sharing between IMT-2000 and non-IMT-2000 systems, – other sharing schemes. | The proposed RTT utilizes OFDMA which has inherent interference protection capabilities due to allocation of a varying subset of available sub-carriers to different users. This capability, complemented by interference mitigation techniques described in Report ITU-R M.2045 such as use of appropriate filters and linear power amplifiers would ensure excellent potential for optimum spectrum sharing between the proposed RTT and other IMT-2000 systems. ITU-R WP 8F is in the process of performing sharing studies between fixed/nomadic and mobile IEEE 802.16 and IMT-2000. Preliminary results show similarities with the case of coexistence between IMT-2000 TDD and FDD technologies as captured in Reports ITU-R M.2030 and ITU-R M.2045. |
| A1.2.27 | Dynamic channel allocation : characterize the dynamic channel allocation (DCA) schemes which may be supported and characterize their impact on system performance (e.g. in terms of adaptability to varying interference conditions, adaptability to varying traffic conditions, capability to avoid frequency planning, impact on the reuse distance, etc.). | Various permutations of OFDMA sub-carriers enable dynamic usage of the spectrum among cells to balance the load and/or average interferences. |
| A1.2.28 | Mixed cell architecture : how well does the RTT accommodate mixed cell architectures (pico, micro and macrocells)? Does the proposal provide pico, micro and macro cell user service in a single licensed spectrum assignment, with handoff as required between them? (terrestrial component only). NOTE 1 – Cell definitions are as follows: – pico – cell hex radius: r  100 m – micro: 100 m  r  1 000 m – macro: r  1 000 m. | The proposed RTT can support flexible frequency reuse operation thus mixed cell architecture is supported well on the same or different frequencies depending on the implementation. |
| A1.2.29 | Describe any battery saver/intermittent reception capability. |  |
| A1.2.29.1 | Ability of the MS to conserve standby battery power : provide details about how the proposal conserves standby battery power. | The battery power saving of mobile station is supported by the sleep mode and the idle mode operations. Since the RTT basically provides packet-based transmission, both two modes operate in a slotted mode. In those modes, a mobile station communicates to its serving base station only in a listening interval and saves its power consumption otherwise. The information on listening, sleep and idle intervals are determined by the negotiation between the base station and the mobile station before the mobile station transits to either of two modes. A mobile station maintains the connection to its serving base station even in the sleep mode, while a mobile station in the idle mode returns system resources relevant to the existing connection to a base station. In latter case, the mobile station is managed by the multiple base stations grouped in a paging zone. Further information can be found in the IEEE 802.16 standard Sections 6.3.21, Sleep Mode, and 6.3.24, Idle Mode. |
| A1.2.30 | Signaling transmission scheme: if the proposed system will use RTTs for signaling transmission different from those for user data transmission, describe the details of the signaling transmission scheme over the radio interface between terminals and base (satellite) stations. | The same RTT is used for both user data and signaling transmission. |
| A1.2.30.1 | Describe the different signaling transfer schemes which may be supported, e.g. in connection with a call, outside a call. Does the RTT support: – new techniques? Characterize. – Signaling enhancements for the delivery of multimedia services? Characterize. | Flexible message-based signaling scheme is used. |
| A1.2.31 | Does the RTT support a bandwidth on demand (BOD) capability? BOD refers specifically to the ability of an end-user to request multi-bearer services. Typically, this is given as the capacity in the form of bits per second of throughput. Multi-bearer services can be implemented by using such technologies as multi-carrier, multi-time slot or multi-codes. If so, characterize these capabilities. NOTE 1 – BOD does not refer to the self-adaptive feature of the radio channel to cope with changes in the transmission quality (see § A1.2.5.1). | Yes. The scheduling service is provided for both downlink and uplink traffic. In order for the scheduler to make an efficient resource allocation and provide the desired QoS and data rate in the uplink, mobile stations must feedback accurate and timely information as to the traffic conditions and QoS requirements. To this end, multiple uplink bandwidth request mechanisms, such as bandwidth request through ranging channel, piggyback request and polling are provided to support uplink bandwidth requests. Frequency and time resource allocation in both downlink and uplink is on a per frame basis to duly react to the traffic and channel conditions. Additionally, the amount of resource in each allocation can range from one slot to the entire frame. Further information can be found in the IEEE 802.16 standard, Sections 6.3.6 Bandwidth Allocation and Request mechanism, 6.3.7.3 DL-MAP, 6.3.7.4 UL-MAP, and 8.4.4 Frame Structure. |
| A1.2.32 | Does the RTT support channel aggregation capability to achieve higher user bit rates? | No. |

| A1.3 | Expected performances. |  |
| --- | --- | --- |
| A1.3.1 | For terrestrial test environment only. |  |
| A1.3.1.1 | What is the achievable BER floor level (for voice)? NOTE 1 – The BER floor level is evaluated under the BER measuring conditions defined in Annex 2 using the data rates indicated in § 1 of Annex 2. | Coded BER floor is implementation-dependent but achievable floor is significantly below GoS requirements (10-3) within the specified ranges of tolerable delay spread (20 µs) and Doppler shifts (250Hz). |
| A1.3.1.2 | What is the achievable BER floor level (for data)? NOTE 1 – The BER floor level is evaluated under the measuring conditions defined in Annex 2 using the data rates indicated in § 1 of Annex 2. | Coded BER floor is implementation-dependent but achievable floor is significantly below GoS requirements (10-6) within the specified ranges of tolerable delay spread (20 µs) and Doppler shifts (250 Hz). |
| A1.3.1.3 | What is the maximum tolerable delay spread (ns) to maintain the voice and data service quality requirements? NOTE 1 – The BER is an error floor level measured with the Doppler shift given in the BER measuring conditions of Annex 2. | The maximum specified range of delay spread (20 µ s in Vehicular B) can be tolerated without an equalizer. |
| A1.3.1.4 | What is the maximum tolerable Doppler shift (Hz) to maintain the voice and data service quality requirements? NOTE 1 – The BER is an error floor level measured with the delay spread given in the BER measuring conditions of Annex 2. | At least 500 Hz, based on the observation that Doppler frequency shows about 570 Hz for 250 km/h at 2.5 GHz |
| A1.3.1.5 | Capacity : the capacity of the radio transmission technology has to be evaluated assuming the deployment models described in Annex 2 and technical parameters from § A1.2.22 through § A1.2.23.2. |  |
| A1.3.1.5.1 | What is the voice traffic capacity per cell (not per sector): provide the total traffic that can be supported by a single cell (E/MHz/cell) in a total available assigned non-contiguous bandwidth of 30 MHz (15 MHz forward/15 MHz reverse) for FDD mode or contiguous bandwidth of 30 MHz for TDD mode. Provide capacities for all penetration values defined in the deployment model for the test environment in Annex 2. The procedure to obtain this value is described in Annex 2. The capacity supported by not a standalone cell but a single cell within contiguous service area should be obtained here. | See Section for details Voice capacity (ITU Vehicular path loss model, Pedestrian B 3 channel model): - 90 Erlangs/MHz/cell for reuse 3, SIMO,10 MHz PUSC - 80 Erlangs/MHz/cell for reuse 3, SIMO, 5 MHz PUSC |
| A1.3.1.5.2 | What is the information capacity per cell (not per sector): provide the total number of user-channel information bits which can be supported by a single cell (Mbit/s/MHz/cell) in a total available assigned non-contiguous bandwidth of 30 MHz (15 MHz forward/15 MHz reverse) for FDD mode or contiguous bandwidth of 30 MHz for TDD mode. Provide capacities for all penetration values defined in the deployment model for the test environment in Annex 2. The procedure to obtain this value is described in Annex 2. The capacity supported by not a standalone cell but a single cell within contiguous service area should be obtained here. | See reference for details : Data capacity (PUSC, ITU Vehicular, 60% Pedestrian B 3, 30% Vehicular A 30, 10% Vehicular A 120, DL:UL=28:9) SIMO: 10 MHz DL = 3.57 Mbps/MHz/cell UL = 1.59 Mbps/MHz/cell 5 MHz DL = 3.45 Mbps/MHz/cell UL = 1.6 Mbps/MHz/cell MIMO: 10 MHz DL = 5.52 Mbps/MHz/cell UL = 2.10 Mbps/MHz/cell - Beamforming technology increases the spectral efficiency of the system. |
| A1.3.1.6 | Does the RTT support sectorization? If yes, provide for each sectorization scheme and the total number of user-channel information bits which can be supported by a single site (Mbit/s/MHz) (and the number of sectors) in a total available assigned non-contiguous bandwidth of 30 MHz (15 MHz forward/15 MHz reverse) in FDD mode or contiguous bandwidth of 30 MHz in TDD mode. | Yes, the RTT supports sectorization. The sectorization and frequency reuse schemes are implementation-dependent and consequently, so are the capacities achieved. The tri-sector scheme is the typical scenario with frequency reuse 1 or reuse 3. |
| A1.3.1.7 | Coverage efficiency: the coverage efficiency of the radio transmission technology has to be evaluated assuming the deployment models described in Annex 2. |  |
| A1.3.1.7.1 | What is the base site coverage efficiency (km2/site) for the lowest traffic loading in the voice only deployment model? Lowest traffic loading means the lowest penetration case described in Annex 2. | See Link Budget in Section 2.3.4 |
| A1.3.1.7.2 | What is the base site coverage efficiency (km2/site) for the lowest traffic loading in the data only deployment model? Lowest traffic loading means the lowest penetration case described in Annex 2. | See Link Budget in Section 2.3.4 |
| A1.3.2 | For satellite test environment only |  |
| A1.3.2.1 | What is the required C/N0 to achieve objective performance defined in Annex 2? |  |
| A1.3.2.2 | What are the Doppler compensation method and residual Doppler shift after compensation? |  |
| A1.3.2.3 | Capacity : the spectrum efficiency of the radio transmission technology has to be evaluated assuming the deployment models described in Annex 2. |  |
| A1.3.2.3.1 | What is the voice information capacity per required RF bandwidth (bit/s/Hz)? |  |
| A1.3.2.3.2 | What is the voice plus data information capacity per required RF bandwidth (bit/s/Hz)? |  |
| A1.3.2.4 | Normalized power efficiency : the power efficiency of the radio transmission technology has to be evaluated assuming the deployment models described in Annex 2. |  |
| A1.3.2.4.1 | What is the supported information bit rate per required carrier power-to-noise density ratio for the given channel performance under the given interference conditions for voice? |  |
| A1.3.2.4.2 | What is the supported information bit rate per required carrier power-to-noise density ratio for the given channel performance under the given interference conditions for voice plus data? |  |
| A1.3.3 | Maximum user bit rate (for data) : specify the maximum user bit rate (kbit/s) available in the deployment models described in Annex 2. | The maximum bit rates are well above 20160 kbit/s. (DL/UL ratio = 2:1, PUSC, 64QAM, 5/6 coding rate) |
| A1.3.4 | What is the maximum range (m) between a user terminal and a BS (prior to hand-off, relay, etc.) under nominal traffic loading and link impairments as defined in Annex 2? | See Link Budget in Section . The maximum range depends on the deployment and the QoS of a connection |
| A1.3.5 | Describe the capability for the use of repeaters. | Repeaters can be used. There is nothing in the technology that precludes the use of repeaters. |
| A1.3.6 | Antenna systems : fully describe the antenna systems that can be used and/or have to be used; characterize their impacts on systems performance, (terrestrial only); e.g., does the RTT have the capability for the use of: – remote antennas: describe whether and how remote antenna systems can be used to extend coverage to low traffic density areas; – distributed antennas: describe whether and how distributed antenna designs are used, and in which IMT-2000 test environments; – Smart antennas (e.g., switched beam, adaptive, etc.): describe how smart antennas can be used and what is their impact on system performance; – other antenna systems. | The air-interface does not place any restrictions on the types of antenna systems such as smart antenna technologies, including Beamforming, Transmit/Receive diversity and MIMO, as well as a combination of these like Beamforming plus MIMO. The uses of remote and distributed antennas are not precluded. |
| A1.3.7 | Delay (for voice) | Voice services are provided in the PS-domain with appropriate QoS setting (UGS, rtPS or ErtPS) |
| A1.3.7.1 | What is the radio transmission processing delay due to the overall process of channel coding, bit interleaving, framing, etc., not including source coding? This is given as transmitter delay from the input of the channel coder to the antenna plus the receiver delay from the antenna to the output of the channel decoder. Provide this information for each service being provided. In addition, a detailed description of how this parameter was calculated is required for both the uplink and the downlink. | The minimum delay is roughly 10ms assuming a 5ms TDD frame and the maximum is implementation and traffic load-dependent (scheduling metric, traffic load, buffer sizes, retransmission scheme etc) |
| A1.3.7.2 | What is the total estimated round trip delay (ms) to include both the processing delay, propagation delay (terrestrial only) and vocoder delay? Give the estimated delay associated with each of the key attributes described in Fig. 6 that make up the total delay provided. | Assuming a 20 ms vocoder, 5ms frame and ignoring queuing delay (typically <30ms), the RTD delay is approximately 60 ms |
| A1.3.7.3 | Does the proposed RTT need echo control? | Yes |
| A1.3.8 | What is the MOS level for the proposed codec for the relevant test environments given in Annex 2? Specify its absolute MOS value and its relative value with respect to the MOS value of ITU-T Recommendation G.711 (64 k PCM) and ITU-T Recommendation G.726 (32 k ADPCM). NOTE 1 – If a special voice coding algorithm is indispensable for the proposed RTT, the proponent should declare detail with its performance of the codec such as MOS level. (See § A1.2.19) | The RTT supports VoIP and is not limited to any particular codecs. Applications/implementations determine the choice of codec. |
| A1.3.9 | Description of the ability to sustain quality under certain extreme conditions. |  |
| A1.3.9.1 | System overload (terrestrial only) : characterize system behaviour and performance in such conditions for each test services in Annex 2, including potential impact on adjacent cells. Describe the effect on system performance in terms of blocking grade of service for the cases that the load on a particular cell is 125%, 150%, 175%, and 200% of full load. Also describe the effect of blocking on the immediate adjacent cells. Voice service is to be considered here. Full load means a traffic loading which results in 1% call blocking with the BER of 1  10–3 maintained. | The RTT provides many features that can be used to ensure optimal loading in the event of system overload. Among these are admission control, handover, rate adaptation, fractional frequency reuse and power control. |
| A1.3.9.2 | Hardware failures : characterize system behaviour and performance in such conditions. Provide detailed explanation on any calculation. | This is implementation-dependent. The RTT does not preclude any means to build in redundancy or other reliability features. |
| A1.3.9.3 | Interference immunity : characterize system immunity or protection mechanisms against interference. What is the interference detection method? What is the interference avoidance method? | In addition to frequency reuse, and intelligent scheduling/RRM, the RTT’s TDD OFDM interface is inherently robust against delay spread, suitable for multi-user detection and supports various smart antenna schemes. Also, the RTT does not preclude any means to cancel interference or to protect against interference |
| A1.3.10 | Characterize the adaptability of the proposed RTT to different and/or time-varying conditions (e.g. propagation, traffic, etc.) that are not considered in the above attributes of § A1.3. | The RTT supports modulation and coding adaptation, HARQ, power control and opportunistic scheduling |
| A1.4 | Technology design constraints |  |
| A1.4.1 | Frequency stability : provide transmission frequency stability (not oscillator stability) requirements of the carrier (include long term – 1 year – frequency stability requirements (ppm)). |  |
| A1.4.1.1 | For BS transmission (terrestrial component only). | BS frequency tolerance ≤  2ppm of carrier frequency BS to BS frequency accuracy ≤  1% of subcarrier spacing |
| A1.4.1.2 | For MS transmission. | MS to BS frequency synchronization tolerance ≤ 2% of the subcarrier spacing |
| A1.4.2 | Out-of-band and spurious emissions : specify the expected levels of base or satellite and mobile transmitter emissions outside the operating channel, as a function of frequency offset. | Base stations and terminals supporting this RTT will comply with local, regional, and international regulations for out of band and spurious emissions, wherever applicable. Similar to other IMT-2000 RTTs, terminals adhering to a single global mask will be used to provide global roaming. |
| A1.4.3 | Synchronisation requirements : describe RTT’s timing requirements, e.g. – Is BS-to-BS or satellite land earth station (LES)-to-LES synchronisation required? Provide precise information, the type of synchronisation, i.e., synchronisation of carrier frequency, bit clock, spreading code or frame, and their accuracy. – Is BS-to-network synchronisation required? (terrestrial only). – State short-term frequency and timing accuracy of BS (or LES) transmit signal. – State source of external system reference and the accuracy required, if used at BS (or LES) (for example: derived from wireline network, or GPS receiver). – State free run accuracy of MS frequency and timing reference clock. – State base-to-base bit time alignment requirement over a 24 h period (s). | BS-to-BS synchronisation : Yes. All BSs should be time and frequency synchronized to a common source signal. The common source signal is typically provided by GPS. BS-to-network synchronisation: No. BS-to-network synchronisation is not required. Frequency accuracy : BS frequency tolerance ≤  2ppm of carrier frequency Timing accuracy ≤ 1usec compared to reference timing. Source of external system reference and the accuracy : GPS (the synchronizing reference shall be a 1 ps timing pulse and a 10 MHz frequency reference) Free run accuracy : MS frequency tolerance ≤ maximum 2% of the subcarrier spacing Timing tolerance: 25% of minimum guard interval( (Tb/32)/4) The BS's timing accuracy is required to be 1 μs compared to reference timing when GPS locked. |
| A1.4.4 | Timing jitter : for BS (or LES) and MS give: – the maximum jitter on the transmit signal, – the maximum jitter tolerated on the received signal. Timing jitter is defined as r.m.s. value of the time variance normalized by symbol duration. | BS The BS's timing accuracy is required to be 1μsec compared to reference timing. MS MS Transmit symbol timing accuracy within ± (Tb/32)/4 |
| A1.4.5 | Frequency synthesizer: what is the required step size, switched speed and frequency range of the frequency synthesizer of MSs? | Frequency step size : 200 and 250 KHz Switched speed : 200 μs Frequency range : 5, 10 MHz Start frequencies are various, depending on channel bandwidth and profile |
| A1.4.6 | Does the proposed system require capabilities of fixed networks not generally available today? | No |
| A1.4.6.1 | Describe the special requirements on the fixed networks for the handover procedure. Provide handover procedure to be employed in proposed RTT in detail. | The RTT supports handover and also provides means for expediting handover. Each base station broadcasts the information on the list of neighboring base stations and their channel information such as the operating center frequency, preamble index and synchronization periodically. The channel information in this broadcasting is used for a mobile station to synchronize with the neighboring base station. After a mobile station monitors the signal strength of a neighboring base station and seeks suitable base station(s) for handover, the mobile station or its serving base station can initiate handover by handover request message. But only the mobile station can transmit handover indication message to the its serving base station. After transmitting handover indication message, the mobile station stops monitoring the downlink frame of its serving base station and performs network re-entry to target base station. To reduce the handover latency further, the serving base station provides the target base station with network entry information on a mobile station to be handed over the target base station. |
| A1.4.7 | Fixed network feature transparency |  |
| A1.4.7.1 | Which service(s) of the standard set of ISDN bearer services can the proposed RTT pass to users without fixed network modification. | Convergence Sublayer in the proposed RTT supports interface to various fixed networks such as ATM, Ethernet, IP, and VLAN. |
| A1.4.8 | Characterize any radio resource control capabilities that exist for the provision of roaming between a private (e.g., closed user group) and a public IMT-2000 operating environment. | Handover between the different access networks is basically supported. Furthermore, Operator ID in the signalling during the handover enable mobile stations to recognize the operator of access network they are handed over to. |
| A1.4.9 | Describe the estimated fixed signaling overhead (e.g., broadcast control channel, power control messaging). Express this information as a percentage of the spectrum which is used for fixed signaling. Provide detailed explanation on your calculations. | The fixed MAP overhead is typically about 10% in a 10 MHz channel with a 5ms frame size. |
| A1.4.10 | Characterize the linear and broadband transmitter requirements for BS and MS (terrestrial only). | BS - Tx dynamic Range = 10 dB - Spectral flatness according to the following: ≤ ±2 dB for spectral lines from – Nused/4 to 1 and +1 to Nused/4 Within +2/-4 dB for spectral lines from – Nused/2 to Nused/4 and +Nused/4 to Nused/2 - Per sub-carrier flatness ≤ 0.1 dB - Power difference between adjacent subcarriers according to the following: Tx downlink radio frame shall be time-aligned with the 1pps timing pulse within 1 μsec - Tx relative constellation error according to the following: QPSK-1/2 ≤ -15.0 dB QPSK-3/4 ≤ -18.0 dB 16QAM-1/2 ≤ -20.5 dB 16QAM-3/4 ≤ -24.0 dB 64QAM-1/2 (if 64-QAM supported) ≤ -26.0 dB 64QAM-2/3 (if 64-QAM supported) ≤ -28.0 dB 64QAM-3/4 (if 64-QAM supported)≤ -30.0 dB MS - Tx dynamic Range = 45 dB - Tx power level min adjustment step = 1 dB - Tx power level min relative step accuracy = ± 0.5 dB - Spectral flatness according to the following: ≤ ±2 dB for spectral lines from –Nused/4 to –1 and +1 to Nused/4 Within +2/-4 dB for spectral lines from –Nused/2 to –Nused/4 and +Nused/4 to Nused/2 - Power difference between adjacent subcarriers ≤ 0.1 dB - Tx relative constellation error according to the following: QPSK-1/2 ≤ -15.0 dB QPSK-3/4 ≤ -18.0 dB 16QAM-1/2 ≤ -20.5 dB 16QAM-3/4 ≤ -24.0 dB |
| A1.4.11 | Are linear receivers required? Characterize the linearity requirements for the receivers for BS and MS (terrestrial only). | BS No. The PAPR of the proposed RTT is around 12dB, and which is not required a stringent linear receiver. |
| A1.4.12 | Specify the required dynamic range of receiver (terrestrial only). | BS Max input level on-channel reception tolerance = -45 dBm Max input level on-channel damage tolerance = -10 dBm MS Max input level on-channel reception tolerance = -30 dBm Max input level on-channel damage tolerance = 0 dBm BS and MS Max input level sensitivity (Distributed permutation of subcarriers) for 10 MHz case: -88.5 dBm - QPSK-1/2 -85.1 dBm - QPSK-3/4 -82.8 dBm - 16QAM-1/2 -78.7 dBm - 16QAM-3/4 -77.6 dBm - 64QAM-1/2 -74.5 dBm - 64QAM-2/3 -73.4 dBm - 64QAM-3/4 -71.5 dBm - 64QAM-5/6 Max input level sensitivity (Distributed permutation of subcarriers) for 5 MHz case: -91.5 dBm - QPSK-1/2 -88.1 dBm - QPSK-3/4 -85.8 dBm - 16QAM-1/2 -81.7 dBm - 16QAM-3/4 -80.6 dBm - 64QAM-1/2 -77.5 dBm - 64QAM-2/3 -76.4 dBm - 64QAM-3/4 -74.5 dBm - 64QAM-5/6 Sensitivity numbers are calculated based on assumption of repetition factor 1 and Distributed permutation of subcarriers. |
| A1.4.13 | What are the signal processing estimates for both the handportable and the BS? – MOPS (millions of operations per second) value of parts processed by DSP (digital signal processing), – gate counts excluding DSP, – ROM size requirements for DSP and gate counts (kbytes), – RAM size requirements for DSP and gate counts (kbytes). NOTE 1 – At a minimum the evaluation should review the signal processing estimates (MOPS, memory requirements, gate counts) required for demodulation, equalization, channel coding, error correction, diversity processing (including Rake receivers), adaptive antenna array processing, modulation, A-D and D-A converters and multiplexing as well as some IF and baseband filtering. For new technologies, there may be additional or alternative requirements (such as FFTs etc.). NOTE 2 – The signal processing estimates should be declared with the estimated condition such as assumed services, user bit rate and etc. | It is an implementation issue not covered by the description. |
| A1.4.14 | Dropped calls : describe how the RTT handles dropped calls. Does the proposed RTT utilize a transparent reconnect procedure – that is, the same as that employed for handoff? | No specific process to handle call dropping recovery is defined. However, mobile station can recover the connection after call dropping by means of the Idle mode re-entry procedure. |
| A1.4.15 | Characterize the frequency planning requirements: – frequency reuse pattern: given the required C/I and the proposed technologies, specify the frequency cell reuse pattern (e.g. 3-cell, 7-cell, etc.) and, for terrestrial systems, the sectorization schemes assumed; – characterize the frequency management between different cell layers; – does the RTT use an interleaved frequency plan? – are there any frequency channels with particular planning requirements? – all other relevant requirements. NOTE 1 – The use of the second adjacent channel instead of the adjacent channel at a neighbouring cluster cell is called “interleaved frequency planning”. If a proponent is going to employ an interleaved frequency plan, the proponent should state so in § A1.2.4 and complete § A1.2.15 with the protection ratio for both the adjacent and second adjacent channel. | The RTT supports frequency reuse configuration of 1 and 3. In order for MS to provide BS with a correct DL channel quality information, MS is required to properly measure CINR of preamble with considering the frequency reuse configuration: i.e. For frequency reuse of 3, consider the modulated subcarriers of the preamble only. For frequency reuse of 1, consider both the un-modulated and the modulated subcarriers of the preamble. There are 114 different preamble code sets in the proposed RTT to differentiate the cell ID and sector ID's per each sector. The RTT can use both the interleaved frequency plan and the non-interleaved frequency plan. |
| A1.4.16 | Describe the capability of the proposed RTT to facilitate the evolution of existing radio transmission technologies used in mobile telecommunication systems migrate toward this RTT. Provide detail any impact and constraint on evolution. |  |
| A1.4.17 | Are there any special requirements for base site implementation? Are there any features which simplify implementation of base sites? (terrestrial only) | No |
| A1.5 | Information required for terrestrial link budget template Proponents should fulfill the link budget template given in Table 6 and answer the following questions. | see Section Link Budget |
| A1.5.1 | What is the BS noise figure (dB)? | 4 dB used for Section Link Budget Max 8 dB Noise Figure is considered in RTT. |
| A1.5.2 | What is the MS noise figure (dB)? | 7 dB see Section Link Budget Max 8 dB Noise Figure is considered in RTT for MS clients supporting multi band operation. |
| A1.5.3 | What is the BS antenna gain (dBi)? | 15 dBi (see Section Link Budget) |
| A1.5.4 | What is the MS antenna gain (dBi)? | -1 dBi (see Section Link Budget) |
| A1.5.5 | What is the cable, connector and combiner losses (dB)? | 0 dB (see Section Link Budget) |
| A1.5.6 | What are the number of traffic channels per RF carrier? | Function of required QoS |
| A1.5.7 | What is the RTT operating point (BER/FER) for the required Eb/N0 in the link budget template? | 1% FER |
| A1.5.8 | What is the ratio of intra-sector interference to sum of intra-sector interference and inter-sector interference within a cell (dB)? | Depends on environment and receiver implementation |
| A1.5.9 | What is the ratio of in-cell interference to total interference (dB)? | Negligible at low Doppler (<300 Hz) and depends on receiver implementation at high Doppler |
| A1.5.10 | What is the occupied bandwidth (99%) (Hz)? | Depends on nominal bandwidth, permutation scheme, and on the subchannelization. For the case considered in Section Link Budget, it is approximately 9.2 MHz on the downlink and 2.4 MHz on the uplink |
| A1.5.11 | What is the information rate (dBHz)? | Depends on service rate with the maximum subject to the channel bandwidth employed. (see Section Link Budget) |

## 2.2	Requirements and Objectives Template

Note: Table 8, Table 9 and Table 10, respectively, correspond to responses to TABLE 1, TABLE 2 and TABLE 3 of the Requirements and Objectives Template in M.1225.

Table 8

TABLE 1, Generic Requirements and Objectives Relevant to the Evaluation 
of Candidate Radio Transmission Technologies

| IMT-2000 Item Description | Obj/Req | Source | Meets?* |
| --- | --- | --- | --- |
| Voice and data performance requirements |  |  |  |
| One-way end to end delay less than 40 ms | Req | G.174, § 7.5 |  Yes  No Yes |
| For mobile videotelephone services, the IMT-2000 terrestrial component should operate so that the maximum overall delay (as defined in ITU-T Rec. F.720) should not exceed 400 ms, with the one way delay of the transmission path not exceeding 150 ms | Req | Suppl. F.720, F.723, G.114 |  Yes  No Yes |
| Speech quality should be maintained during  3% frame erasures over any 10 second period. The speech quality criterion is a reduction of  0.5 mean opinion score unit (5 point scale) relative to the error-free condition (G.726 at 32 kb/s) | Req | G.174, § 7.11 & M.1079 § 7.3.1 |  Yes  No Yes |
| DTMF signal reliable transport (for PSTN is typically less than one DTMF errored signal in 104) | Req | G.174, § 7.11 & M.1079 § 7.3.1 |  Yes  No Yes |
| Voiceband data support including G3 facsimile | Req | M.1079 § 7.2.2 |  Yes  No Yes |
| Support packet switched data services as well as circuit switched data; requirements for data performance given in ITU-T G.174 | Req | M.1034 §§ 10.8, 10.9 |  Yes  No Yes and No (see Note 1 below) |
| Radio interfaces and subsystems, network related performance requirements |  |  |  |
| Network interworking with PSTN and ISDN in accordance with Q.1031 and Q.1032 | Req | M.687-1 § 5.4 |  Yes  No Yes |
| Meet spectral efficiency and radio channel performance requirements of M.1079 | Req | M.1034 § 12.3.3/4 |  Yes  No Yes |
| Provide phased approach with data rates up to 2 Mbit/s in phase 1 | Obj | M.687, § 1.1.14 |  Yes  No Yes |
| Maintain bearer channel bit-count integrity (e.g. synchronous data services and many encryption techniques) | Obj | M.1034, § 10.12 |  Yes  No Yes |
| Support for different cell sizes, for example - Mega cell Radius ~100-500 km Macro cell Radius  35 km,Speed  500 km/h Micro cell Radius  1 km,Speed  100 km/h Pico cell Radius  50m,Speed 10 km/h | Obj | M.1035 § 10.1 |  Yes  No Yes |
| Application of IMT-2000 for fixed services and developing countries |  |  |  |
| Circuit noise - idle noise levels in 99% of the time about 100 pWp | Obj | M.819-1, § 10.3 |  Yes  No Yes |
| Error performance - as specified in ITU-R F.697 | Obj | M.819-1, § 10.4 |  Yes  No Yes |
| Grade of service better than 1% | Obj | M.819-1, § 10.5 |  Yes  No Yes |

Note 1: The RTT is purely a Packet switch data technology. Circuit switched data is not supported. But will support seamless interworking with circuit switched systems using media gateways and support for QoS classes.

Table 9

TABLE 2, Generic Requirements and Objectives Relevant to the Evaluation 
of Candidate Radio Transmission Technologies

| IMT-2000 Item Description | Obj/Req | Source | Meets* |
| --- | --- | --- | --- |
| Radio interfaces and subsystems, network related performance requirements |  |  |  |
| Security comparable to that of PSTN/ISDN | Obj | M.687-1 § 4.4 | Yes  No Yes |
| Support mobility, interactive and distribution services | Req | M.816 § 6 |  Yes  No Yes |
| Support UPT and maintain common presentation to users | Obj | M.816 § 4 |  Yes  No |
| Voice quality comparable to the fixed network (applies to both mobile and fixed service) | Req | M.819-1 Table 1, M.1079 § 7.1 |  Yes  No Yes |
| Support encryption and maintain encryption when roaming and during handover | Req | M.1034 § 11.3 |  Yes  No Yes |
| Network access indication similar to PSTN (e.g. dialtone) | Req | M.1034 § 11.5 |  Yes  No Yes (see Note 2 below the Table) |
| Meet safety requirements and legislation | Req | M.1034 § 11.6 |  Yes  No Yes |
| Meet appropriate EMC regulations | Req | M.1034 § 11.7 |  Yes  No Yes |
| Support multiple public/private/ residential IMT-2000 operators in the same locality | Req | M.1034 § 12.1.2 |  Yes  No Yes |
| Support multiple mobile station types | Req | M.1034 § 12.1.4 |  Yes  No Yes |
| Support roaming between IMT-2000 operators and between different IMT-2000 radio interfaces/ environments | Req | M.1034 § 12.2.2 |  Yes  No Yes |
| Support seamless handover between different IMT-2000 environments such that service quality is maintained and signalling is minimized | Req | M.1034 § 12.2.3 |  Yes  No Yes |
| Simultaneously support multiple cell sizes with flexible base location, support use of repeaters and umbrella cells as well as deployment in low capacity areas | Req | M.1034 § 12.2.5 |  Yes  No Yes |
| Support multiple operator coexistence in a geographic area | Req | M.1034 § 12.2.5 |  Yes  No Yes |
| Support different spectrum and flexible band sharing in different countries including flexible spectrum sharing between different IMT-2000 operators (see M.1036) | Req | M.1034 § 12.2.8 |  Yes  No Yes |
| Support mechanisms for minimizing power and interference between mobile and base stations | Req | M.1034 § 12.2.8.3 |  Yes  No Yes |
| Support various cell types dependent on environment (M.1035 § 10.1) | Req | M.1034 § 12.2.9 |  Yes  No Yes |
| High resistance to multipath effects | Req | M.1034 § 12.3.1 |  Yes  No Yes |
| Support appropriate vehicle speeds (as per § 7) NOTE: applicable to both terrestrial and satellite proposals | Req | M.1034 § 12.3.2 |  Yes  No Yes |
| Support possibility of equipment from different vendors | Req | M.1034 § 12.1.3 |  Yes  No Yes |
| Offer operational reliability as least as good as 2nd generation mobile systems | Req | M.1034 § 12.3.5 |  Yes  No Yes |
| Ability to use terminal to access services in more than one environment, desirable to access services from one terminal in all environments | Obj | M.1035 § 7.1 |  Yes  No Yes |
| End-to-end quality during handover comparable to fixed services | Obj |  |  Yes  No Yes |
| Support multiple operator networks in a geographic area without requiring time synchronization | Obj |  |  Yes  No Yes |
| Layer 3 contains functions such as call control, mobility management and radio resource management some of which are radio dependent. It is desirable to maintain layer 3 radio transmission independent as far as possible | Obj | M.1035 § 8 |  Yes  No Yes |
| Desirable that transmission quality requirements from the upper layer to physical layers be common for all services | Obj | M.1035 § 8.1 |  Yes  No Yes |
| The link access control layer should as far as possible not contain radio transmission dependent functions | Obj | M.1035 § 8.3 |  Yes  No Yes |
| Traffic channels should offer a functionally equivalent capability to the ISDN B-channels | Obj | M.1035 § 9.3.2 |  Yes  No Yes |
| Continually measure the radio link quality on forward and reverse channels | Obj | M.1035 § 11.1 |  Yes  No Yes |
| Facilitate the implementation and use of terminal battery saving techniques | Obj | M.1035 § 12.5 |  Yes  No Yes |
| Accommodate various types of traffic and traffic mixes | Obj | M.1036 § 1.10 |  Yes  No Yes |
| Application of IMT-2000 for fixed services and developing countries |  |  |  |
| Repeaters for covering long distances between terminals and base stations, small rural exchanges with wireless trunks etc. | Req | M.819-1 Table 1 |  Yes  No Yes |
| Withstand rugged outdoor environment with wide temperature and humidity variations | Req | M.819-1 Table 1 |  Yes  No Yes |
| Provision of service to fixed users in either rural or urban areas | Obj | M.819-1 § 4.1 |  Yes  No Yes |
| Coverage for large cells (terrestrial) | Obj | M.819-1 § 7.2 |  Yes  No Yes |
| Support for higher encoding bit rates for remote areas | Obj | M.819-1 § 10.1 |  Yes  No Yes |
| Additional satellite- component specific requirements and objectives |  |  |  |
| Links between the terrestrial and satellite control elements for handover and exchange of other information | Req | M.818-1 § 3.0 |  Yes  No NA |
| Take account for constraints for sharing frequency bands with other services (WARC-92) | Obj | M.818-1 § 4.0 |  Yes  No NA |
| Compatible multiple access schemes for terrestrial and satellite components | Obj | M.818-1 § 6.0 |  Yes  No NA |
| Service should be comparable quality to terrestrial component as far as possible | Obj | M.818-1 § 10.0 |  Yes  No NA |
| Use of satellites to serve large cells for fixed users | Obj | M.819-1 § 7.1 |  Yes  No NA |
| Key features e.g. coverage, optimization, number of systems | Obj | M.1167 § 6.1 |  Yes  No NA |
| Radio interface general considerations | Req | M.1167 § 8.1.1 |  Yes  No NA |
| Doppler effects | Req | M.1167 § 8.1.2 |  Yes  No NA |

Note 2: These are application specific and not mandated by the RTT. But applications may support this.

Table 10

TABLE 3, Generic Requirements and Objectives Relevant to the Evaluation 
of Candidate Radio Transmission Technologies

| IMT-2000 Item Description | Obj/Req | Source | Proponents Description |
| --- | --- | --- | --- |
| Fixed Service - Power consumption as low as possible for solar and other sources | Req | M.819-2 Table 1 | These are implementation dependent and are not restricted by the RTT definition |
| Minimize number of radio interfaces and radio sub-system complexity, maximize commonality (M.1035 § 7.1) | Req | M.1034-1 § 11.2.1 | Yes |
| Minimize need for special interworking functions | Req | M.1034-1 § 11.2.4 | Yes. Interworking functions are only needed when interfacing to non-IP networks. |
| Minimum of frequency planning and inter-network coordination and simple resource management under time-varying traffic | Req | M.1034-1 § 11.2.6 | Yes |
| Support for traffic growth, phased functionality, new services or technology evolution | Req | M.1034-1 § 11.2.7 | Yes |
| Facilitate the use of appropriate diversity techniques avoiding significant complexity if possible | Req | M.1034-1 § 11.2.10 | Yes |
| Maximize operational flexibility | Req | M.1034-1 § 11.2.11 | Yes |
| Designed for acceptable technological risk and minimal impact from faults | Req | M.1034-1 § 11.2.12 | Yes |
| When several cell types are available, select the cell that is the most cost and capacity efficient | Obj | M.1034-1 §[9.2] M.1035 § 10.3.3 | Yes |
| Minimize terminal costs, size and power consumption, where appropriate and consistent with other requirements | Obj | M.1036 § 2.1.12 | Yes |

## 2.3	Capacity and Coverage

### 2.3.1	Voice Capacity

The VoIP capacity results were generated from OPNET simulations based on a 19-cell scenario and detailed modeling of the IP-OFDMA MAC protocols, overhead and latencies. The simulation statistics are collected on the center sector whose loading is controlled by varying the number of users.

### Methodology

- Configuration with N-users in a sector (N is varied).

- Pick a Nominal SINR at random for each MSS, reflecting the path loss, shadowing and interference

- Propagate the UL and DL channels for fast fading using ITU Pedestrian B

- UGS QoS for the voice service flow

  - Allocate 32B of grant on an average every 4 frames (20 msec) on the UL

  - Schedule the voice flows on the DL at an average of every 20msec.

  - Max queueing latency bound on the transmit queue = 50msec

    - Packets waiting in the queue, beyond this bound, are dropped.

- Xrtps QoS modeling for voice activity detection with 40% activity factor

- Link Adaptation is done via MAC Management messages.

- For each mobile during the simulation, FER, throughput and latency statistics are collected.

  - FER accounts for frame drops due to both channel errors and transmit queue latency > 50msec.

### Simulation Assumptions

- Results generated from OPNET MAC-E2E simulator

- Pedestrian channel modeling– frequency selective fast fading

- 3 sector cell

- SIMO (2 Rx antennas, 1 Tx antenna)

- DL:UL 3:2

- 5 MHz/10 MHz TDD channel

- 1x3x3 PUSC reuse scheme

- Target PER = 3%

- Sub MAPs

- MLWDF scheduler – channel aware and resource fair

  - Channel aware and resource fair scheduler.

  - Metric for user i at time t:

    - M(i, t) = c(i) * (m(i, t)/A(i, t)) * W(i, t)

    - Where

      - m(i, t) = instantaneous rate of the user

      - A(i, t) = (a(i)*m(i, t) + (1-a(i))*A(i, t-1)) = mean rate of user

      - W(i, t) = queue length at time t

      - c(i) = a constant for user i, indicating his relative priority

  - Advantage of MLWDF over PF:

    - Adding the queue length to the metric, forces a user with to get served even if the channel is poor for a long time. This increases the overall fairness.

  - Optimized and Proprietary rectangularization algorithms

- VoIP packet assumptions

  - RTP/UDP/IP encapsulation

  - G.729 codec: 20B of voice payload every 20msec

  - G722.2 (AMR) codec: 12-31B of voice payload every 20msec

  - G711 codec: 8B of voice payload every 1msec

  - PHS

    - All the RTP/UDP/IP headers can be suppressed except for RTP sequence numbers (2B)

      - CID of the MAC header can be made to uniquely map to the header parameters

    - The resulting RTP/UDP/IP header is 2B

- Other System parameters

  - CellSize 				1.5 km

  - PathLoss Model 			ITU Vehicular

  - CarrierFrequency 		2.3 GHz

  - BSHeight 			30 m

  - BSTxPower 			36 dBm

  - BSAntennaGain 			16 dB

  - 3 dB antenna beamwidth	70 degrees

  - SSHeight 			Vehicular(1.5m)

  - SSTxPower 			23 dBm

  - SSAntennaGain 			0 dB

Figure 12 traces the latencies incurred, assuming no queuing delays, from the generation of a voice frame in a MS to its reception at the BS.

Figure 12

VoIP UL Packet processing timeline

### Results

The VoIP capacity is defined here as the Erlangs supported for a 2% blocking rate per 3-sector cell at a FER of <3% and a 1-way delay of <100 ms (ITU-R M.1079-1). The capacity was FER-limited, with the 1-way latencies (queuing and transmission latencies) at <85 ms and typically 55-65 ms for the uplink, well under the 100 ms bound. The downlink has a lower latency bound because of the absence of scheduling grant latencies.

The voice capacity for the above assumptions was determined to be 90 Erlangs/MHz/cell for the 10MHz bandwidth and 80 Erlangs/MHz/cell for 5MHz bandwidth. Note that the voice capacity includes the downlink and uplink portions of the TDD frame..

### 2.3.2	Handover Performance

Handovers may be initiated by the Mobile or the Base Station based on signal quality, loading or service criteria. The handover results are based on analysis of the protocol specifications.

The handover performance metrics used here are the minimum time required to execute handover and the interruption time during the handover process during which time data transfer is stopped.

The handover performance requirements are dictated primarily by the QoS requirements. Voice service requires minimum interruption times and delays but can tolerate some loss of data. On the other hand, data services (non-realtime or best-effort data) are more tolerant of interruption and delays but require minimum or no loss of data.

### Assumptions and Procedures

The following assumptions were used in deriving the HO performance:

- 5 ms frame

- MS-assisted-NW-controlled for

  - Neighbor BS  prioritizing

  - Inter-frequency scan profiles & reporting criteria

- All measurements based on frame preamble

The Handover procedure is as follows:

- Preparation

- Handover request from MS to BS (if MS-initiated)

- Source base station selects target base station after backbone pre-notification procedure (collocated case optimizations)

- BS-MS exchange HO decision

- HO (Collocated & intra-frequency case optimizations)

- HO Ranging in target base station

- NW Re-entry

### Handover Performance Results

The Handover performance metrics are defined as follows:

- Preparation time - duration between the HO decision and the HO trigger.

- Interruption time - duration between stopping serving BS TX/RX and starting target BS TX/RX.

The results are summarized in Table 11 for the scenario described below.  shows the calculation for scenarios 1 and 2.

Scenario 1. MS initiated HHO, intra-FA, non-collocated, full optimized NW re-entry (only RNG-REQ/RSP)

This is a scenario, where NW conditions may be comparable to FBSS (frequency reuse factor 1, TEK sharing between BS's, no ranging), and where HHO break-off duration is very short.. HO is between two separate BS's with same FA's

Scenario 2. MS initiated HHO, intra-FA, collocated, full optimized NW re-entry

Similar to preceding scenario, except HO is between two PUSC sectors within same BS. HO latency is same as in the non-collocated case.

Scenario 3. MS initiated HHO, inter-FA, non-collocated, full optimized NW re-entry

Similar to preceding scenario, except HO is between BS's with different FA's

Scenario 4. MS initiated HHO, optimized NW re-entry with TEKs update

Similar to preceding scenario, except NW re-entry procedure includes TEKs update

Scenario 5. MS initiated HHO due to link loss (drop situation)

Link loss is MAC synchronization loss:

600 ms max since last good received DL-MAP or 5*DCD interval max value (10 s max) since last good received DCD (the earliest of the two). MS may decide earlier than that to HO.

MS will HO to 1st HO candidate (from cell selection list, best received NBR BS), and perform CDMA ranging using HO codes.

TBS identifies HO code in ranging region and may allocate ranging regions more frequently (e.g. every frame).

Upon receiving RNG-REQ, TBS will request MS context from SBS, and perform optimized NW re-entry (like any coordinated HO).

Scenario 6. BS initiated HHO due to scan result

All HO decision making is done by Serving BS.

MS reports scan results of neighboring BSs.

BS decides to HO according to scan report.

Table 11

Summary of HO latency analysis for optimized HHO

| Scenario | Preparation time (ms) | Interruption time (ms) |
| --- | --- | --- |
| 1. MS initiated HHO, intra-FA, non-collocated, full optimized NW re-entry (only RNG-REQ/RSP) | 70-85 | 45 |
| 2. MS initiated HHO, intra-FA, collocated, fully optimized NW re-entry | 70-85 | 45 |
| 3. MS initiated HHO, inter-FA, non-collocated, fully optimized NW re-entry. | 70-85 | 50 |
| 4. MS initiated HHO, optimized NW re-entry with TEKs update | 60-75 | 80-85 |
| 5. MS initiated HHO due to link loss (drop situation) | 0 | 140-165 |
| 6. BS initiated HHO | 35-50 | 50 |

Table 12

HO Duration Analysis for Scenarios 1 and 2

|  | Action | Duration (frames) | Remarks |
| --- | --- | --- | --- |
|  | MS decides to initiate HO to Target BS | 0 | MS has a valid (updated) MOB_NBR_ADV list MS is able to estimate ranging (Tx PHY) parameters |
|  | MS sends MOB_MSHO_REQ to SBS (with preferred TBS) | 4-7 | If currently allocated user BW is insufficient to accommodate this message, a BW request must be issued first (7 frames, CDMA BW request to UL allocation) |
|  | SBS processes MOB_MSHO_REQ and responds with MOB_BSHO_RSP (with recommended TBS) | 6 | Assumes that recommended BS list includes MS' preferred TBS (no "reject" expect from MS) At this time, the SBS informs TBS about MS' intent to HO SBS sends pre-notification request and receives response within 2 frames |
|  | MS processes MOB_BSHO_RSP and transmits MOB_HO_IND to SBS (with HO type and preferred TBS) | 4 | Unsolicited UL allocation by TBS (for MOB_HO-IND). If TBS is different from the TBS in the HO_REQ message, the SBS informs the final TBS about MS' intent to HO to it From this moment and on, the SBS will retain resources of MS with timeout |
|  | MS switches to TBS, acquires DL signal and implements channel estimation result | 2 | 1st frame at TBS is used for channel estimation. In 2nd frame at TBS, MS is ready to read MAPs |
|  | TBS allocates FAST_RANGING_IE (at estimated HO time) | 0 | TBS may allocate FAST_RANGING_IE in two consecutive frames (for robustness). |
|  | MS sends RNG-REQ to TBS (with OMAC) | 1 | UL-MAP relevance is next frame RNG-REQ is CMAC signed. |
|  | TBS processes RNG-REQ message and responds with RNG_RSP (with CID update and HO optimization flag) | 3 | RNG_RSP message includes SBC_RSP and REG_RSP TLV's (for CID update) RNG_RSP message includes security related TLV's. |
|  | MS processes RNG-RSP (CID update) | 3 | Unsolicited UL allocation by TBS (for MOB_HO-IND). Assumes key sharing between sectors within same BS |
|  | TBS starts normal operations with MS | 0 |  |
|  | HO preparation period | 14-17 | 70-85 ms (@frame=5ms) |
|  | HO break-off duration | 9 | 45 ms (@frame=5ms) |

### 2.3.3	Data Capacity

### System Parameters

We consider IP-OFDMA system with the following characteristics as a case study for a quantitative evaluation of the system performance. In the following, Table 13 provides the system parameters,  summarizes the OFDMA parameters, and Table 15 shows the propagation model used for the performance evaluation.

Table 13

Modeled System Parameters

| Parameters | Value |
| --- | --- |
| Number of 3-Sector Cells | 19 |
| Operating Frequency | 2500 MHz |
| Duplex | TDD |
| Channel Bandwidth | 5/10 MHz |
| BS-to-BS Distance | 2.8 km |
| Minimum Mobile-to-BS Distance | 36 m |
| Antenna Pattern | 70° (-3 dB) with 20 dB front-to-back ratio |
| BS Height | 32 m |
| Mobile Terminal Height | 1.5 m |
| BS Antenna Gain | 15 dBi |
| MS Antenna Gain | -1 dBi |
| BS Maximum Power Amplifier Power | 43 dBm |
| Mobile Terminal Maximum PA Power | 23 dBm |
| # of BS Tx/Rx Antenna | Tx: 2 or 4; Rx: 2 or 4 |
| # of MS Tx/Rx Antenna | Tx: 1; Rx: 2 |
| BS Noise Figure | 4 dB |
| MS Noise Figure | 7 dB |

Table 14

OFDMA Parameters

| Parameters |  | Values |
| --- | --- | --- |
| System Channel Bandwidth (MHz) |  | 10 |
| Sampling Frequency (Fp in MHz) |  | 11.2 |
| FFT Size (NFFT) |  | 1024 |
| Sub-Carrier Frequency Spacing |  | 10.94 kHz |
| Useful Symbol Time (Tb = 1/f) |  | 91.4 µs |
| Guard Time (Tg =Tb/8) |  | 11.4 µs |
| OFDMA Symbol Duration (Ts = Tb + Tg) |  | 102.9 µs |
| Frame duration |  | 5 ms |
| Number of OFDMA Symbols |  | 48 (including ~1.6 symbols for TTG/RTG) |
| DL PUSC | Null Sub-carriers | 184 |
|  | Pilot Sub-carriers | 120 |
|  | Data Sub-carriers | 720 |
|  | Subchannels | 30 |
| UL PUSC | Null Sub-carriers | 184 |
|  | Pilot Sub-carriers | 280 |
|  | Data Sub-carriers | 560 |
|  | Subchannels | 35 |

Table 15

Propagation Model

| Parameters | Value |
| --- | --- |
| Propagation Model | COST 231 Suburban |
| Log-Normal Shadowing SD (σs) | 8 dB |
| BS Shadowing Correlation | 0.5 |
| Penetration Loss | 10 dB |

### System Performance

Simulations based on system parameters described in Table 13 - Table 15 have been performed to assess the performance of IP-OFDMA. The performance simulation assumes heterogeneous users with a mix of mobile users as described in Table 16 and Table 17.

Table 16

Multi-Path Channel Models for Performance Simulation

| Channel Model | Path 1 (dB) | Path 2 (dB) | Path 3 (dB) | Path 4 (dB) | Path 5 (dB) | Path 6 (dB) | Rake Fingers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ITU Pedestrian B Ch-103 | -3.92 | -4.82 | -8.82 | -11.92 | -11.72 | -27.82 | 1,2,3,4,5,6 |
| ITU Vehicular A Ch-104 | -3.14 | -4.14 | -12.14 | -13.14 | -18.14 | -23.14 | 1,2,3,4,5,6 |

Table 17

Mixed User Channel Model for Performance Simulation

| Channel Model | Number of Paths | Speed | Fading | Assignment Probability |
| --- | --- | --- | --- | --- |
| ITU Pedestrian B Ch-103 | 6 | 3 km/hr | Jakes | 0.60 |
| ITU Vehicular A Ch-104 | 6 | 30 km/hr | Jakes | 0.30 |
|  | 6 | 120 km/hr | Jakes | 0.10 |

There are 10 users per sector. The traffic is assumed to be full buffer FTP traffic. Proportional Fair scheduler is assumed. Each base station is configured with three (3) sectors with a cell and sector frequency reuse factor equal to one. Ideal channel estimation and realistic link adaptation is also assumed. The carrier frequency for the simulation is 2.5 GHz The frame overhead to account for Preamble, MAP OH, and UL Control Channel is 7 OFDMA symbols in the DL and 3 in the UL. 1 symbol is allocated for TTG for a total of 11 overhead symbols and 37 data symbols for both DL and UL.  Further configuration and assumption details are listed in Table 18.

Table 18

System Configuration Assumptions

| Parameters |  | Value |
| --- | --- | --- |
| Cell Configuration |  | 3 Sectors/Cell |
| Frequency Reuse |  | 1 |
| Users/Sector |  | 10 |
| Traffic Type |  | Full Buffer |
| Channel Estimation |  | Ideal |
| PHY Abstraction |  | EESM |
| Scheduler |  | Proprietary Proportional Fair |
| Link Adaptation |  | Realistic with delay feedback |
| Antenna Configuration |  | 1x2, 2x2 |
| MIMO Support | DL | Alamouti STC, VSM |
|  | UL | Collaborative SM |
| MIMO Switch |  | Adaptive STC/VSM switch |
| HARQ |  | CC, 3 Retransmissions |
| Coding |  | CTC |
| Frame Overhead |  | 11 OFDM Symbols (7 DL, 3 UL, 1 TTG) |
| Data Symbols per Frame |  | 37 |
| DL/UL Partition | A | 28:9 |
|  | B | 22:15 |

The performance is summarized in Table 19 for a TDD implementation with 5 and 10 MHz channel bandwidths, SIMO and MIMO antenna configurations and DL/UL ratios of 28:9 and 22:15 respectively. The results show that the IP-OFDMA system has high spectral efficiency. For 10 MHz SIMO, the DL sector spectral efficiency is about 1.2 bits/sec/Hz and UL sector spectral efficiency is 0.55 bits/s/Hz. With 2x2 MIMO, the spectral efficiency is further improved by 55% in the DL and 40% in the UL. The high spectral efficiency combined with wide channel bandwidth provides very high sector throughput for the IP-OFDMA system. With 2x2 MIMO and a DL/UL ratio of 3:1, the DL sector throughput is 13.60 Mbit/s and the UL sector throughput is 1.83 Mbit/s; with a DL/UL ratio of 3:2, the sector throughput is 10.63 Mbit/s and 2.74 Mbit/s respectively for DL and UL. The high sector data throughput is essential to enable broadband data services including video and VoIP.

It should be noted that 11 symbols of overhead is a conservative estimate for overhead. For most data applications, the traffic is bursty and the system can operate more efficiently with less overhead. Additionally, the subchannel considered for this case is PUSC diversity subchannelization and frequency selective scheduling gain is not taken into account in the simulation. With frequency selective AMC subchannelization, the spectral efficiency can be further increased by 15 to 25% . Therefore, with an optimized system, the spectral efficiency and throughput can be further improved by 20 to 30% compared to the results shown in Table 19. The spectral efficiency improvement for this case is illustrated in Figure 13 for the 2x2 MIMO antenna configuration.

Table 19

IP-OFDMA System Performance

| Cases |  | DL: 28 data symbols UL: 9 data symbols |  | DL: 22 data symbols UL: 15 data symbols |  |
| --- | --- | --- | --- | --- | --- |
| Antenna | Link | Sector Throughput Mbps | Spectral Efficiency bps/Hz/sector | Sector Throughput Mbps | Spectral Efficiency bps/Hz/sector |
| SIMO 5MHz | DL | 4.3 | 1.18 | 3.2 | 0.88 |
|  | UL | 0.7 | 0.54 | 1.1 | 0.57 |
| SIMO 10MHz | DL | 8.8 | 1.21 | 6.6 | 1.09 |
|  | UL | 1.38 | 0.55 | 2.2 | 0.59 |
| MIMO 10MHz | DL | 13.6 | 1.87 | 10.63 | 1.76 |
|  | UL | 1.83 | 0.73 | 2.74 | 0.73 |

Figure 13

Spectral Efficiency Improvement with Optimized IP-OFDMA

Figure 14

Throughput with Varied DL/UL Ratios and Optimized IP-OFDMA

Another advantage of the IP-OFDMA system is its ability to dynamically reconfigure the DL/UL ratio to adapt to the network traffic profile so as to maximize spectrum utilization. This is illustrated in Figure 14 where the cross-hatched bars represent the base line values shown in Table 19. It shows that the maximum DL sector throughput can be greater than 20 Mbit/s and maximum UL sector throughput can be greater than 8 Mbit/s. With a typical DL/UL ratio range between 3:1 and 1:1, the DL sector throughput can vary between 10 Mbit/s and 17 Mbit/s; the UL sector throughput can vary between 2 Mbit/s and 4 Mbit/s.

The results here are based on the Mobile System Profile basic SIMO(1x2) and MIMO (2x2) configurations, further performance improvements can be realized with additional advanced features such as beamforming (AAS).

## Link Budget

### Pedestrian and Vehicular Link Budgets based on M.1225 assumptions

Table 20 to Table 22 are link budgets for the speech, LCD and UDD in Pedestrian and Vehicular deployments, per the M.1225 assumptions and format.

The Eb/(No+Io) are for a 5MHz bandwidth, PUSC SIMO (1Tx-2Rx antennas in both downlink and uplink) system. The diversity gain is included in the Eb/(No+Io), hence there is no explicit diversity gain. The bearer rates take into account the overhead due to retransmissions. Since the system is lightly-loaded per the M.1225 requirements, the downlink repetition and uplink subchannelization gains are explicitly accounted for. The log-normal fade margins are calculated for a 95% area coverage and path loss exponents of 4 and 3.76 for Pedestrian and Vehicular environments. Note that a site is defined as an omni cell for the Pedestrian case and 3-sector cell for the Vehicular case.

Table 20

Link Budgets for Pedestrian and Vehicular Speed

|  |  | Downlink | Uplink | Downlink | Uplink |
| --- | --- | --- | --- | --- | --- |
| Test environment |  | Pedestrian | Pedestrian | Vehicular | Vehicular |
| Multipath channel class |  | A | A | A | A |
| Mobile speed |  | 3 km/h | 3 km/h | 120 km/h | 120 km/h |
| Test service |  | Speech | Speech | Speech | Speech |
| Note |  |  |  |  |  |
| Bit rate | bits/s | 17800 | 17800 | 17800 | 17800 |
| Average TX power per traffic ch | dBm | <20 | <14 | <30 | <24 |
| Maximum TX power per traffic ch | dBm | 20 | 14 | 30 | 24 |
| Maximum total TX power | dBm | 20 | 14 | 30 | 24 |
| Cable, conn and combiner losses | dBm | 2 | 0 | 2 | 0 |
| Tx antenna gain | dBi | 10 | 0 | 13 | 0 |
| TX EIRP per traffic channel | dBm | 28 | 14 | 41 | 24 |
| Total TX EIRP | dBm | 28 | 14 | 41 | 24 |
| RX antenna gain | dBi | 0 | 10 | 0 | 13 |
| Cable and connector losses | dB | 0 | 2 | 0 | 2 |
| Receiver noise figure | dB | 5 | 5 | 5 | 5 |
| Thermal noise density | dBm/Hz | -174 | -174 | -174 | -174 |
| RX interference density | dBm/Hz | -1000 | -1000 | -1000 | -1000 |
| Total effect noise+interf density | dBm/Hz | -169 | -169 | -169 | -169 |
| Information rate | dBHz | 42.5 | 42.5 | 42.5 | 42.5 |
| Required Eb/(No+Io) | dB | 8.9 | 8.6 | 7.3 | 7.8 |
| RX sensitivity | dB | -117.6 | -117.9 | -119.2 | -118.7 |
| Explicit diversity gain | dB | 0 | 0 | 0 | 0 |
| Other gain (DL repetition / UL subchannelization) | dB | 7.8 | 12.3 | 7.8 | 12.3 |
| Log-normal fade margin | dB | 11.2 | 11.2 | 11.4 | 11.4 |
| Maximum path loss | dB | 142.2 | 141.0 | 156.6 | 154.6 |
| Maximum range | km | 0.72 | 0.67 | 5.71 | 5.06 |
| Coverage efficiency | sq km/site | 1.61 | 1.40 | 63.60 | 49.92 |
| Optimized Parameters |  |  |  |  |  |
| Maximum TX power per traffic ch | dBm | 23 | 20 | 40 | 24 |
| Tx antenna gain | dBi | 10 | 0 | 17 | 0 |
| RX antenna gain | dBi | 0 | 10 | 0 | 17 |
| Maximum path loss | dB | 145.2 | 147.0 | 170.6 | 158.6 |
| Maximum range | km | 0.85 | 0.94 | 13.46 | 6.47 |
| Coverage efficiency | sq km/site | 2.27 | 2.80 | 353.27 | 81.48 |

Table 21

Link Budgets for 384 kbps Pedestrian and 144kbps Vehicular LCD

|  |  | Downlink | Uplink | Downlink | Uplink |
| --- | --- | --- | --- | --- | --- |
| Test environment |  | Pedestrian | Pedestrian | Vehicular | Vehicular |
| Multipath channel class |  | A | A | A | A |
| Mobile speed |  | 3 km/h | 3 km/h | 120 km/h | 120 km/h |
| Test service |  | 384 kbps | 384 kbps | 144 kbps | 144 kbps |
| Note |  |  |  |  |  |
| Bit rate | bits/s | 426200 | 426200 | 159800 | 159800 |
| Average TX power per traffic ch | dBm | <20 | <14 | <30 | <24 |
| Maximum TX power per traffic ch | dBm | 20 | 14 | 30 | 24 |
| Maximum total TX power | dBm | 20 | 14 | 30 | 24 |
| Cable, conn and combiner losses | dBm | 2 | 0 | 2 | 0 |
| Tx antenna gain | dBi | 10 | 0 | 13 | 0 |
| TX EIRP per traffic channel | dBm | 28 | 14 | 41 | 24 |
| Total TX EIRP | dBm | 28 | 14 | 41 | 24 |
| RX antenna gain | dBi | 0 | 10 | 0 | 13 |
| Cable and connector losses | dB | 0 | 2 | 0 | 2 |
| Receiver noise figure | dB | 5 | 5 | 5 | 5 |
| Thermal noise density | dBm/Hz | -174 | -174 | -174 | -174 |
| RX interference density | dBm/Hz | -1000 | -1000 | -1000 | -1000 |
| Total effect noise+interf density | dBm/Hz | -169 | -169 | -169 | -169 |
| Information rate | dBHz | 56.3 | 56.3 | 52.0 | 52.0 |
| Required Eb/(No+Io) | dB | 6.3 | 7.3 | 4.6 | 7.1 |
| RX sensitivity | dB | -106.4 | -105.4 | -112.4 | -109.9 |
| Explicit diversity gain | dB | 0 | 0 | 0 | 0 |
| Other gain (DL repetition / UL subchannelization) | dB | 7.0 | 6.3 | 7.8 | 9.3 |
| Log-normal fade margin | dB | 11.2 | 11.2 | 11.4 | 11.4 |
| Maximum path loss | dB | 130.2 | 122.5 | 149.8 | 142.8 |
| Maximum range | km | 0.36 | 0.23 | 3.76 | 2.45 |
| Coverage efficiency | sq km/site | 0.40 | 0.17 | 27.58 | 11.71 |
| Optimized Parameters |  |  |  |  |  |
| Maximum TX power per traffic ch | dBm | 23 | 20 | 40 | 24 |
| Tx antenna gain | dBi | 10 | 0 | 17 | 0 |
| RX antenna gain | dBi | 0 | 10 | 0 | 17 |
| Maximum path loss | dB | 133.2 | 128.5 | 163.8 | 146.8 |
| Maximum range | Km | 0.43 | 0.32 | 8.87 | 3.13 |
| Coverage efficiency | sq km/site | 0.57 | 0.33 | 153.22 | 19.11 |

Table 22

Link Budgets for 384 kbps Pedestrian and 144kbps Vehicular UDD

|  |  | Downlink | Uplink | Downlink | Uplink |
| --- | --- | --- | --- | --- | --- |
| Test environment |  | Pedestrian | Pedestrian | Vehicular | Vehicular |
| Multipath channel class |  | A | A | A | A |
| Mobile speed |  | 3 km/h | 3 km/h | 120 km/h | 120 km/h |
| Test service |  | 384 kbps | 384 kbps | 144 kbps | 144 kbps |
| Note |  |  |  |  |  |
| Bit rate | bits/s | 559800 | 559800 | 209900 | 209900 |
| Average TX power per traffic ch | dBm | <20 | <14 | <30 | <24 |
| Maximum TX power per traffic ch | dBm | 20 | 14 | 30 | 24 |
| Maximum total TX power | dBm | 20 | 14 | 30 | 24 |
| Cable, conn and combiner losses | dBm | 2 | 0 | 2 | 0 |
| Tx antenna gain | dBi | 10 | 0 | 13 | 0 |
| TX EIRP per traffic channel | dBm | 28 | 14 | 41 | 24 |
| Total TX EIRP | dBm | 28 | 14 | 41 | 24 |
| RX antenna gain | dBi | 0 | 10 | 0 | 13 |
| Cable and connector losses | dB | 0 | 2 | 0 | 2 |
| Receiver noise figure | dB | 5 | 5 | 5 | 5 |
| Thermal noise density | dBm/Hz | -174 | -174 | -174 | -174 |
| RX interference density | dBm/Hz | -1000 | -1000 | -1000 | -1000 |
| Total effect noise+interf density | dBm/Hz | -169 | -169 | -169 | -169 |
| Information rate | dBHz | 57.5 | 57.5 | 53.2 | 53.2 |
| Required Eb/(No+Io) | dB | 2.6 | 4.5 | 1.7 | 4.6 |
| RX sensitivity | dB | -108.9 | -107.0 | -114.1 | -111.2 |
| Explicit diversity gain | dB | 0 | 0 | 0 | 0 |
| Other gain (DL repetition / UL subchannelization) | dB | 7.0 | 5.3 | 7.8 | 9.3 |
| Log-normal fade margin | dB | 11.2 | 11.2 | 11.4 | 11.4 |
| Maximum path loss | dB | 132.7 | 123.1 | 151.5 | 144.1 |
| Maximum range | km | 0.41 | 0.24 | 4.18 | 2.65 |
| Coverage efficiency | sq km/site | 0.54 | 0.18 | 33.97 | 13.73 |
| Optimized Parameters |  |  |  |  |  |
| Maximum TX power per traffic ch | dBm | 23 | 20 | 40 | 24 |
| Tx antenna gain | dBi | 10 | 0 | 17 | 0 |
| RX antenna gain | dBi | 0 | 10 | 0 | 17 |
| Maximum path loss | dB | 135.7 | 129.1 | 165.5 | 148.1 |
| Maximum range | km | 0.49 | 0.34 | 9.84 | 3.39 |
| Coverage efficiency | sq km/site | 0.76 | 0.36 | 188.68 | 22.40 |

### Generic Link Budget

The link budget below is based on the system parameters and channel propagation model in Table 13-Table 15, for a 2x2 (2-transmit and 2-receive antennas) antenna configuration in the downlink and a 1x2 (1-transmit and 2-receive antennas) antenna configuration in the uplink. The downlink parameters include Cyclic Shift Transmit Diversity (CSTD) and pilot boosting.  The value of 5.56 dB used for the Shadow Fade margin in the table assures a 75% coverage probability at the cell edge and 90% coverage probability over the entire area. Note that the maximum allowable path loss, 128.2 dB, corresponds to a DL cell-edge data rate of 5.76 Mbit/s and an UL cell-edge data rate of 115 kbit/s. Higher data rate at the cell edge and higher carrier frequency results in smaller cell size. Alternatively, better link budget and larger cell size can be achieved at lower cell edge data rates, as shown in the link budget.

Table 23

Link Budget

| Test environment | Outdoor to Indoor |  |  |  |
| --- | --- | --- | --- | --- |
| Test service | UDD(PUSC permutation) |  |  |  |
|  | Downlink |  | Uplink |  |
| Bit rate | 2.88 Mbit/s | 5.76Mbit/s | 38 kbit/s | 115 kbit/s |
| Average TX power per traffic ch. dBm | <40 | <40 | <23 | <23 |
| Maximum TX power per traffic ch. dBm | 40 | 40 | 23 | 23 |
| Maximum total TX power dBm | 40 | 40 | 23 | 23 |
| TX antenna gain dBi | 15 | 15 | -1 | -1 |
| Cyclic Combining Gain dB | 3 | 3 | 0 | 0 |
| Pilot Boosting Gain dB | -0.7 | -0.7 | 0 | 0 |
| TX EIRP per traffic channel dBm | 57.3 | 57.3 | 22 | 22 |
| Total TX EIRP dBm | 57.3 | 57.3 | 22 | 22 |
| RX antenna gain dBi | -1 | -1 | 15 | 15 |
| Receiver noise figure dB | 7 | 7 | 4 | 4 |
| Thermal noise density dBm/Hz | -174 | -174 | -174 | -174 |
| RX interference density dBm/Hz | -176.3 | -176.3 | -174 | -174 |
| Total effect. noise + interf. density dBm/Hz -169 | -165 | -165 | -167 | -167 |
| Information rate dBHz | 64.6 | 67.6 | 45.8 | 50.6 |
| Required Eb/(No+Io) dB | 10.5 | 13 | 12.6 | 12.6 |
| RX sensitivity dBm | -89.9 | -84.4 | -108.5 | -103.7 |
| Explicit diversity gain dB | 3 | 3 | 3 | 3 |
| Other gain dB (Building penetration) | 10 | 10 | 10 | 10 |
| Log-normal fade margin dB | 5.56 | 5.56 | 5.56 | 5.56 |
| Maximum path loss dB | 133.7 | 128.2 | 133 | 128.2 |
| Maximum range m | 436.2 | 318.9 | 420.4 | 319.4 |
| Coverage efficiency km2/site | 0.6 | 0.32 | 0.56 | 0.32 |

# 3	Self Evaluation

This section is in reference to Annex 3 of M.1225.

| Index | Criteria and attributes |  | Q or q |  | Gn |  | Related attributes in Annex 1 | Proponents Comments | Evaluators Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A3.1 | Spectrum efficiency : The following entries are considered in the evaluation of spectrum efficiency |  |  |  |  |  |  |  |  |
| A3.1.1 | For terrestrial environment |  |  |  |  |  |  |  |  |
| A3.1.1.1 | Voice traffic capacity (E/MHz/cell) in a total available assigned non-contiguous bandwidth of 30 MHz (15 MHz forward/15 MHz reverse) for FDD mode or contiguous bandwidth of 30 MHz for TDD mode. This metric must be used for a common generic continuous voice bearer with characteristics 8 kbit/s data rate and an average BER 1  10-3 as well as any other voice bearer included in the proposal which meets the quality requirements (assuming 50% voice activity detection (VAD) if it is used). For comparison purposes, all measures should assume the use of the deployment models in Annex 2, including a 1% call blocking. The descriptions should be consistent with the descriptions under criterion § 6.1.7 – Coverage/power efficiency. Any other assumptions and the background for the calculation should be provided, including details of any optional speech codecs being considered. |  | Q and q |  | G1 |  | A1.3.1.5.1 | TDD mode Voice capacity using VoIP: -90 Erlangs/MHz/cell for reuse 3, SIMO, 10 MHz PUSC Subchannelization -80 Erlangs/MHz/cell for reuse 3, SIMO, 5 MHz PUSC Subchannelization Assumptions: -ITU vehicular path loss model -Pedestrian B3 channel model |  |
| A3.1.1.2 | Information capacity (Mbit/s/MHz/cell) in a total available assigned non-contiguous bandwidth of 30 MHz (15 MHz forward/15 MHz reverse) for FDD mode or contiguous bandwidth of 30 MHz for TDD mode. The information capacity is to be calculated for each test service or traffic mix for the appropriate test environments. This is the only measure that would be used in the case of multimedia, or for classes of services using multiple speech coding bit rates. Information capacity is the instantaneous aggregate user bit rate of all active users over all channels within the system on a per cell basis. If the user traffic (voice and/or data) is asymmetric and the system can take advantage of this characteristic to increase capacity, it should be described qualitatively for the purposes of evaluation. |  | Q and q |  | G1 |  | A1.3.1.5.2 | For the packet data bearer (UDD) service: Data capacity: -DL SIMO 5MHz= 3.45 Mbit/s/MHz/cell -DL SIMO 10MHz = 3.57 Mbit/s/MHz/cell -UL SIMO 5MHz = 1.6 Mbit/s/MHz/cell -DL MIMO 10MHz= 5.52 Mbit/s/MHz/cell -UL SIMO 10MHz= 1.59 Mbit/s/MHz/cell -UL MIMO 10MHz= 2.1 Mbit/s/MHz/cell Assumptions: - PUSC, ITU vehicular, 60% Pedestrian B 3, 30% Vehicular A 30, 10% Vehicular A 120, -DL:UL=28:9 (payload only) |  |
| A3.1.2 | For satellite environment These values (§ A3.1.2.1 and A3.1.2.2) assume the use of the simulation conditions in Annex 2. The first definition is valuable for comparing systems with identical user channel rates. The second definition is valuable for comparing systems with different voice and data channel rates. |  |  |  |  |  |  |  |  |
| A3.1.2.1 | Voice information capacity per required RF bandwidth (bit/s/Hz) |  | Q |  | G1 |  | A1.3.2.3.1 | NA |  |
| A3.1.2.2 | Voice plus data information capacity per required RF bandwidth (bit/s/Hz) |  | Q |  | G1 |  | A1.3.2.3.2 | NA |  |
| A3.2 | Technology complexity – Effect on cost of installation and operation The considerations under criterion § 6.1.2 – Technology complexity apply only to the infrastructure, including BSs (the handportable performance is considered elsewhere). |  |  |  |  |  |  |  |  |
| A3.2.1 | Need for echo control The need for echo control is affected by the round trip delay, which is calculated as shown in Fig. 6. Referring to Fig. 6, consider the round trip delay with the vocoder (D1, ms) and also without that contributed by the vocoder (D2, ms). NOTE 1 – The delay of the codec should be that specified by ITU-T for the common generic voice bearer and if there are any proposals for optional codecs include the information about those also. |  | Q |  | G4 |  | A1.3.7.2 A1.3.7.3 | Echo control is needed for voice applications. The voice delay is also dependent on the codec used. Selection of the codec is implementation dependent and no specific codec is mandated. Echo control is used on the MS and also optionally on a need basis at the BS or Gateways. The performance characteristics meet the delay requirements outlined in ITU-R M.1079. |  |
| A3.2.2 | Transmitter power and system linearity requirements NOTE 1 – Satellite e.i.r.p. is not suitable for evaluation and comparison of RTTs because it depends very much on satellite orbit. The RTT attributes in this section impact system cost and complexity, with the resultant desirable effects of improving overall performance in other evaluation criteria. They are as follows. |  |  |  |  |  |  |  |  |
| A3.2.2.1 | Peak transmitter/carrier (Pb) power (not applicable to satellite) |  | Q |  | G1 |  | A1.2.16.2.1 | This is not limited by RTT but rather by regulations for the specific RF bands. Mobile Station @ 2.5GHz 23 dBm EIRP (Power class I, QPSK, Refer to Section A3.2.2.2) |  |
|  | Peak transmitter power for the BS should be considered because lower peak power contributes to lower cost. Note that Pb may vary with test environment application. This is the same peak transmitter power assumed in Appendix 2, link budget template (Table 23). |  |  |  |  |  |  | This is not limited by RTT but rather by regulations for the specific RF bands. |  |
| A3.2.2.2 | Broadband power amplifier (PA) (not applicable to satellite) Is a broadband power amplifier used or required? If so, what are the peak and average transmitted power requirements into the antenna as measured in watts. |  | Q |  | G1 |  | A1.4.10 A1.2.16.2.1 A1.2.16.2.2 A1.5.5 A1.2.5 | A broadband power amplifier is required. Tx Power is not limited by RTT but by regulations. BS - Tx dynamic range = 10 dB - Spectral flatness as per conditions in A.1.4.10 - Peak Tx power on BS is limited only by regulations and not by the RTT. MS - Tx dynamic range = 45 dB - Spectral flatness as per conditions in A.1.4.10 - 4 power classes are supported as shown below: Peak Transmit power (dBm) for 16QAM 1. 18 <= Ptx,max < 21 2. 21 <= Ptx,max < 25 3. 25 <= Ptx,max < 30 4. 30 <= Ptx,max Peak Transmit power (dBm) for QPSK 1. 20 <= Ptx,max < 23 2. 23 <= Ptx,max < 27 3. 27 <= Ptx,max < 30 4. 30 <= Ptx,max |  |
| A3.2.2.3 | Linear base transmitter and broadband amplifier requirements (not applicable to satellite) |  |  |  |  |  |  |  |  |
| A3.2.2.3.1 | Adjacent channel splatter/emission and intermodulation affect system capacity and performance. Describe these requirements and the linearity and filtering of the base transmitter and broadband PA required to achieve them. | q |  | G3 |  | A1.4.2 A1.4.10 |  | Base stations and terminals supporting this RTT will comply with local, regional, and international regulations for out of band and spurious emissions, wherever applicable. |  |
| A3.2.2.3.2 | Also state the base transmitter and broadband PA (if one is used) peak to average transmitter output power, as a higher ratio requires greater linearity, heat dissipation and cost. | Q and q |  | G2 |  | A1.4.10 A1.2.16.2.1 A1.2.16.2.2 |  | These are implementation dependent. The PAPR of the proposed RTT is around 12dB |  |
| A3.2.2.4 | Receiver linearity requirements (not applicable to satellite) Is BS receiver linearity required? If so, state the receiver dynamic range required and the impact of signal input variation exceeding this range, e.g., loss of sensitivity and blocking. | q |  | G4 |  | A1.4.11 A1.4.12 |  | BS Max input level on-channel reception tolerance = -45 dBm Max input level on-channel damage tolerance = -10 dBm MS Max input level on-channel reception tolerance = -30 dBm Max input level on-channel damage tolerance = 0 dBmBS/MS BS and MS Max input level sensitivity (Distributed permutation of subcarriers) for 10 MHz case: -88.5 dBm - QPSK-1/2 -85.1 dBm - QPSK-3/4 -82.8 dBm - 16QAM-1/2 -78.7 dBm - 16QAM-3/4 -77.6 dBm - 64QAM-1/2 -74.5 dBm - 64QAM-2/3 -73.4 dBm - 64QAM-3/4 -71.5 dBm - 64QAM-5/6 Max input level sensitivity (Distributed permutation of subcarriers) for 5 MHz case: -91.5 dBm - QPSK-1/2 -88.1 dBm - QPSK-3/4 -85.8 dBm - 16QAM-1/2 -81.7 dBm - 16QAM-3/4 -80.6 dBm - 64QAM-1/2 -77.5 dBm - 64QAM-2/3 -76.4 dBm - 64QAM-3/4 -74.5 dBm - 64QAM-5/6 Sensitivity numbers are calculated based on assumption of repetition factor 1 and Distributed permutation of subcarriers. |  |
| A3.2.3 | Power control characteristics (not applicable to satellite) Does the proposed RTT utilize transmitter power control? If so, is it used in both forward and reverse links? State the power control range, step size (dB) and required accuracy, number of possible step sizes and number of power controls per second, which are concerned with BS technology complexity. | Q and q |  | G4 |  | A1.2.22 A1.2.22.1 A1.2.22.2 A1.2.22.3 A1.2.22.4 A1.2.22.5 |  | Open loop and closed loop transmitter power control methods are used. Power control is done on the DL as well as the UL. Power control step size is variable ranging from 0.25 dB to 32 dB. An 8-bit signed integer in power control information element indicates the power control step size in 0.25 dB units. Normally implemented in 1 dB increments. The power control cycle of closed-loop or open-loop power control is dependent on the rate of power control information element transmission, but less than 200 Hz. The accuracy for power level control can vary from ± 0.5 dB to ± 2 dB depending on the power control step size. Single step size m ¦ Required relative accuracy ¦m¦ = 1dB¦ ± 0.5 dB ¦m¦ = 2dB¦± 1 dB ¦m¦ = 3dB¦± 1.5 dB 4dB <¦m¦< = 10 dB¦± 2 dB Two exception points of at least 10 dB apart are allowed over the 45 dB range, where in these two points an accuracy of up to ± 2 dB is allowed for any size step. The minimum power control dynamic range is 45 dB. The RTT supports 45 dB under the full power assumption |  |
| A3.2.4 | Transmitter/receiver isolation requirement (not applicable to satellite) If FDD is used, specify the noted requirement and how it is achieved. | q |  | G3 |  | A1.2.2 A1.2.2.2 A1.2.2.1 |  | Not Applicable as it is TDD. |  |
| A3.2.5 | Digital signal processing requirements |  |  |  |  |  |  |  |  |
| A3.2.5.1 | Digital signal processing can be a significant proportion of the hardware for some radio interface proposals. It can contribute to the cost, size, weight and power consumption of the BS and influence secondary factors such as heat management and reliability. Any digital circuitry associated with the network interfaces should not be included. However any special requirements for interfacing with these functions should be included. This section of the evaluation should analyse the detailed description of the digital signal processing requirements, including performance characteristics, architecture and algorithms, in order to estimate the impact on complexity of the BSs. At a minimum the evaluation should review the signal processing estimates (MOPS, memory requirements, gate counts) required for demodulation, equalization, channel coding, error correction, diversity processing (including Rake receivers), adaptive antenna array processing, modulation, A-D and D-A converters and multiplexing as well as some IF and baseband filtering. For new technologies, there may be additional or alternative requirements (such as FFTs). Although specific implementations are likely to vary, good sample descriptions should allow the relative cost, complexity and power consumption to be compared for the candidate RTTs, as well as the size and the weight of the circuitry. The descriptions should allow the evaluators to verify the signal processing requirement metrics, such as MOPS, memory and gate count, provided by the RTT proponent. | Q and q |  | G2 |  | A1.4.13 |  | The Hardware requirements are implementation dependent. For 5 MHz a 512 FFT and for 10 MHz and 1024 FFT is required. Memory and Processing needs are very much specific to the type of hardware. |  |
| A3.2.5.2 | What is the channel coding/error handling for both the forward and reverse links? Provide details and ensure that implementation specifics are described and their impact considered in DSP requirements described in § A3.2.5.1. | q |  | G4 |  | A1.2.12 A1.4.13 |  | An 8bit CRC is used for MAC PDU errors. Forward Error Correction schemes Convolutional Coding and Convolutional Turbo Coding are supported Modulation schemes: QPSK, 16 QAM and 64 QAM for downlink, QPSK and 16 QAM for uplink. Coding rates: QPSK 1/2, QPSK 3/4, 16 QAM 1/2, 16 QAM 3/4, 64 QAM 1/2, 64 QAM 2/3, 64 QAM 3/4, 64 QAM 5/6. Coding repetition rates: 1x, 2x, 4x and 6x. |  |
| A3.2.6 | Antenna systems |  |  |  |  |  |  |  |  |
|  | The implementation of specialized antenna systems while potentially increasing the complexity and cost of the overall system can improve spectrum efficiency (e.g. smart antennas), quality (e.g. diversity), and reduce system deployment costs (e.g. remote antennas, leaky feeder antennas). |  |  |  |  |  |  | MS: 1 Tx Antenna 2 Rx Antennas BS: 2 or more Tx Antennas 2 or more Rx Antennas Both MIMO and Beamforming support are mandatory at the Mobile Stations. Base Stations may support either MIMO or Beamforming. In general, it is expected for Beamforming to be deployed in scenarios where increased coverage is required (urban and suburban scenarios), while MIMO is expected to be employed in scenarios requiring high system capacity (urban scenarios). For MIMO operation: Adaptive switching between STC and SM is supported, see Section 1.3. 5 for a detailed description. Two transmit and two or more receive antennas are employed at the BS; one transmit and two receive antennas are supported at the MS. The typical antenna spacing at the BS and MS is 10 λ and 0.5 λ, respectively, where λ stands for the carrier wavelength. Regarding the type of equalizers for the SM MIMO mode, either minimum mean squared error (MMSE) or maximum-likelihhod (ML) based receivers will be implemented by MS vendors. Regarding the CSI, this is based either on physical or effective carrier-to-interference-and-noise ratio (CINR), while the communication of the MIMO mode is also enabled by the Mobile WiMAX system profiles. Please see also Section 1.3.5 for a detailed description. For Beamforming operation: Typically, a BS transceiver is equipped with 4 transmit and receive antennas but larger number of antennas can be used. The antenna spacing depends on the used Beamforming algorithm and can range from 0.5 λ to 3 λ. Regarding the weight update operation, see also Section 1.3.5, this is based on channel sounding, which is the process of channel estimation during the uplink operation for updating the antenna weights to be used for the subsequent transmission to a particular user in the downlink. Note that due to the channel reciprocity enabled by the TDD operation, the weights are accurate for low MS speeds, e.g., up to 30 km/h, while a graceful degradation of the performance is expected for higher speeds. Certainly, the accuracy of the antenna weights is also highly dependent on the specific Beamforming algorithm used at the BS, which may lead to smaller performance degradation at higher MS speeds. |  |
|  | NOTE 1 – For the satellite component, diversity indicates the number of satellites involved; the other antenna attributes do not apply. |  |  |  |  |  |  |  |  |
| A3.2.6.1 | Diversity : describe the diversity schemes applied (including micro and macro diversity schemes). Include in this description the degree of improvement expected, and the number of additional antennas and receivers required to implement the proposed diversity design beyond and omni-directional antenna. | Q |  | G2 |  | A1.2.23 A1.2.23.1 A1.2.23.2 |  | When the MIMO option is deployed: In the downlink, both transmit diversity and receive diversity is supported through the use of STC (use of the Alamouti code which is a space-time block coding code for two transmit antennas, while two receive antennas are used at the MS for receive diversity). Note that when SM is used, although there is also inherent transmit and receive diversity due to the use of two antennas at both the BS and MS, the target is the increase of the peak rate by transmitting two data streams over one OFDMA symbol per subcarrier, see also Section 1.3.5 for a detailed description. In the uplink where CSM (collaborative spatial multiplexing) is supported, receive diversity is applied by the use of two or more receive antennas at the BS. Depending on the propagation environment (mainly characterized by the frequency and time diversity of the link-level channel model), the signal-to-noise ratio (SNR) gain of STC ranges from 4 dB to 7dB compared to a single antenna system; the SNR gain of SM ranges from 2 dB to 4 dB compared to a single antenna system, where there is double data throughput supported by SM compared to the single antenna system. Regarding the CSM mode, higher gains on the order of 1 dB to 2 dB are expected compared to the SM gains reported above. When the Beamforming option is applied: In the downlink, transmit diversity is supported, while receive diversity is also applied when two receive antennas are used at the MS. In the uplink, receive diversity is supported by using multiple antenna reception at the BS. For a typical implementation of 4 receive and transmit antennas for Bemaforming, the SNR gains at both the uplink and the downlink are expected to range from 6 dB to 12 dB. |  |
| A3.2.6.2 | Remote antennas : describe whether and how remote antenna systems can be used to extend coverage to low traffic density areas. | q |  | G2 |  | A1.3.6 |  | These can be used for extending coverage. Performance is implementation and deployment scenario specific. |  |
| A3.2.6.3 | Distributed antennas : describe whether and how distributed antenna designs are used. | q |  | G3 |  | A1.3.6 |  | They can be used in microcellular environments. |  |
| A3.2.6.4 | Unique antenna : describe additional antenna systems which are either required or optional for the proposed system, e.g., beam shaping, leaky feeder. Include in the description the advantage or application of the antenna system. | q |  | G4 |  | A1.3.6 |  | MIMO and Beamforming types of Smart Antenna capability are supported. MIMO is used for capacity enhancements. Beamforming is used for coverage enhancement. |  |
| A3.2.7 | BS frequency synchronization/time alignment requirements Does the proposed RTT require base transmitter and/or receiver station synchronization or base-to-base bit time alignment? If so, specify the long term (1 year) frequency stability requirements, and also the required bit-to-bit time alignment. Describe the means of achieving this. | Q and q |  | G3 |  | A1.4.1 A1.4.3 |  | As it is a TDD system, BS synchronization is required. Methods used are implementation dependent. GPS based methods are typically used. BS frequency tolerance ≤ ± 2ppm of carrier frequency BS to BS frequency accuracy ≤ ± 1% of subcarrier spacing MS to BS frequency synchronization tolerance ≤ 2% of the subcarrier spacing. Time alignment between BS and MS is achieved using the Downlink Preambles and the Uplink ranging operation which corrects time offset errors. The OFDMA Cyclic Prefix marks the Symbol level time alignment. |  |
| A3.2.8 | The number of users per RF carrier/frequency channel that the proposed RTT can support affects overall cost – especially as bearer traffic requirements increase or geographic traffic density varies widely with time. Specify the maximum number of user channels that can be supported while still meeting ITU-T Recommendation G.726 performance requirements for voice traffic. | Q |  | G1 |  | A1.2.17 |  | The maximum number of voice channels per 1 RF channel depends on the bit rate and sampling rate supported by the codecs defined in the G.726. For instance, in case of the bit rate of 16 kbit/s with 20 msec sampling rate, up to 256 users can be supported simultaneously by a 10 MHz RF channel, while meeting the delay requirements of VoIP. In the case of a 5 MHz channel up to 120 users can be supported. The performance characteristics meet the delay and traffic requirements outlined in ITU-R M.1079. |  |
| A3.2.9 | Base site implementation/installation requirements (not applicable to satellite) BS size, mounting, antenna type and height can vary greatly as a function of cell size, RTT design and application environment. Discuss its positive or negative impact on system complexity and cost. | q |  | G1 |  | A1.4.17 |  | No RTT specific requirements exist. |  |
| A3.2.10 | Handover complexity Consistent with handover quality objectives defined in criterion § 6.1.3, describe how user handover is implemented for both voice and data services and its overall impact on infrastructure cost and complexity. | Q and q |  | G1 |  | A1.2.24 A1.4.6.1 |  | Simple Hard Handover and Optimized Hard Handover is supported. As the MS is only attached to one BS at a time significantly less complexity is expected. As voice is supported as an application over the IP data bearer the handover is always treated as a data connection. Base stations and Mobile stations implement the ability to buffer data during handover as well the protocols necessary for handover. See section 2.2.2.2 for handover performance analysis. |  |
| A3.3 | Quality |  |  |  |  |  |  |  |  |
| A3.3.1 | Transparent reconnect procedure for dropped calls Dropped calls can result from shadowing and rapid signal loss. Air interfaces utilizing a transparent reconnect procedure – that is, the same as that employed for hand-off – mitigate against dropped calls whereas RTTs requiring a reconnect procedure significantly different from that used for hand-off do not. | q |  | G2 |  | A1.4.14 |  | Voice is supported as an application over the RTT. The RTT is primarily designed to support Voice using Voice Over IP Protocols. MAC connections that provide reliable Quality of Service for Voice Over IP data flows are supported. These data connections are managed using timers and well as MAC layer signaling to ensure a reliable connection is maintained. Transparent reconnects are provided by the application layer for the voice traffic. As the RTT supports Adaptive Modulation and Coding, and Link Adaptation methods, the MAC level transport connections are managed to make them reliable. |  |
| A3.3.2 | Round trip delay, D1 (with vocoder (ms)) and D2 (without vocoder (ms)) (See Fig. 6). NOTE 1 – The delay of the codec should be that specified by ITU-T for the common generic voice bearer and if there are any proposals for optional codecs include the information about those also. (For the satellite component, the satellite propagation delay is not included.) | Q |  | G2 |  | A1.3.7.1 A1.3.7.2 |  | Assuming G.729 with a vocoder delay of 20ms for a 20 Byte voice sample. D1 = 20ms (vocoder) + 50ms (max one-way air interface delay) x 2 = 120ms D2 = 50ms x 2 = 100ms |  |
| A3.3.3 | Handover/ALT quality Intra switch/controller handover directly affects voice service quality. Handover performance, minimum break duration, and average number of handovers are key issues. | Q |  | G2 |  | A1.2.24 A1.2.24.1 A1.2.24.2 A1.4.6.1 |  | Handover signaling is designed to minimize loss of data. Handover latency is <= 50ms if no network re-entry is required. This ensures minimum disruption to data transfer. If NW re-entry is required the latency is <= 85ms. Handover frequency is scenario specific. |  |
| A3.3.4 | Handover quality for data There should be a quantitative evaluation of the effect on data performance of handover. | Q |  | G3 |  | A1.2.24 A1.2.24.1 A1.2.24.2 A1.4.6.1 |  | Handover for voice and data are treated the same way in this RTT. |  |
| A3.3.5 | Maximum user bit rate for data (bit/s) A higher user bit rate potentially provides higher data service quality (such as high quality video service) from the user’s point of view. | Q |  | G1 |  | A1.3.3 |  | The maximum bit rates are well above 20160 kbit/s. (DL/UL ratio = 2:1, PUSC, 64QAM, 5/6 coding rate) |  |
| A3.3.6 | Channel aggregation to achieve higher user bit There should also be a qualitative evaluation of the method used to aggregate channels to provide higher bit rate services. | q |  | G4 |  | A1.2.32 |  | No channel aggregation is necessary as IP-OFDMA can operate over the entire 10 MHz channel. However, flexible allocation of subchannels (in frequency domain) within an RF channel can be used to dynamically allocate bandwidth to individual users for various bit rate services (see also Section s 1.3.1 to 1.3.3) . |  |
| A3.3.7 | Voice quality Recommendation ITU-R M.1079 specifies that FPLMTS speech quality without errors should be equivalent to ITU-T Recommendation G.726 (32 kbit/s ADPCM) with desired performance at ITU-T Recommendation G.711 (64 kbit/s PCM). NOTE 1 – Voice quality equivalent to ITU-T Recommendation G.726 error free with no more than a 0.5 degradation in MOS in the presence of 3% frame erasures might be a requirement. | Q and q |  | G1 |  | A1.2.19 A1.3.8 |  | The vocoder is independent of the RTT. Any suitable vocoder can be used as voice is supported over using Voice over IP protocol. Therefore the MOS values for the G.726 or any other vocoder used will apply. |  |
| A3.3.8 | System overload performance (not applicable to satellite) Evaluate the effect on system blocking and quality performance on both the primary and adjacent cells during an overload condition, at e.g. 125%, 150%, 175%, 200%. Also evaluate any other effects of an overload condition. | Q and q |  | G3 |  | A1.3.9.1 |  | System overload causes graceful degradation as data transmission bandwidth can be traded off for lower quality connections. As adaptive modulation and coding are supported the system adapts to the load conditions as per the policies implemented. |  |
| A3.4 | Flexibility of radio technologies |  |  |  |  |  |  |  |  |
| A3.4.1 | Services aspects |  |  |  |  |  |  |  |  |
| A3.4.1.1 | Variable user bit rate capabilities Variable user bit rate applications can consist of the following: – adaptive signal coding as a function of RF signal quality; – adaptive voice coder rate as a function of traffic loading as long as ITU-T Recommendation G.726 performance is met; – variable data rate as a function of user application; – variable voice/data channel utilization as a function of traffic mix requirements. Some important aspects which should be investigated are as follows: – how is variable bit rate supported? – what are the limitations? Supporting technical information should be provided such as – the range of possible data rates, – the rate of changes (ms). | q and Q |  | G2 |  | A1.2.18 A1.2.18.1 |  | The user bit rates are variable according to the number of subchannels assigned and modulation and coding rate used. The rates can be changed every 5ms which is every frame. The DL-MAP and UL-MAP signal the changes every frame. DOWNLINK BW: 10 MHz Modulation : QPSK, 16 QAM, 64 QAM Coding rate : 1/2, 2/3, 3/4, 5/6 Data rates: 9.6 kbit/s to 23040 kbit/s UPLINK BW: 10 MHz Modulation : QPSK, 16 QAM Coding rate : 1/2, 3/4 Data rates: 9.6kbit/s to 6048 kbit/s |  |
| A3.4.1.2 | Maximum tolerable Doppler shift, Fd (Hz) for which voice and data quality requirements are met (terrestrial only) Supporting technical information: Fd | q and Q |  | G3 |  | A1.3.1.4 |  | Fd ~500 Hz Voice and Data are treated the same way from the Physical layer perspective. |  |
| A3.4.1.3 | Doppler compensation method (satellite component only) What is the Doppler compensation method and residual Doppler shift after compensation? | Q and q |  | G3 |  | A1.3.2.2 |  | NA |  |
| A3.4.1.4 | How the maximum tolerable delay spread of the proposed technology impact the flexibility (e.g., ability to cope with very high mobile speed)? | q |  | G3 |  | A1.3.1.3 A1.2.14 A1.2.14.1 A1.2.14.2 A1.3.10 |  | ~20µs of delay spread can be tolerated without an equalizer. |  |
| A3.4.1.5 | Maximum user information bit rate, Ru (kbit/s) How flexibly services can be offered to customers ? What is the limitation in number of users for each particular service? (e.g. no more than two simultaneous 2 Mbit/s users) | Q and q |  | G2 |  | A1.3.3 A1.3.1.5.2 A1.2.31 A1.2.32 |  | Assuming 10 MHz PUSC: - 23040 kbit/s for the Downlink (DL:UL=35:12) - 6048 kbit/s for the Uplink for (DL:UL=26:21) Services are very flexible as the Subchannels can be grouped to increase data rates. |  |
| A3.4.1.6 | Multiple vocoder rate capability – bit rate variability, – delay variability, – error protection variability. | Q and q |  | G3 |  | A1.2.19 A1.2.19.1 A1.2.7 |  | Yes. Vocoders are however independent of the RTT and are implementation specific. The data transports for voice can operate at varying levels of Packet error rate and using H-ARQ can significantly boost performance. |  |
| A3.4.1.7 | Multimedia capabilities The proponents should describe how multimedia services are handled. The following items should be evaluated: – possible limitations (in data rates, number of bearers), – ability to allocate extra bearers during of the communication, – constraints for handover. | Q and q |  | G1 |  | A1.2.21 A1.2.20 A1.3.1.5.2 A1.2.18 A1.2.24 A1.2.30 A1.2.30.1 |  | The Data bearers have no constraints on the type of media they can carry. However typically they are mapped to the QoS of the media type being transmitted. There are no limits on the number of bearers as long as bandwidth is available. Extra bearers can be allocated during communication. There are no handover constraints as long as coverage is available. |  |
| A3.4.2 | Planning |  |  |  |  |  |  |  |  |
| A3.4.2.1 | Spectrum related matters |  |  |  |  |  |  |  |  |
| A3.4.2.1.1 | Flexibility in the use of the frequency band The proponents should provide the necessary information related to this topic (e.g., allocation of sub-carriers with no constraints, handling of asymmetric services, usage of non-paired band). | q |  | G1 |  | A1.2.1 A1.2.2 A1.2.2.1 A1.2.3 A1.2.5.1 |  | A 5 MHz or 10 MHz TDD carrier may be deployed with 1:3:3 frequency re-use or 1:3:1 reuse. |  |
| A3.4.2.1.2 | Spectrum sharing capabilities The proponent should indicate how global spectrum allocation can be shared between operators in the same region. The following aspects may be detailed: – means for spectrum sharing between operators in the same region, – guardband between operators in case of fixed sharing. | q and Q |  | G4 |  | A1.2.26 |  | The proposed RTT utilizes OFDMA which has inherent interference protection capabilities due to allocation of a varying subset of available sub-carriers to different users. So spectrum sharing is carried out using multiple channel carriers. The guard bands are RF band specific. |  |
| A3.4.2.1.3 | Minimum frequency band necessary to operate the system in good conditions Supporting technical information: – impact of the frequency reuse pattern, – bandwidth necessary to carry high peak data rate. | Q and q |  | G1 |  | A1.2.1 A1.4.15 A1.2.5 |  | 5 MHz or 10 MHz 1x3x3 PUSC or 1x3x1 PUSC may be used. 10 MHz gives the optimal data rate. |  |
| A3.4.2.2 | Radio resource planning |  |  |  |  |  |  |  |  |
| A3.4.2.2.1 | Allocation of radio resources The proponents and evaluators should focus on the requirements and constraints imposed by the proposed technology. More particularly, the following aspects should be considered: – what are the methods used to make the allocation and planning of radio resources flexible? – what are the impacts on the network side (e.g. synchronization of BSs, signalling,)? – other aspects. Examples of functions or type of planning required which may be supported by the proposed technology: – DCA, – frequency hopping, – code planning, – time planning, – interleaved frequency planning. NOTE 1 – The use of the second adjacent channel instead of the adjacent channel at a neighbouring cluster cell is called “interleaved frequency planning”. In some cases, no particular functions are necessary (e.g. frequency reuse  1). | q |  | G2 |  | A1.2.25 A1.2.27 A1.4.15 |  | Subchannelization schemes and zones namely PUSC and AMC are supported to provide flexibility in utilizing the frequency and time resources. Sectorized deployments are possible with flexible frequency re-use (1x3x3 or 1x3x1) using PUSC subchannelization schemes. Slots of multiple subchannels and OFDM symbols are used to manage the resource allocation granularity BSs need to be synchronized. This is typically done using GPS on the BS. No frequency planning is required across cells. |  |
| A3.4.2.2.2 | Adaptability to adapt to different and/or time varying conditions (e.g., propagation, traffic) How the proposed technology cope with varying propagation and/or traffic conditions? Examples of adaptive functions which may be supported by the proposed technology: – DCA, – link adaptation, – fast power control, – adaptation to large delay spreads. Some adaptivity aspects may be inherent to the RTT. | q |  | G2 |  | A1.3.10 A1.2.27 A1.2.22 A1.2.14 |  | Subchannelization and slot structure capability provides the ability to schedule frequency/time resources to mitigate the effects of propagation losses and also for traffic load balancing. Link adaptation schemes with CQI feedback capability allow operating the link more efficiently. H-ARQ also allows operations at high packet error rates resulting higher spectral efficiency as higher order coding and modulation rates can be used. The OFDMA symbol structure is designed to reduce the effects of delay spreads up to 20µs. |  |
| A3.4.2.3 | Mixed cell architecture (not applicable to satellite component) |  |  |  |  |  |  |  |  |
| A3.4.2.3.1 | Frequency management between different layers What kind of planning is required to manage frequencies between the different layers? e.g. – fixed separation, – dynamic separation, – possibility to use the same frequencies between different layers. Possible supporting technical information: – guard band. | q and Q |  | G1 |  | A1.2.28 A1.4.15 |  | Hierarchical layered cells are possible. The type of frequency planning is implementation/deployment scenario specific. The same frequencies can be used across layers by proper segmentation of the PUSC Subchannels. |  |
| A3.4.2.3.2 | User adaptation to the environment What are the constraints to the management of users between the different cell layers? e.g. – constraints for handover between different layers, – adaptation to the cell layers depending on services, mobile speed, mobile power. | q |  | G2 |  | A1.2.28 A1.3.10 |  | The RTT does not impose constraints on the management of users between different cell layers in such a hierarchical deployment. |  |
| A3.4.2.4 | Fixed-wireless access |  |  |  |  |  |  |  |  |
| A3.4.2.4.1 | The proponents should indicate how well its technology is suited for operation in the fixed wireless access environment. Areas which would need evaluation include (not applicable to satellite component): – ability to deploy small BSs easily, – use of repeaters, – use of large cells, – ability to support fixed and mobile users within a cell, – network and signaling simplification. | q |  | G4 |  | A1.1.3 A1.3.5 A1.4.17 A1.4.7 A1.4.7.1 |  | The RTT is very much suited for fixed wireless access as well. Pico or Micro cells or Macro cells and repeaters are possible. Both fixed and mobile users can work in the same cell. Network signaling for fixed devices are simpler compared to mobile devices. |  |
| A3.4.2.4.2 | Possible use of adaptive antennas (how well suited is the technology) (not applicable to satellite component) Is RTT suited to introduce adaptive antennas? Explain the reason if it is. | q |  | G4 |  | A1.3.6 |  | Yes the RTT supports adaptive antenna/Beamforming solutions. |  |
| A3.4.2.4.3 | Existing system migration capability | q |  | G1 |  | A1.4.16 |  | NA |  |
| A3.5 | Implication on network interface |  |  |  |  |  |  |  |  |
| A3.5.1 | Examine the synchronization requirements with respect to the network interfaces. Best case : no special accommodation necessary to provide synchronization. Worst case : special accommodation for synchronization is required, e.g. additional equipment at BS or special consideration for facilities. | q |  | G4 |  | A1.4.3 |  | Synchronization of the BSs across the network is required and this is typically accomplished using GPS. |  |
| A3.5.2 | Examine the RTTs ability to minimize the network infrastructure involvement in cell handover. Best case : neither PSTN/ISDN nor mobile switch involvement in handover. Worst case : landline network involvement essential for handover. | q |  | G3 |  | A1.2.24 A1.4.6.1 |  | Handover within the same ASN (Access Service Network) does not involve the CSN (Core Service Network). In most handover scenarios with neighboring cells there is minimal involvement of the CSN. Only the BS and ASN GW may need to be involved in these scenarios. |  |
| A3.5.3 | Landline feature transparency |  |  |  |  |  |  |  |  |
| A3.5.3.1 | Examine the network modifications required for the RTT to pass the standard set of ISDN bearer services. Best case : no modifications required. Worst case: substantial modification required, such as interworking functions. | q |  | G1 |  | A1.4.7.1 |  | ISDN is supported as an application running over the IP protocol and is not natively supported. As voice is supported using Voice over IP protocols, the use of ISDN is only involved interworking functions between the IP networks and PSTN. |  |
| A3.5.3.2 | Examine the extent of the PSTN/ISDN involvement in switching functionality. Best case : all switching of calls is handled by the PSTN/ISDN. Worst case : a separate mobile switch is required. | q |  | G2 |  | A1.4.6 A1.4.8 |  | PSTN/ISDN is not used for switching within the IP network. |  |
| A3.5.3.3 | Examine the depth and duration of fading that would result in a dropped call to the PSTN/ISDN network. The robustness of an RTTs ability to minimize dropped calls could be provided by techniques such as transparent reconnect. | Q and q |  | G3 |  | A1.2.24 A1.4.14 |  | Voice is supported as an application over the RTT. The robustness of the link maintained is implementation dependent. The RTT supports HARQ and hence can operate in higher Packer Error Rates up to 10%. |  |
| A3.5.3.4 | Examine the quantity and type of network interfaces necessary for the RTT based on the deployment model used for spectrum and coverage efficiencies. The assessment should include those connections necessary for traffic, signalling and control as well as any special requirements, such as soft handover or simulcast. | Q |  | G2 |  | A1.2.30 A1.2.30.1 A1.4.9 |  | The RTT design is to minimize impacts on the network. All the connections necessary for traffic, signaling and control terminate on the BS for PHY/MAC layer. The Radio Resource Management functions implemented over the IP protocol reside in the ASN. So most RTT configuration parameters are controlled on the BS which is interfaced using an IP connection to the ASN-GW . |  |
| A3.6 | Handportable performance optimization capability |  |  |  |  |  |  |  |  |
| A3.6.1 | Isolation between transmitter and receiver Isolation between transmitter and receiver has an impact on the size and weight of the handportable. | Q |  | G2 |  | A1.2.2 A1.2.2.1 A1.2.2.2 |  | As the RTT is a TDD based technology, no specific isolation requirements exist. |  |
| A3.6.2 | Average terminal power output P0 (mW) Lower power gives longer battery life and greater operating time. | Q |  | G2 |  | A1.2.16.1.2 |  | This is implementation dependent. The terminals have different power classes to which they belong as shown in A3.2.2.2.2. |  |
| A3.6.3 | System round trip delay impacts the amount of acoustical isolation required between hand portable microphone and speaker components and, as such, the physical size and mechanical design of the subscriber unit. NOTE 1 – The delay of the codec should be that specified by ITU-T for the common generic voice bearer and if there are any proposals for optional codecs include the information about those also. (For the satellite component, the satellite propagation delay is not included.) | Q and q |  | G2 |  | A1.3.7 A1.3.7.1 A1.3.7.2 A1.3.7.3 |  | The Round trip delay will be well within the ITU-T specified limits for a typical Voice application that may be implemented using the RTT. |  |
| A3.6.4 | Peak transmission power | Q |  | G1 |  | A1.2.16.1.1 |  | This is not limited by RTT but by regulations. The peak terminal power output P0 = 1000 mW (Power class 3). Also see A3.2.2.2.2 for more details. |  |
| A3.6.5 | Power control characteristics Does the proposed RTT utilize transmitter power control? If so, is it used in both forward and reverse links? State the power control range, step size (dB) and required accuracy, number of possible step sizes and number of power controls per second, which are concerned with mobile station technology complexity. |  |  |  |  |  |  | Yes the RTT does utilize transmitter power control for both Downlink and Uplink. |  |
| A3.6.5.1 | Power control dynamic range Larger power control dynamic range gives longer battery life and greater operating time. | Q |  | G3 |  | A1.2.22 A1.2.22.3 A1.2.22.4 |  | The minimum power control dynamic range is 45 dB. |  |
| A3.6.5.2 | Power control step size, accuracy and speed | Q |  | G3 |  | A1.2.22 A1.2.22.1 A1.2.22.2 A1.2.22.5 |  | The accuracy for power level control can vary from ± 0.5 dB to ± 2 dB depending on the power control step size. Single step size m ¦ Required relative accuracy ¦m¦ = 1dB¦ ± 0.5 dB ¦m¦ = 2dB¦ ± 1 dB ¦m¦ = 3dB¦ ± 1.5 dB 4dB< ¦m¦< = 10dB¦ ± 2 dB Two exception points of at least 10 dB apart are allowed over the 45 dB range, where in these two points an accuracy of up to +/- 2 dB is allowed for any size step. |  |
| A3.6.6 | Linear transmitter requirements | q |  | G3 |  | A1.4.10 |  | Linear transmitters are used on the BS and MS. |  |
| A3.6.7 | Linear receiver requirements (not applicable to satellite) | q |  | G3 |  | A1.4.11 |  | Linear receivers are used on the BS and MS. |  |
| A3.6.8 | Dynamic range of receiver The lower the dynamic range requirement, the lower the complexity and ease of design implementation. | Q |  | G3 |  | A1.4.12 |  | 80dB for the MS receiver and 65dB for the BS receiver |  |
| A3.6.9 | Diversity schemes Diversity has an impact on hand portable complexity and size. If utilized describe the type of diversity and address the following two attributes. | Q and q |  | G1 |  | A1.2.23 A1.2.23.1 A1.2.23.2 |  | MIMO and Beamforming are supported. Within the MIMO scheme both Transmit Diversity and Spatial Multiplexing are supported. |  |
| A3.6.10 | The number of antennas | Q |  | G1 |  | A1.2.23.1 |  | BS: 2 Tx, 2 Rx MS: 1 Tx, 2 Rx |  |
| A3.6.11 | The number of receivers | Q |  | G1 |  | A1.2.23.1 |  | BS: 2 Receivers MS : 2 Receivers |  |
| A3.6.12 | Frequency stability Tight frequency stability requirements contribute to handportable complexity. | Q |  | G3 |  | A1.4.1.2 |  | BS frequency tolerance ≤ ± 2ppm of carrier frequency BS to BS frequency accuracy ≤ ± 1% of subcarrier spacing MS to BS frequency synchronization tolerance ≤ 2% of the subcarrier spacing |  |
| A3.6.13 | The ratio of “off (sleep)” time to “on” time | Q |  | G1 |  | A1.2.29 A1.2.29.1 |  | This implementation dependent and is programmable by the BS or MS implementations. |  |
| A3.6.14 | Frequency generator step size, switched speed and frequency range Tight step size, switch speed and wide frequency range contribute to handportable complexity. Conversely, they increase RTT flexibility. | Q |  | G2 |  | A1.4.5 |  | Frequency step size : 200 and 250 KHz Switched speed : 200 μsec Frequency range : 5, 10 MHz |  |
| A3.6.15 | Digital signal processing requirements Digital signal processing can be a significant proportion of the hardware for some radio interface proposals. It can contribute to the cost, size, weight and power consumption of the BS and influence secondary factors such as heat management and reliability. Any digital circuitry associated with the network interfaces should not be included. However any special requirements for interfacing with these functions should be included. This section of the evaluation should analyse the detailed description of the digital signal processing requirements, including performance characteristics, architecture and algorithms, in order to estimate the impact on complexity of the BSs. At a minimum the evaluation should review the signal processing estimates (MOPS, memory requirements, gate counts) required for demodulation, equalization, channel coding, error correction, diversity processing (including Rake receivers), adaptive antenna array processing, modulation, A-D and D-A converters and multiplexing as well as some IF and baseband filtering. For new technologies, there may be additional or alternative requirements (such as FFTs). Although specific implementations are likely to vary, good sample descriptions should allow the relative cost, complexity and power consumption to be compared for the candidate RTTs, as well as the size and the weight of the circuitry. The descriptions should allow the evaluators to verify the signal processing requirement metrics, such as MOPS, memory and gate count, provided by the RTT proponent. | Q and q |  | G1 |  | A1.4.13 |  | These are again implementation dependent. |  |
| A3.7.1.1 | Base site coverage efficiency The number of base sites required to provide coverage at system start-up and ongoing traffic growth significantly impacts cost. From § 1.3.2 of Annex 2, determine the coverage efficiency, C (km2/base sites), for the lowest traffic loadings. Proponent has to indicate the background of the calculation and also to indicate the maximum coverage range. | Q |  | G1 |  | A1.3.1.7 A1.3.1.7.1 A1.3.1.7.2 A1.3.4 |  | 80-95% at system startup 95-100% in a mature system See section 2.2.4.2 for more details. |  |
| A3.7.1.2 | Method to increase the coverage efficiency Proponent describes the technique adopted to increase the coverage efficiency and drawbacks. Remote antenna systems can be used to economically extend vehicular coverage to low traffic density areas. RTT link budget, propagation delay system noise and diversity strategies can be impacted by their use. Distributed antenna designs – similar to remote antenna systems – interconnect multiple antennas to a single radio port via broadband lines. However, their application is not necessary limited to providing coverage, but can also be used to economically provide continuous building coverage for pedestrian applications. System synchronization, delay spread, and noise performance can be impacted by their use. | q |  | G1 |  | A1.3.5 A1.3.6 |  | MIMO and Beamforming can be used to increase coverage efficiency. Remote or Distributed antenna systems can also be used. However the use of these methods is deployment scenario specific based on the implementations. |  |
| A3.7.2 | Satellite Normalized power efficiency Supported information bit rate per required carrier power-to-noise density ratio for the given channel performance under the given interference conditions for voice Supported information bit rate per required carrier power-to-noise density ratio for the given channel performance under the given interference conditions for voice plus data mixed traffic. | Q |  | G1 |  | A1.3.2.4 A1.3.2.4.1 A1.3.2.4.2 |  | NA |  |

# 4	References

[1]	IEEE Std 802.16e-2005, Amendment for Combined Fixed and Mobile Broadband Wireless Access Systems, December 2005 http://standards.ieee.org/getieee802/download/802.16e-2005.pdf

[2]	IEEE Std 802.16™-2004, IEEE Standard for Local and metropolitan area networks: Part 16: Air Interface for Fixed Broadband Wireless Access Systems, June 2004 http://standards.ieee.org/getieee802/download/802.16-2004.pdf

[3]	WiMAX Forum™ Mobile System Profile, Release 1.0 Approved Specification, Revision 1.2.2: 2006-11-17

http://www.wimaxforum.org/technology/documents/WiMAX_Forum_Mobile_System_Profile_v1_2_2.pdf

[4]	Mobile WiMAX – Part I: A Technical Overview and Performance Evaluation, August 2006, WiMAX Forum

http://www.wimaxforum.org/news/downloads/Mobile_WiMAX_Part1_Overview_and_Performance.pdf

[5]	Mobile WiMAX Network Architecture and Specifications

http://www.wimaxforum.org/technology/documents/WiMAX_NWG_Stage_2_VandV_Readiness_Draft.zip

[6]	3GPP TSG-RAN-1, "Effective SIR Computation for OFDM System-Level Simulations", R1-03-1370, Meeting #35, Lisbon, Portugal, November 2003

[7]	Hujun Yin and Siavash Alamouti, “OFDMA – A Broadband Wireless Access Technology”, IEEE Proc. of Sarnoff Symposium, March 2006

______________

“WiMAX Forum®” is a registered trademark of the WiMAX Forum and “WiMAX” and “Mobile WiMAX” are trademarks of the WiMAX Forum.  All other trademarks are the properties of their respective owners.

An ASN profile represents an allocation of functional entities (e.g. authenticator, radio resource manager, etc.) to the various elements belonging to the access network.

PHY Data Rate=(Data sub-carriers/Symbol period)*(information bits per symbol).

*	Explanation is requested when the candidate SRTT checks the No box.

*	Explanation is requested when the candidate SRTT checks the No box.