3GPP TSG RAN WG1 Meeting #84bis 	R1-163672

- Busan, South Korea, 11th - 15th April 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 0645 | rev | - | Current version: | 13.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 36.213 on multi-channel access procedure Type A2 in LAA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Alcatel-Lucent Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Core |  |  |  |  |  |  | Date: |  | 2016-04-14 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | For multi-channel access procedure Type A2, the reset of backoff counter is only defined under the condition that “the absence of any other technology sharing the carrier cannot be guaranteed on a long term basis (e.g. by level of regulation)”. However, the reset should occur regardless of whether LAA is sharing the carrier with other technologies or not. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The backoff counter in multi-channel access procedure Type A2 is reset after transmission regardless of whether LAA is sharing the carrier with other technologies or not. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reset condition of the backoff counter for multi-channel access procedure Type A2 in the absence of any other technology is not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.1.5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 15	Channel access procedures for LAA

## 15.1	Downlink channel access procedures

### …

### 15.1.5	Channel access procedure for transmission(s) on multiple channels

An eNB can access multiple channels on which LAA Scell(s) transmission(s) are performed, according to one of the Type A or Type B procedures described in this subclause.

#### 15.1.5.1	Type A multi-channel access procedures

The eNB shall perform channel access on each channel ci∈C, according to the procedures described in subclause 15.1.1, where C is a set of channels on which the eNB intends to transmit, and i=0,1,…q−1, and qis the number of channels on which the eNB intends to transmit.

The counter N described in subclause 15.1.1 is determined for each channel ci and is denoted asNci. Nci is maintained according to subclause 15.1.5.1.1or 15.1.5.1.2.

##### 15.1.5.1.1	Type A1

Counter N as described in subclause 15.1.1 is independently determined for each channel ci and is denoted asNci.

If the absence of any other technology sharing the carrier cannot be guaranteed on a long term basis (e.g. by level of regulation), when the eNB ceases transmission on any one channel cj∈C, for each channelci≠cj, the eNB can resume decrementing Nci when idle slots are detected either after waiting for a duration of 4⋅Tsl, or after reinitialisingNci.

##### 15.1.5.1.1	Type A2

Counter Nis determined as described in subclause 15.1.1 for channel cj∈C, and is denoted as Ncj, where cjis the channel that has the largest CWpvalue. For each channel ci, Nci=Ncj.

When the eNB ceases transmission on any one channel for which Nci is determined, the eNB shall reinitialiseNcifor all channels.