3GPP TSG-RAN WG4 Meeting #78bis 	R4-162716

- San José Del Cabo, México, 11-15 April, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 3402 | rev | 1 | Current version: | 13.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on measurement reference channel and OCNG with FS3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Perf |  |  |  |  |  |  | Date: |  | 2016-04-04 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Measurement reference channels with FS3 are needed for measurement accuracy test case to verifie the requirements defined in clause 9.1.19.2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add measurement reference channels with FS3. Change 1 : add PDSCH Reference Measurement Channels for FS 3 Change 2 : add PCFICH/PDCCH/PHICH Reference Channel for FS 3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no measurement reference channel with FS3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.1.1.6, A.3.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS3 36.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

#### A.3.1.1.6	Frame Structure 3

Table A.3.1.1.6-1: PDSCH Reference Measurement Channels for FS 3

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  |  |  | R.0 FS3 |  |
| Channel bandwidth | MHz |  |  |  | 20 |  |
| Number of transmitter antennas |  |  |  |  | 1 |  |
| Allocated resource blocks (Note 4) |  |  |  |  | 24 |  |
| Allocated subframes per Radio Frame |  |  |  |  | 10 |  |
| Modulation |  |  |  |  | QPSK |  |
| Target Coding Rate |  |  |  |  | 1/3 |  |
| Information Bit Payload |  |  |  |  |  |  |
| For Sub-Frames 4, 9 | Bits |  |  |  | 2088 |  |
| For Sub-Frame 5 | Bits |  |  |  | 2088 |  |
| For Sub-Frame 0 | Bits |  |  |  | 2088 |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits |  |  |  | 2088 |  |
| Number of Code Blocks per Sub-Frame (Note 5) |  |  |  |  |  |  |
| For Sub-Frames 4, 9 |  |  |  |  | 1 |  |
| For Sub-Frame 5 |  |  |  |  | 1 |  |
| For Sub-Frame 0 |  |  |  |  | 1 |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 |  |  |  |  | 1 |  |
| Binary Channel Bits Per Sub-Frame |  |  |  |  |  |  |
| For Sub-Frames 4, 9 | Bits |  |  |  | 6624 |  |
| For Sub-Frame 5 | Bits |  |  |  | 6336 |  |
| For Sub-Frame 0 | Bits |  |  |  | 6336 |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits |  |  |  | 6624 |  |
| Max. Throughput averaged over 1 frame | kbps |  |  |  | 2088 |  |
| Note 1: 2 symbols allocated to PDCCH for 20 MHz channel BW. Note 2: Reference signal, synchronization signals allocated as defined in 3GPP TS 36.211 [16]. Note 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 36.213 [3]. Note 4: Allocation is located in the middle of bandwidth. Note 5: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 6: PDSCH allocation applies only to subframes not configured as PRS subframes. Note 7: PDSCH allocation applies only to subframes where there is no DRS transmission Note 8: PDSCH is not transmitted in subframes where it is determined that transmission should not occur according to the listen before talk (LBT model). Note 9: Max throughput averaged over 1 frame does not account for missed PDSCH transmission due to LBT or DRS transmission |  |  |  |  |  |  |

< END OF CHANGE #1 >

< START OF CHANGE #2 >

#### A.3.1.2.3	FS 3

Table A.3.1.2.3-1: PCFICH/PDCCH/PHICH Reference Channel for FS 3

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  |  |  | R.0 FS3 |  |
| Channel bandwidth | MHz | 10 | 10 | 10 | 20 |  |
| Number of transmitter antennas |  |  |  |  | 1 |  |
| Control region OFDM symbolsNote1 | symbols |  |  |  | 2 |  |
| Aggregation level | CCE |  |  |  | 8 |  |
| DCI Format |  |  |  |  | Note 3 |  |
| Cell ID |  |  |  |  | Note 4 |  |
| Payload (without CRC) | Bits |  |  |  | Note 5 |  |
| Note 1: The control region consists of PCFICH, PHICH and PDCCH. Note 2: DCI formats are defined in TS 36.212. Note 3: DCI format shall depend upon the test configuration. Note 4: Cell ID shall depend upon the test configuration. Note 5: Payload size shall depend upon the test configuration. Note 6: For PDCCH using SI/RA/P-RNTI, Aggregation level 4 is used. Note 7: PCFICH/PDCCH/PHICH allocation applies only to subframes where there is no DRS transmission Note 8: PCFICH/PDCCH/PHICH are not transmitted in subframes where it is determined that transmission should not occur according to the listen before talk (LBT model). |  |  |  |  |  |  |

< END OF CHANGE #2 >